3GPP TSG-RAN WG2 #100	Tdoc R2-1713344

Reno, Nevada, USA, 27th November – 1st December 2017

Agenda Item:	10.4.1.3.1

Source:	Ericsson

Title:	RRC Reconfiguration of NR-NR DC and NE-DC

Document for:	Discussion, Decision

# Introduction

Even though RAN2 has downprioritised Stage-3 details of NR-NR DC as well as NR-E-UTRA-DC, it is good to look general signalling structure before freezing EN-DC so that the structure is future proof.

# Discussion

## Current status of EN-DC

RAN2 has long time ago agreed that NR configuration embedded in the LTE RRCConnectionReconfiguration is RRC PDU. Later on it was agreed that this is same as NR RRCReconfiguration message. In addition, it was agreed that bearer configuration is given in another transparent container, one used to configure bearers using KeNB and constructed by the MN and another used to configure bearers using S-KgNB and constructed by the SN.

In the current NR RRC TP (as per email discussion 99bis#17), NR RRC Reconfiguration structure is following:

RRCReconfiguration-IEs ::= SEQUENCE {

-- Configuration of Radio Bearers (DRBs, SRBs) including SDAP/PDCP

-- In EN-DC, only used with SRB3

radioBearerConfig						RadioBearerConfig 														OPTIONAL, -- Need M

-- Configuration of primary and secondary cell groups (Dual Connectivity):

masterCellGroupConfig					CellGroupConfig															OPTIONAL, -- Cond M

secondaryCellGroupToAddModList			SEQUENCE (SIZE (1..maxSCellGroups)) OF CellGroupConfig					OPTIONAL, -- Need M

secondaryCellGroupToReleaseList			SEQUENCE (SIZE (1..maxSCellGroups)) OF CellGroup-Identity				OPTIONAL, -- Need M

measConfig								MeasConfig																OPTIONAL, -- Need M

lateNonCriticalExtension				OCTET STRING ()															OPTIONAL,

nonCriticalExtension					SEQUENCE{}																OPTIONAL

}

-- TAG-RRCRECONFIGURATION-STOP

-- ASN1STOP

In the embedded NR RRC PDU in the LTE RRCConnectionReconfiguration, the fields that can be present are

- secondaryCellGroupToAddModList to configure 1 SCG

- measConfig to configure NR measurements

Confirm understanding that embedded NR RRC PDU in the LTE RRRCConnectionReconfiguration can include secondaryCellGroupToAddModList and measConfig and nothing else

RadioBearerConfig in the structure can be used to reconfigure PDCP when SRB3 is used. Usage of that is not very clear as PDCP parameters do not change so often. What could be change often is UL direction for the split bearer and PDCP re-establishment indication. However, these parameter needs to be coordinated with MN and thus also sent via SRB1.

Usage of radioBearerConfig in RRCReconfiguration message is not so clear.

Looking at the RadioBearerConfig: this includes security parameters:

SecurityConfig	::=					SEQUENCE {

securityAlgorithmConfig					SecurityAlgorithmConfig									OPTIONAL,	-- Need M

keyToUse								ENUMERATED{KeNB, S-KgNB}								OPTIONAL	-- Need M

...

}

These parameters are for the bearers configured with NR PDCP. Rest of the parameters the UE gets via LTE RRC (SecurityModeCommand and RRCConnectionReconfiguration with MobilityControlInfo). It is worthwhile to think how the security configuration looks in the standalone NR and NR-NR DC.

## Introduction of NR-NR-DC

When NR-NR DC is introduced, there are some open issues in RRC:

- Is secondary node configuration also constructed from SCG configuration and bearer configuration for PDCP?

- Is SCG configuration part full RRC message or IE?

- How security is configured?

We consider that it is good to reuse the structure where PDCP configuration and SCG configuration are separated also in NR-NR DC. This makes it possible to use harmonised bearer concept also in the other cases and have aligned structure.

In NR-NR DC, introduce separate bearer configuration (RadioBearerConfig) per termination  point

About the second question, we consider that SCG configuration does not need to be full RRC message in NR-NR DC. This is because in that case, both cell groups are NR and specified in the same RRC specification and there is no need to encapsulate the message similar to inter-RAT HO.

In NR-NR DC, SCG configuration is not separate RRC PDU

Signalling structure example is given here:

RRCReconfiguration-IEs ::= SEQUENCE {

-- Configuration of Radio Bearers (DRBs, SRBs) including SDAP/PDCP

-- In EN-DC, only used with SRB3

-- Each configuration corresponds to one termination point

radioBearerConfigToAddModList		    SEQUENCE (SIZE (1..maxCellGroups)) OF RadioBearerConfig															OPTIONAL, -- Need M

-- Configuration of primary and secondary cell groups (Dual Connectivity):

masterCellGroupConfig					CellGroupConfig															OPTIONAL, -- Cond M

secondaryCellGroupToAddModList			SEQUENCE (SIZE (1..maxSCellGroups)) OF CellGroupConfig					OPTIONAL, -- Need M

secondaryCellGroupToReleaseList			SEQUENCE (SIZE (1..maxSCellGroups)) OF CellGroup-Identity				OPTIONAL, -- Need M

measConfig								MeasConfig																OPTIONAL, -- Need M

lateNonCriticalExtension				OCTET STRING ()															OPTIONAL,

nonCriticalExtension					SEQUENCE{}																OPTIONAL

}

-- TAG-RRCRECONFIGURATION-STOP

-- ASN1STOP

RadioBearerConfig information element

-- ASN1START

-- TAG-RADIO-BEARER-CONFIG-START

RadioBearerConfig ::=					SEQUENCE {

srb-ToAddModList						SRB-ToAddModList										OPTIONAL, -- Cond M

srb-ToReleaseList						INTEGER (3)												OPTIONAL, -- Cond M

drb-ToAddModList						DRB-ToAddModList										OPTIONAL, -- Cond M

drb-ToReleaseList						DRB-ToReleaseList										OPTIONAL, -- Need M

securityConfig 							SecurityConfig											OPTIONAL  -- Cond HO

}

SRB-ToAddModList ::=						SEQUENCE (SIZE (1..2)) OF SRB-ToAddMod

SRB-ToAddMod ::=							SEQUENCE {

srb-Identity								SRB-Identity,

-- may only be set if the cell groups of all linked logical channels are reset or released

reestablishPDCP								ENUMERATED{true}									OPTIONAL, 		-- Cond HO

pdcp-Config									PDCP-Config											OPTIONAL,		-- Cond PDCP

...

}

DRB-ToAddModList ::=						SEQUENCE (SIZE (1..maxDRB)) OF DRB-ToAddMod

DRB-ToAddMod ::=							SEQUENCE {

cnAssociation								CHOICE {

-- The EPS bearer ID determines the EPS bearer when NR connects to EPC using EN-DC

eps-BearerIdentity							INTEGER (0..15)										OPTIONAL,		-- Cond EPS-DRB-Setup

--	The SDAP configuration determines how to map QoS flows to DRBs when NR connects to the 5GC

sdap-Config									SDAP-Config											OPTIONAL		-- Cond NGC

},

drb-Identity								DRB-Identity,

-- may only be set if the cell groups of all linked logical channels are reset or released

reestablishPDCP								ENUMERATED{true}									OPTIONAL, 		-- Cond HO

recoverPDCP									ENUMERATED{true}									OPTIONAL, 		-- Need N

pdcp-Config									PDCP-Config											OPTIONAL,		-- Cond PDCP

}

DRB-ToReleaseList ::=				SEQUENCE (SIZE (1..maxDRB)) OF DRB-Identity

SecurityConfig	::=					SEQUENCE {

securityAlgorithmConfig					SecurityAlgorithmConfig									OPTIONAL,	-- Need M

keyToUse								ENUMERATED{KeNB, S-KgNB}								OPTIONAL	-- Need M

...

}

-- TAG-RADIO-BEARER-CONFIG-STOP.

We propose to discuss the signalling structure as presented in this contribution further in RAN2#100 meeting.

Discuss RadioBearerConfig structure and SecurityConfig structure as presented in this contribution

# Conclusion

In section 2 we made the following observations:

Observation 1	Usage of radioBearerConfig in RRCReconfiguration message is not so clear.

Based on the discussion in section 2 we propose the following:

Proposal 1	Confirm understanding that embedded NR RRC PDU in the LTE RRRCConnectionReconfiguration can include secondaryCellGroupToAddModList and measConfig and nothing else

Proposal 2	In NR-NR DC, introduce separate bearer configuration (RadioBearerConfig) per termination  point

Proposal 3	In NR-NR DC, SCG configuration is not separate RRC PDU

Proposal 4	Discuss RadioBearerConfig structure and SecurityConfig structure as presented in this contribution