3GPP TSG-RAN WG2 #100	R2-1713393

Reno, Nevada, USA, 27th Nov – 1st Dec 2017

Agenda Item:	10.4.1.5.3

Source:	Ericsson

Title:	Discussion and TP on remaining issues on SCG Failure

Document for:	Discussion, Decision

# Introduction

Regarding SCG RLF, there are still some open issues that need to be resolved, one is from RAN2-NR-AH#2 about UE behaviour in case of SCG failure when SCG is the configured path for SRB transmission (see below):

Agreements:

1: MN determines to use MCG duplication SRB and configures MCG duplication SRB by MN RRC signalling.

2: For all DC cases (all MR-DC and NR-NR DC cases) for 'duplication SRB', UL packet transmission is configured by RRC to use MCG path, SCG path or duplicate on both MCG and SCG.

FFS Duplication on SRB for CA cases

FFS Behaviour in the case of SCG failure when SCG is the configured path.

This contribution will address the remaining open issue above regarding the SCG failure.

# Discussion

In LTE, when a Radio Link Failure (RLF) is detected on the MCG, RRC connection reestablishment procedure is triggered [1], suspending the ongoing communication until the UE re-synchronizes with the network. On the other hand, if the UE experiences a failure on the SCG, then the UE does not have to initiate an RRC connection reestablishment procedure but instead it sends SCGFailureInformation via MCG SRB. Upon receiving SCGFailureInformation, MN can then decide to initiate an SN release (and optionally, change the SCG to an SN with a better radio conditions). In this case, as the MCG radio link is available and the traffic can be rerouted via MCG, UE does not experience a notable service interruption as in MCG RLF.

In NR, the actions triggered upon an RLF are mainly based on the existing LTE procedures. Accordingly, in RAN2#97bis meeting, it was agreed that if an RLF is detected on the MCG (i.e., MCG RLF), the UE initiates the RRC connection re-establishment procedure. It was also agreed that when an RLF is detected on the SCG (i.e., SCG RLF), UE suspends all SCG SRBs/DRBs as well as SCG transmission for MCG split SRBs/DRBs / SCG split DRBs. In this case, instead of directly triggering the RRC re-establishment procedure, UE indicates to the MN an SCG RLF has occurred with an SCGFailureInformation via the MCG path of MCG Split SRB or MCG SRB. However, if an MCG split SRB1 is configured and the SCG i.e., SRB1S, is the only configured path, upon the detection of an SCG failure it is not clear how the UE can notify the MN with SCGFailureInformation. As illustrated in Figure 1, this observation is valid independent from whether there is a direct SCG SRB i.e., SRB3, configured or not.

If an MCG split SRB1 is configured and the SCG i.e., SRB1S, is the only configured path for that SRB, upon the detection of an SCG failure, it is not clear how the UE can send the SCGFailureInformation message to the MN.

| - MCG Split SRB configured and SCG path (SRB1S) activated | - MCG Split SRB configured and SCG path (SRB1S) activated as well as direct SCG SRB (SRB3) configured |
| --- | --- |

Figure 1: Illustration of the CP scenarios where an MCG split SRB1 is configured and the SCG (SRB1S) is the only configured path. Configured CP paths for SRB1 and SRB3 are illustrated in red.

If an MCG split SRB1 is configured and the SCG (SRB1S) is the only configured path for that SRB, after a UE has experienced an RLF on the SCG, UE may simply initiate the RRC connection re-establishment procedure to restore the connectivity. However, this would cause a significant service interruption time, especially if there are other active bearers that are configured to use the MCG path (e.g., VoLTE bearers) when the SCG failure is detected. Doing re-establishment will not only cause service interruption for such bearers, but it is also rather unnecessary to begin with, as the MCG could be experiencing good (radio) conditions at that time. Thus, a preferred alternative for UE is to switch the path referred to MCG Split SRB1 over the MCG and notify the network with SCGFailureInformation. When sending the SCGFailureInformation, if no scheduling grant is available on the MCG, then the UE shall send a scheduling request to the MN.

If an MCG split SRB1 is configured and the SCG is the only configured path for that SRB, after a UE has experienced a failure on the SCG, the UE could initiate an RRC connection re-establishment procedure to restore the connectivity. However, this would result in a significant service interruption time.

Upon detecting an SCG failure, a UE that is configured with MCG split SRB1 and the SCG i.e., SRB1S, as the only configured path for that SRB, switches the path of the split SRB1 to the MCG to send SCGFailureInformation message to the MN. Stage 3 description should capture the attached Text Proposal accordingly.

# Text proposal to TS 36.331

--- Beginning of the change proposal ---

#### 5.6.13.3	Actions related to transmission of SCGFailureInformation message

The UE shall set the contents of the SCGFailureInformation message as follows:

1>	if the UE initiates transmission of the SCGFailureInformation message to provide SCG radio link failure information:

2>	include failureType and set it to the trigger for detecting SCG radio link failure;

1>	else if the UE initiates transmission of the SCGFailureInformation message to provide SCG change failure information:

2>	include failureType and set it to scg-ChangeFailure;

1>	else if the UE initiates transmission of the SCGFailureInformation message due to exceeding maximum uplink transmission timing difference:

2>	include failureType and set it to maxUL-TimingDiff;

1>	set the measResultServFreqList to include for each SCG cell that is configured, if any, within measResultSCell the quantities of the concerned SCell, if available according to performance requirements in [16];

1>	for each SCG serving frequency included in measResultServFreqList, include within measResultBestNeighCell the physCellId and the quantities of the best non-serving cell, based on RSRP, on the concerned serving frequency;

1>	set the measResultNeighCells to include the best measured cells on non-serving E-UTRA frequencies, ordered such that the best cell is listed first, and based on measurements collected up to the moment the UE detected the failure, and set its fields as follows;

2>	if the UE was configured to perform measurements for one or more non-serving EUTRA frequencies and measurement results are available, include the measResultListEUTRA;

2>	for each neighbour cell included, include the optional fields that are available;

NOTE 2:	The measured quantities are filtered by the L3 filter as configured in the mobility measurement configuration. The measurements are based on the time domain measurement resource restriction, if configured. Blacklisted cells are not required to be reported.

1>	If PDCP-config is configured for SRB1 with configuredRLC (FFS: The exact name of the parameter) set to LCID associated to SCG, the UE shall change the configuredRLC to LCID associated to MCG.

The UE shall submit the SCGFailureInformation message to lower layers for transmission.

--- End of the change proposal ---

# Conclusion

In summary, based on the discussion in Section 2 we have the following observations:

Observation 1	If an MCG split SRB1 is configured and the SCG i.e., SRB1S, is the only configured path for that SRB, upon the detection of an SCG failure, it is not clear how the UE can send the SCGFailureInformation message to the MN.

Observation 2	If an MCG split SRB1 is configured and the SCG is the only configured path for that SRB, after a UE has experienced a failure on the SCG, the UE could initiate an RRC connection re-establishment procedure to restore the connectivity. However, this would result in a significant service interruption time.

And we propose the following:

Proposal 1	Upon detecting an SCG failure, a UE that is configured with MCG split SRB1 and the SCG i.e., SRB1S, as the only configured path for that SRB, switches the path of the split SRB1 to the MCG to send SCGFailureInformation message to the MN. Stage 3 description should capture the attached Text Proposal accordingly.

# References

3GPP, TS 36.331 V14.0.0, 3rd Generation Partnership Project; Technical Specification Group Radio Access Network; Evolved Universal Terrestrial Radio Access (E-UTRA); Radio Resource Control (RRC); Protocol specification (Release 14).