3GPP TSG-RAN WG2 #100	R2-1713401

Reno, Nevada, USA, 27th Nov – 1st Dec 2017	(Resubmission of R2-1711140)

Agenda Item:	10.2.20

Source:	Ericsson

Title:	Support of SRB3 for NR-NR DC

Document for:	Discussion, Decision

# Introduction

Based on an offline discussion ([1]), it has been decided to add an FFS whether SCG SRB should be supported for the NR-NR DC case.

-   FFS on the support of SCG SRB for intra NR DC;

In this contribution, this is discussed further.

# Discussion

In LTE DC ([2]), the RRC entity is located only at the MeNB and UE side (i.e., shown in Figure 1 left). The purpose of terminating the UE RRC connection only at the MeNB was to reduce complexity and signalling overhead. In fact, in this case we have a centralized architecture where SeNB delivers RRC configurations for the SCG to the MeNB through an RRC container (scg-config) and it is this latter node that makes the final decision and constructs the eventual RRC configuration message that is transmitted to the UE.

However, when it comes to the EN-DC, in the RAN2 #97bis meeting it was agreed that direct RRC messages between NR (as secondary node) and the UE are supported (i.e., Figure 1 right). Further, it was agreed to support the possibility of sending those messages on a separate SRB (i.e., SCG SRB). The motivation for having a direct SRB between NR and the UE for EN-DC is due to the anticipated benefits that this could bring in terms of latency for some RRC procedures (e.g., measurement report sent directly from UE to the SN).

Figure 1 C-plane architecture for Dual Connectivity in LTE DC and EN-DC

However, the improvements brought by the support of the SCT SRB towards the secondary node in EN-DC are not substantial, unless there is a big congestion/delay over the backhaul link between the MN and SN. Also, a drawback of adopting this solution is an increased complexity on both the UE and network.

Further, since in NR-NR DC both the MN and SN have support the same RAT (i.e., NR), it is straightforward to consider as a baseline what has been already adopted for the LTE DC case. In this way, coordination between the master and secondary node can be handled in a centralized manner without the need to introduce unnecessary complexity as for the EN-DC case (i.e., where UE has to deal with two RRC entities).

Since in NR-NR DC the master and secondary nodes have the same radio technology, the simplest solution would be to apply the same control plane architecture of LTE DC.

- Thus, we propose to consider LTE DC has a baseline for the NR-NR DC control plane architecture. Along these lines, we propose:

SRB3 is not supported for NR-NR DC.

# Conclusion

In section 2 we made the following observations:

- Since in NR-NR DC the master and secondary nodes have the same radio technology, the simplest solution would be to apply the same control plane architecture of LTE DC.

- SRB3 is not supported for NR-NR DC.

# References

R2-1703966, [Offline discussion 97bis #31] [NR] Common necessary RRC procedures and functionality for NSA and SA, Huawei, RAN2 #97bis, April 2017.

3GPP, TS 36.330 V14.3.0, 3rd Generation Partnership Project; Technical Specification Group Radio Access Network; Evolved Universal Terrestrial Radio Access (E-UTRA) and Evolved Universal Terrestrial Radio Access Network (E-UTRAN); Overall description; Stage 2 (Release 14).

3GPP, TS 38.330 V1.0.0, 3rd Generation Partnership Project; Technical Specification Group Radio Access Network; NR; NR and NG-RAN Overall Description; Stage 2 (Release 15).