3GPP TSG-RAN WG2 #100	Tdoc R2-1713452

Reno, Nevada, USA, 27th November – 1st December 2017	Update of R2-1711938

Agenda Item:	9.4.4.1

Source:	Ericsson

Title:	Mobility Simulations of Aerial UEs - full buffer case

Document for:	Discussion, Decision

# Introduction

In this contribution, we show the mobility simulation results with a system-level simulator.

# Discussion

The simulation assumptions follow those defined in TR 36.777 Section A.2.1 with UMa full-buffer scenario. The handover failure definition follows TR 36.839. The following KPI are captured in this paper.

| KPI | Unit | Description |
| --- | --- | --- |
| Handover rate | HO/UE/sec | Number of HO attempts over time (including HOF) |
| HOF rate | % | Number of HO failures/Total number of HO attempts (including HOF) |
| Radio Link Failure (RLF) rate | RLF/UE/sec | Number of RLFs over time |
| Time in handoff | % | Fraction of time a UE is in HO procedure including time for successful HO (HO execution delay) and HOF (reestablishment delay) |
| Ping pong rate | % | Number of ping-pongs/Total number of successful handovers (excluding handover failures) |

## UMa full buffer

Figure 1 Radio Link Failure (RLF) rate, RLF/UE/sec

Since a UE at higher altitudes sees many cells that are not detectable on the ground, the amount of interference is higher for drones compared to terrestrial UEs, hence the SINR becomes worse. One reason for RLF to occur is that the physical layer determines that the quality of the signal is poor and hence the expected Packet Loss Rate goes above a threshold. This is also what we observe in the simulations as shown in Figure 1 above, which shows RLF rate increases with higher altitude, at least in a low-to-medium speed.

Figure 2 HOF rate in percentage (top); Handover rate, HO/UE/sec (down)

We also observe a higher fraction of handover failure with higher altitude. But the total number of HO decreases with an increasing height. Combined with the above Figure 1 on RLF, one possible explanation is that those aerials UEs in high altitude are more likely not able to enter the handover phase due to a strong interference.

Figure 3 Time in handoff

Since the total number of HO decreases with an increasing height, the fraction of time in handover also decreases with height, shown in Figure 3.  Note that in both Figure 2 and Figure 3, a higher number of handovers and a longer timer in handoff are observed with a higher speed.

Figure 4 Ping pong rate in percentage

Figure 4 shows that the ping-pong rate does not depend on the speed. In addition, a smaller ping-pong rate is observed with a higher altitude.

These results suggest that one of the largest problems impacting mobility performance is the massive interference which is prominent when number of drone UEs in the network is large.

One potential problem for mobility is RLF due to interference which is prominent when number of drone UEs in the network is large.

# Conclusion

We observe that:

Observation 1	One potential problem for mobility is RLF due to interference which is prominent when number of drone UEs in the network is large.