3GPP TSG-RAN WG2 meeting #100	Tdoc R2-17135000

# Reno, USA, 27 November- 1 December 2017

Agenda Item:		10.4.1.2

Souce:	Samsung

Title:	NR RRC specification, protocol extension aspects & introduction of SA

Document for:		Discussion and decision

# Introduction

This paper discusses two aspects related to mechodology for NR RRC specification. First of all, the paper discusses an improved way for doing protocol extension for size critical messages. In particular, for the extension of IEs defining the entry of lists within SIBs & UE capabilities, we propose RAN2 to consider use of the VLEC rather than the extension marker.

Secondly, the paper discusses what extension mechanism to use for introducing the SA related changes. For this, we propose to use the regular non-critical extension approach as baseline. Critical extensions should however be considered in specific cases e.g. when a change of information structure is highly desirable.

# Discussion

## Size critical extension, SIB5 example

In previous meetings we discussed ways to improve extension of size critical messages (SIBs and UE capabilities) i.e. avoiding excessive extension marker overhead without the cumbersome parallel lists, sometimes involving many levels. Based on previous discussions, we understand there is some interest but preference is to stick to proven concepts. Given that in LTE the non-critical extensions in the middle of a message only excists within Variable Length Extension Containers (and not after an extension marker, we propose to only consider the VLEC framework from now.

SIB5 includes a list of neighbouring Freqs, and each entry carryies the cell re-selection parameters are provided. These re-selection parameters been extended several times. It would be nice to introduce these simply at the level of the corresponding IE. One way how this could be done while just having a single length determinant per frequency (rather than one per release in which extensions are introduced), would be as follows:

SystemInformationBlockType5 ::=		SEQUENCE {

neighFreqList						NeighFreqInfoist,

lateNonCriticalExtension			OCTET STRING					OPTIONAL,

...

}

NeighFreqInfoList ::=		SEQUENCE (SIZE (1..maxFreq)) OF NeighFreqInfo

NeighFreqInfo ::=	SEQUENCE {

dl-CarrierFreq						ARFCN-ValueEUTRA,

q-RxLevMin							Q-RxLevMin,

p-Max								P-Max							OPTIONAL,		-- Need OP

-- Other parameters omitted

lateNonCriticalExtension			OCTET STRING	(CONTAINING NeighFreqInfo-v8h0)	OPTIONAL,

nonCriticalExtension				OCTET STRING	(CONTAINING NeighFreqInfo-v8h0)	OPTIONAL,

...,

}

-- Late non- critical extensions

NeighFreqInfo-v8h0 ::=		SEQUENCE {

multiBandInfoList					MultiBandInfoList				OPTIONAL	-- Need OR

nonCriticalExtension				NeighFreqInfo-v9e0				OPTIONAL

}

NeighFreqInfo-v9e0 ::=	SEQUENCE {

dl-CarrierFreq-v9e0					ARFCN-ValueEUTRA-v9e0	OPTIONAL,	-- Cond dl-FreqMax

multiBandInfoList-v9e0				MultiBandInfoList-v9e0	OPTIONAL	-- Need OR

nonCriticalExtension				NeighFreqInfo-v10j0				OPTIONAL

}

NeighFreqInfo-v10j0 ::=	SEQUENCE {

freqBandInfo-r10					NS-PmaxList-r10				OPTIONAL,	-- Need OR

multiBandInfoList-v10j0				MultiBandInfoList-v10j0		OPTIONAL	-- Need OR

nonCriticalExtension				SEQUENCE {}				OPTIONAL

}

-- Regular non- critical extensions

NeighFreqInfo-v940 ::=	SEQUENCE {

q-QualMin-r9					Q-QualMin-r9					OPTIONAL,		-- Need OP

threshX-Q-r9					SEQUENCE {

threshX-HighQ-r9				ReselectionThresholdQ-r9,

threshX-LowQ-r9					ReselectionThresholdQ-r9

}																OPTIONAL		-- Cond RSRQ

nonCriticalExtension				SEQUENCE {}				OPTIONAL

}

Proposal 1	RAN2 is requested to consider more extensive use of the VLEC i.e. for the extension of IEs defining the entry of lists within size critical messages (i.e. mainly SIBs & UE capabilities).

## Introduction of SA

So far there has not been very much discussion about how to best introduce SA in TS 38.331.I.e. there has not been much analysis of:

- What kind of changes can be expected and

- Which kind of extension method is best suited for adding these changes

- How to best handle NSA evolvement and SA development alongside

In this document we perform some initial analysis from our side.

Overview of expected changes

We did so far not manage to do a proper exercise but expect changes in many areas, although obviously most changes are CP related. However, there may also be quite some changes to the radio resource configuration.

| Area | SA/ JUN-18 add ons |
| --- | --- |
| L1 | Several changes expected (i.e. further completion). May be somewhat difficult to separate SA specific |
| MAC | Prioritzed RA, support of PDCP duplication (MAC CE) |
| RLC | None? |
| PDCP | PDCP duplication. |
| SDAP | All |
| Measurements | Additional events, additional one-shot? |
| System information | All but MIB, SIB1 |
| Idle mode steering | SIBs, info upon leaving ative |
| Connection management | Setup, release, page |
| Connecton mobility | Handover, redirection upon release |
| Inactive (to/ from) | Suspend, resume, page |
| IRAT HO (to/ from) | New |
| UE capabilities | Significant changes |
| Oher |  |

Tab. 1: Overview of some changes expected, per area

Which kind of extension method is best suited for adding these changes

We think the following are the main extension mechanism to be considered when introducing SA.

- Regular non-critical extensions: this is the default and most used extension approach for the introduction of new field, the addition of subbields as well as the addition of additional values.

- Critical extension of IEs: in this case the original field is replaced by a field with a new definition. This approach has been used for quite a few fields, in particular when the information strucuture was not well suited to accommodate the new extensions.

  - This approach has also been used in UL in a few cases (UE capabilities)

  - In some cases, the original field was mandatory (so to be ignored when both are signalled)

  - Procedures have been defined for up- and down-switching

- Critical extension of messages: in this case the original message is replaced by a revision including all original functionality as well as extensions. This approach has rarely been used in LTE

Given that in LTE most changes have been supported by the first 2 approaches, we regard these as the primary extension approaches. Moreover, alike in LTE, we think that the regular non-citical extension approach should be the default mechanism. I.e. critical IE extension should only be considered if there is a particular problem. In LTE, the critical extension approach has a.o. been used in cases in which the original information structure was not suitable to cover the extensions.

How to best handle NSA evolvement and SA development alongside

After December, quite some changes may be expected regarding NSA i.e. quite some corrections (a.o. resulting from ASN.1 review) and some finalisation aspects (a.o. related to RAN1 and RAN4). At the same time a significant amount of SA related changes can be expected, affecting the same ASN.1 sections. Some furthere remarks regarding this:

- We think it would be good to collect all NSA related changes in a single CR

- We think the NSA related changes should be introduced in the SA running CR at the same time

  - As the NSA changes may affect hot to best introduce the SA related changes

- We think that this merging in of NSA related changes will be similar regardless of whethr critical extensions are used to introduce the SA related changes.

Summary

Altogether it seems difficult to predict which approach would be most likely. It may be sufficient to just develop the running CR introducing the SA related changes using the regular non-critical extension approach as baseline. Whenever particular difficulties are spotted, other approaches can be considered. In particular when a change of information structure is desirable, e.g. because the initial specification version was developed in a rush and with an incomplete picture in mind, such critical extensions may be preferred. Based on our analysis, we therefore propose:

Proposal 2	Develop the running CR introducing the SA related changes using the regular non-critical extension approach as baseline. Critical extensions should however be considered in specific cases e.g. when a change of information structure is highly desirable

# Conclusion & recommendation

This paper discusses two aspects related to mechodology for NR RRC specification i.e. how to do protocol extension for size critical messages and how to introduce the SA related changes. The document includes the following proposal that RAN2 is requested to discuss and conclude.

Proposal 1	RAN2 is requested to consider more extensive use of the VLEC i.e. for the extension of IEs defining the entry of lists within size critical messages (i.e. mainly SIBs & UE capabilities).

Proposal 2	Develop the running CR introducing the SA related changes using the regular non-critical extension approach as baseline. Critical extensions should however be considered in specific cases e.g. when a change of information structure is highly desirable

# References

[1] TS 38.331 Radio Resource Control