3GPP TSG-RAN WG2 #100	Tdoc R2-1713574

Reno, Nevada, USA, 27th November – 1st December 2017

Agenda Item:	10.3.1.12

Source:	Ericsson

Title:	PHR Text proposal

Document for:	Discussion, Decision

# Introduction

The text proposal follows [1]. The proposed update is made on version 1.1.0 of TS 38.321.

# Text Proposal for TS 38.321

### 5.4.6	Power Headroom Reporting

The Power Headroom reporting procedure is used to provide the serving gNB with information about the difference between the nominal UE maximum transmit power and the estimated power for UL-SCH transmission per activated Serving Cell and also with information about the difference between the nominal UE maximum power and the estimated power for UL-SCH and PUCCH transmission on SpCell and PUCCH SCell.

RRC controls Power Headroom reporting by configuring the following parameters:

-	phr-PeriodicTimer;

-	phr-ProhibitTimer;

-	phr-Tx-PowerFactorChange configured for the NR UL carrier and optionally for the SUL carrier;

-	phr-Type2PCell;

-	phr-Type2OtherCell;

-	phr-ModeOtherCG.

Editor's note: The name of phr-Tx-PowerFactorChange can be updated as it applies not only to DL pathloss but also for P-MPR backoff. All the names of the parameters should be aligned with RRC.

Editor's note: phr-ModeOtherCG comes from LTE, but RAN2 (and also other WGs) should discuss whether it would be needed for EN-DC. If not needed, the procedural text below should also be updated.

A Power Headroom Report (PHR) shall be triggered if any of the following events occur:

-	phr-ProhibitTimer expires or has expired and the path loss has changed more than phr-Tx-PowerFactorChange dB for at least one activated Serving Cell of any MAC entity which is used as a pathloss reference since the last transmission of a PHR in this MAC entity when the MAC entity has UL resources for new transmission;

-	phr-PeriodicTimer expires;

-	upon configuration or reconfiguration of the power headroom reporting functionality by upper layers, which is not used to disable the function;

-	activation of an SCell of any MAC entity with configured uplink;

-	addition of the PSCell;

-	switch between NR UL carrier and SUL carrier of a serving cell;

-	phr-ProhibitTimer expires or has expired, when the MAC entity has UL resources for new transmission, and the following is true in this NR-UNIT for any of the activated Serving Cells of any MAC entity with configured uplink:

-	there are UL resources allocated for transmission or there is a PUCCH transmission on this cell, and the required power backoff due to power management (as allowed by P-MPRc as specified in TS 38.101 [10]) for this cell has changed more than phr-Tx-PowerFactorChange dB since the last transmission of a PHR when the MAC entity had UL resources allocated for transmission or PUCCH transmission on this cell.

NOTE:	The MAC entity should avoid triggering a PHR when the required power backoff due to power management decreases only temporarily (e.g. for up to a few tens of milliseconds) and it should avoid reflecting such temporary decrease in the values of PCMAX,c/PH when a PHR is triggered by other triggering conditions.

If the MAC entity has UL resources allocated for new transmission the MAC entity shall:

1>	if it is the first UL resource allocated for a new transmission since the last MAC reset:

2>	start periodicPHR-Timer;

1>	if the Power Headroom reporting procedure determines that at least one PHR has been triggered and not cancelled, and;

1>	if the allocated UL resources can accommodate the MAC CE for PHR which the MAC entity is configured to transmit, plus its subheader, as a result of logical channel prioritization:

2>	for each activated Serving Cell with configured uplink associated with any MAC entity:

3>	obtain the value of the Type 1 power headroom;

3>	if this MAC entity has UL resources allocated for transmission on this Serving Cell; or

3>	if the other MAC entity, if configured, has UL resources allocated for transmission on this Serving Cell and phr-ModeOtherCG is set to real by upper layers:

4>	obtain the value for the corresponding PCMAX,c field from the physical layer;

3>	if phr-Type2PCell is configured:

4>	obtain the value of the Type 2 power headroom for the PCell;

4>	obtain the value for the corresponding PCMAX,c field from the physical layer

3>	if phr-Type2OtherCell is configured:

4>	if PUCCH SCell is configured:

5>	obtain the value of the Type 2 power headroom for the PUCCH SCell;

Editor's note: UE may need to obtain type 2 PH from multiple SCells if RAN1 allows having multiple PUCCH SCells. (This would also impact to PHR format). RAN1/2 confirmation requires.

4>	else (i.e. other CG is configured):

5>	obtain the value of the Type 2 power headroom for the SpCell of the other MAC entity;

4>	obtain the value for the corresponding PCMAX,c field from the physical layer

3>	instruct the Multiplexing and Assembly procedure to generate and transmit a PHR MAC CE according to configured ServCellIndex and the PUCCH(s) for the MAC entity as defined in subclause 6.1.3.8 based on the values reported by the physical layer;

2>	start or restart periodicPHR-Timer;

2>	start or restart prohibitPHR-Timer;

2>	cancel all triggered PHR(s).

#### 6.1.3.8	Power Headroom Report MAC CE

The Power Headroom Report (PHR) MAC CE is identified by a MAC PDU subheader with LCID as specified in Table 6.2.1-2. It includes the bitmap, a Type 2 PH field and an octet containing the associated PCMAX,c field (if reported) for the PCell, a Type 2 PH field and an octet containing the associated PCMAX,c field (if reported) for either PSCell or PUCCH SCell, a Type 1 PH field and an octet containing the associated PCMAX,c field (if reported) for the PCell. It further includes, in ascending order based on the ServCellIndex, one or multiple of Type 1 PH fields and octets containing the associated PCMAX,c fields (if reported) for SCells indicated in the bitmap.

Editor's Note: Whether to support four-octet bitmap depends on whether we support 32 carries. Also the detailed design can be affected according to the discussion on uniqueness of cell index across CGs.

The presence of Type 2 PH field for PCell is configured by phr-Type2PCell, and the presence of Type 2 PH field for either PSCell or for PUCCH SCell is configured by phr-Type2OtherCell.

If a single serving cell is configured, the format in Figure 6.1.3.8-1 is used, otherwise a single octet bitmap is used for indicating the presence of PH per SCell when the highest SCellIndex of SCell with configured uplink is less than 8, otherwise four octets are used.

The PHR MAC CEs are defined as follows:

-	Ci: this field indicates the presence of a PH field for the SCell with SCellIndex i as specified in TS 38.331 [5]. The Ci field set to "1" indicates that a PH field for the SCell with SCellIndex i is reported. The Ci field set to "0" indicates that a PH field for the SCell with SCellIndex i is not reported;

-	R: reserved bit, set to "0";

-	V: this field indicates if the PH value is based on a real transmission or a reference format. For Type 1 PH, V=0 indicates real transmission on PUSCH and V=1 indicates that a PUSCH reference format is used. For Type 2 PH, V=0 indicates real transmission on PUCCH and V=1 indicates that a PUCCH reference format is used. Furthermore, for Type 1 and Type 2 PH, V=0 indicates the presence of the octet containing the associated PCMAX,c field, and V=1 indicates that the octet containing the associated PCMAX,c field is omitted;

-	Power Headroom (PH): this field indicates the power headroom level. The length of the field is 6 bits. The reported PH and the corresponding power headroom levels are shown in Table 6.1.3.8-1;

-	P: this field indicates whether the MAC entity applies power backoff due to power management. The MAC entity shall set P=1 if the corresponding PCMAX,c field would have had a different value if no power backoff due to power management had been applied;

-	PCMAX,c: if present, this field indicates the PCMAX,c or P̃CMAX,c (as specified in TS 38.213 [6]) used for calculation of the preceding PH field. The reported PCMAX,c and the corresponding nominal UE transmit power levels are shown in Table 6.1.3.8-2.

Figure 6.1.3.8-1: Extended PHR MAC CE with the highest SCellIndex of SCell with configured uplink is less than 8

Figure 6.1.3.8-2: PHR MAC CE with the highest SCellIndex of SCell with configured uplink is equal to or higher than 8

Figure 6.1.3.8-3: PHR MAC control element for a single cell

Table 6.1.3.8-1: Power Headroom levels for PHR

| PH | Power Headroom Level |
| --- | --- |
| 0 | POWER_HEADROOM_0 |
| 1 | POWER_HEADROOM_1 |
| 2 | POWER_HEADROOM_2 |
| 3 | POWER_HEADROOM_3 |
| … | … |
| 60 | POWER_HEADROOM_60 |
| 61 | POWER_HEADROOM_61 |
| 62 | POWER_HEADROOM_62 |
| 63 | POWER_HEADROOM_63 |

Table 6.1.3.8-2: Nominal UE transmit power level for PHR

| PCMAX,c | Nominal UE transmit power level |
| --- | --- |
| 0 | PCMAX_C_00 |
| 1 | PCMAX_C_01 |
| 2 | PCMAX_C_02 |
| … | … |
| 61 | PCMAX_C_61 |
| 62 | PCMAX_C_62 |
| 63 | PCMAX_C_63 |

# References

R2-1713484, " Power Headroom Reporting in NR ", Ericsson, RAN2#100, Reno, US, Nov 27th – Dec 1st 2017