3GPP TSG-RAN WG2 meeting #100	Tdoc  R2-1713680

# Reno, USA, November 27th - December 1st 2017

Agenda Item:		10.4.1.9

Souce:	HTC

Title:	Capability coordination in Inter-node RRC messages

Document for:		Discussion and decision

Introduction

This document discusses about the BC/BPC indication in the SCG-ConfigInfo messages. We provide our view in capturing the agreement in RAN2#99bis to the TP in INM.

Discussion

During the R2#99bis meeting, RAN2 agreed the following:

Agreements

1	The MN decides the LTE (resp NR) part of BC and BPC and provide SN indicating its choice of LTE (resp NR) part and SN continues further to determine the set of supportable NR (resp LTE) BCs and NR (resp LTE) BPC and then select an NR BC (resp LTE) and NR BPC (resp LTE)

1i	Similar process can be initiated by the SN as a request as part of SN initiated reconfiguration. MN may reject the request.

Therefore, as there had been some discussions on signalling choices for the MN [2][3], instead of signalling restriction information or a set of candidate NR BCs that are supportable with the selected LTE part of BC to the SN, the MN should indicate its own decision of BC and BPC and let the SN determine the supportable set and decide its part.

restrictedBandCombinationNR Indicates restrictions regarding the NR BCs the SN can configure by signalling the LTE BC selected by MN. The SN may configure any EN-BC including the indicated LTE BC selected by MN.

restrictedBasebandCombinationNR Indicates restrictions regarding the NR BPCs the SN can/ cannot configure i.e. by signalling the list of NR BPC the SN may configure.

As the current text shown above has not exactly captured the agreement in such logic, also we think the name of the IEs can be revised to better reflect the purpose, here we propose:

Proposal: RAN2 to adopt the text proposal to the TS 38.331 provided in this paper.

# Text Proposal to the INM in TS 38.331

- –	SCG-ConfigInfo

SCG-ConfigInfo message (only showing the concerned parts)

SCG-ConfigInfo-r15-IEs ::=			SEQUENCE {

eutra-CapabilityInfo			OCTET STRING (CONTAINING UECapabilityInformation)	OPTIONAL,

candidateCellInfoList			CandidateCellInfoList								OPTIONAL,

measResultSSTD					MeasResultSSTD										OPTIONAL,

configCoordinationInfo			ConfigCoordinationInfoSCG							OPTIONAL,

drx-InfoMCG						DRX-Info											OPTIONAL,

sourceConfigSCG-r15				OCTET STRING (CONTAINING RRCReconfiguration)		OPTIONAL,

nonCriticalExtension			SEQUENCE {}							OPTIONAL

}

ConfigCoordinationInfoSCG ::=			SEQUENCE {

supportableBandCombination			INTEGER							OPTIONAL,

supportableBasebandCombination		INTEGER							OPTIONAL,

maxMeasFreqsSCG-NR-r15					INTEGER							OPTIONAL,

...

}

SCG-ConfigInfo field descriptions

...

supportableBandCombination Indicates the LTE BC selected by MN. The SN configures any EN-BC including the indicated LTE BC selected by MN according to this field..

supportableBasebandCombination Indicates the LTE BPC selected by MN. The SN configures any BPC including the indicated LTE BPC selected by MN according to this field.

Conclusion & recommendation

The paper document concerns BC/BPC indication in the SCG-ConfigInfo messages in the report of the e-mail discussion [99bis#25][NR]. As there is conflict between the text captured and the agreement we made in the last meeting, here we proposed:

Proposal: RAN2 to adopt the text proposal to the TS 38.331 provided in this paper.

References

[1] Report of the e-mail [99bis#25][NR] on Inter-node RRC messages

[2] RAN2-99bis-Prague-chair-notes-2017-10-13-1730eom

[3] R2-1710860	Email discussion summary for [99#27][NR] Capability coordination, Part 3	Nokia, Nokia Shanghai Bell