3GPP TSG-RAN WG2 Meeting #100	R2-1713754

Reno, USA, 27th November – 1st December, 2017	Revision of R2-1711158

Agenda item:		9.14.2 (Early data transmission)

Source:	LG Electronics Inc.

Title: 	Issues on EDT (related to email discussion)

Document for:	Discussion and Decision

1. Introduction

One of the objectives in this WI is to evaluate power consumption/latency gain and specify necessary support for DL/UL data transmission on a dedicated resource during the Random Access procedure after NPRACH transmission and before the RRC connection setup is completed.

After RAN2#99bis, RAN2 had email discussion. In this contribution, we further discuss issues related to the email discussion.

2. Discussion

To conduct the RRC Connection Resume procedure, the UE keeps the UE AS context when it receives RRCConnectionRelease or RRCConnectionReject with suspend indication. When the UE sends RRCConnectionResumeRequest, it restores the RRC configuration and security context, restore low layer entities for SRB1, resume SRB1, etc. Upon reception of RRCConnectionResume, the UE performs the radio resource configuration procedure, restores low layer entities, resume SRB2 and DRB(s), etc. and enters RRC_CONNECTED. Then, the UE sends RRCConnectionResumeComplete in MSG5 and the user data via DRB(s).

In legacy operation, the UE transmits user data when the UE is in the CONNECTED state. According to TS36.331, the RRCConnectionResume message is currently used to resume a DTCH logical channel. When the UE sends the RRCConnectionResumeRequest message, DRB is not established yet. Then, upon receiving the RRCConnectionResume message, the UE resumes DRB(s) and transmits the RRCConnectionResumeComplete message over SRB1. Thus, if the UE transmits user data before receiving the RRCConnectionResume message, the UE should set up DTCH logical channel before receiving the RRCConnectionResume message for early data transmission.

Operating with early data transmission, the UE may send user data in parallel with transmission of RRCConnectionResumeRequest for UP solution. It seems likely that transmission of user data starting at this early stage will not continue after completion of the RRC Connection Resume procedure. Thus, there seems a particular DRB applicable for early transmission while the other DRBs seem not applicable for early transmission. For this, the eNB indicates which DRB(s) is(are) subject to early data transmission (e.g. by including a drb-identity) in the suspend message and then only indicated DRB(s) will be early resumed. After that, the UE should selectively resume a DTCH logical channel when the RRC Connection Resume procedure is triggered by the upper layer. Then, the UE can set up DTCH logical channel before receiving RRCConnectionResume.

Observation 1. If early data transmission is supported for User plane CIoT EPS optimisation, the UE should resume DTCH logical channel when the RRC Connection Resume procedure is triggered.

Proposal 1. If early data transmission is supported for User plane CIoT EPS optimisation, eNB should inform UE about a particular DRB information which is subject to early data transmission. Then, for early data transmission during the RRC Connection Resume, the UE resumes the DRB before receiving the RRCConnectionResume message while other DRB(s) are still suspended.

Assuming that both the UE and the eNB agreed to perform the quick release procedure, MSG5 can be omitted. The UE can be configured with a release timer (i.e. dataInactivityTimer). The details can be discussed in the RRC Connection Release Enhancement item. We think the RRC layer does not care about user data transmission failure. Therefore, if RRC connection establishment is successful and there is no more data to be transmitted, the RRC Connection can be release. If the RRC connection establishment fails (i.e. the eNB sends RRCConnectionReject or the UE does not receives MSG4), the UE performs legacy operation

Proposal 2. In a release timer (i.e. dataInactivityTimer) based manner, the UE can release the RRC connection without MSG5 if there is no data to be transmitted.

3. Conclusion

In this contribution, the followings are proposed.

Observation 1. If early data transmission is supported for User plane CIoT EPS optimisation, the UE should resume DTCH logical channel when the RRC Connection Resume procedure is triggered.

Proposal 1. If early data transmission is supported for User plane CIoT EPS optimisation, eNB should inform UE about a particular DRB information which is subject to early data transmission. Then, for early data transmission during the RRC Connection Resume, the UE resumes the DRB before receiving the RRCConnectionResume message while other DRB(s) are still suspended.

Proposal 2. In a release timer (i.e. dataInactivityTimer) based manner, the UE can release the RRC connection without MSG5 if there is no data to be transmitted.