3GPP TSG-RAN WG2 Meeting #100	R2-1713874

Reno, USA, 27th November – 1st December 2017

Agenda item:	9.10.2

Source:	Samsung

Title:	Consideration on Required Reliability for Tx carrier selection

Document for:	Discussion

- Introduction

In RAN2#99bis, there was a discussion on UE’s Tx carrier selection in PC5 CA and parameters to consider for the carrier selection. CBR (Chanel Busy Ratio) and PPPP (ProSe per Packet Priority) were agreed to use for Tx carrier selection, however, consideration on other parameters has not been closed as below [1]:

RAN2 #99bis Meeting eV2X LTE breakout session – Carrier selection in CA:

Agreements:

1: CBR should be considered for the UEs’ Tx carrier selection in PC5 CA from RAN2 perspective.

2: Priority indicated by PPPP should be considered for the UE’s Tx carrier selection in PC5 CA from RAN2 perspective. Not closed for other factors.

3: AS is aware of candidate V2X frequencies for V2X packet transmissions, which configured by upper layers (Same as Rel-14). FFS on the additional need in Rel-15.

4: UE capability on PC5 CA should be considered for the UE’s Tx carrier selection from RAN2 perspective. However no additional specification impacts are foreseen at the moment.

5: Configuration/Preconfiguration of PC5 carriers (at least one candidate set of PC5 CC) for the UE’s Tx carrier selection (like Rel-14). FFS if further standard changes (including UE behaviors) are needed for Rel-15 eV2X.

6: From RAN2 point of view we do NOT need a PCC and SCC.

7: No need of activation/deactivation mechanism for carriers.

8: FFS on how to handle Rx limited V2X UE.

We think that a factor for UE’s Tx carrier selection should be considered together with support of packet duplication which was agreed in RAN2#99bis as below [1]:

RAN2 #99bis Meeting eV2X LTE breakout session – Use case 2:

Agreements:

1: Agreed with the need of packet duplication

This contribution presents Tx carrier selection with consideration of packet duplication..

- Discussion

2.1 Tx carrier selection for packet duplication

The main objective of packet duplication has been well discussed as a mechanism to support high reliability requirements since 3GPP LTE advanced [2] and NR standards development [3]. Use case scenarios targeted in V2X phase 2 such as advanced driving, extended sensors and remote driving require high reliability up to 99.999% [4].

For supporting packet duplication to meet high reliability requirement, there are some issues to discuss such as in which layer packet duplication implemented and which PC5 carrier can be selected for the purpose. For example, if UE transmits packets that require high reliability of advanced driving use case, the UE should decide which carriers should be selected to guarantee required high reliability. Assuming that a selected Tx carrier is busy then the carrier may not be suitable to support high reliability requirement. In this case, the UE would select another carrier for packet duplication which guarantees packet transmission. Thus, reliability should be considered as a factor to select Tx carrier, especially in case of packet duplication.

Proposal 1. The Tx carrier selection should consider required reliability

2.2 Additional factor for Tx carrier selection from reliability perspective

There would be a discussion whether the agreed two factors i.e., PPPP and CBR in RAN2#99bis [1] can cover reliability requirement. PPPP cannot cover reliability requirement because PPPP is more related with latency requirement referring to the descriptions in [5] “The setting of the PPPP value should reflect the latency required in both UE and eNB, i.e. the low Packet Delay Budget (PDB) is mapped to the high priority PPPP value”. The PDB defines an upper bound for the time that a packet may be delayed between the UE and the UPF in [6]. The PDB is used to support the configuration of scheduling and link layer functions (e.g. the setting of scheduling priority weights). So PPPP cannot be a factor to support reliability requirement.

Also CBR represents channel congestion status. Using these two parameters i.e., PPPP and CBR if low latency is required (i.e., the value of PPPP is high), UE can transmit data via less congested carrier (i.e., the value of CBR is low). However, the resource collision may not be avoided. For this reason, the two factors are not sufficient to support reliability requirement. There needs other factor for Tx carrier selection to support reliability from resource collision.

As PDB can represent latency requirement, PER can be used for reliability. The PER defines an upper bound for the rate of SDUs (e.g. IP packets) that have been processed by the sender of a link layer protocol (e.g. RLC in RAN of a 3GPP access) but that are not successfully delivered by the corresponding receiver to the upper layer (e.g. PDCP in RAN of a 3GPP access). The PER defines an upper bound for a rate of non-congestion related packet losses. So PER can be an appropriate factor of reliability for Tx carrier selection.

For Reliability, we can reuse LTE QoS parameters which are described in [6] (See Table 1). According to Table 1, Standardized LTE QCI (QoS Class Identifier) are mapped to QoS characteristics such as priority, packet delay, packet error rate and so on. These characteristics of parameters can also represent certain service scenarios. For example, a V2X message for basic V2X service can be configured with Priority Level 2.5, PDB 50ms, PER 10-2.  Other V2X message for advanced V2X service (e.g., fully advanced driving - Latency Requirement: 3 ms, Reliability Requirement: 99.999%) parameters can be configured with PDB 3 ms, PER 10-3 which is not defined together in Table 1. If advanced V2X service considers only PER for QCI mapping, this scenario can reuse QCI 3 mapping case in Table 1. If advanced V2X service considers PDB and PER together for QCI mapping, this QoS characteristic can be discussed for the advanced V2X services.

Table 1: mapping Standardized LTE QCI to QoS characteristics in 3GPP

| QCI | Resource Type | Priority Level | Packet Delay Budget (NOTE 13) | Packet Error Loss Rate (NOTE 2) | Example Services |
| --- | --- | --- | --- | --- | --- |
| 1 (NOTE 3) |  | 2 | 100 ms (NOTE 1, NOTE 11) | 10-2 | Conversational Voice |
| 2 (NOTE 3) | GBR | 4 | 150 ms (NOTE 1, NOTE 11) | 10-3 | Conversational Video (Live Streaming) |
| 3 (NOTE 3), NOTE 14 |  | 3 | 50 ms (NOTE 1, NOTE 11) | 10-3 | Real Time Gaming, V2X messages |
| 4 (NOTE 3) |  | 5 | 300 ms (NOTE 1, NOTE 11) | 10-6 | Non-Conversational Video (Buffered Streaming) |
| 65 (NOTE 3, NOTE 9, NOTE 12) |  | 0.7 | 75 ms (NOTE 7, NOTE 8) | 10-2 | Mission Critical user plane Push To Talk voice (e.g., MCPTT) |
| 66 (NOTE 3, NOTE 12) |  | 2 | 100 ms (NOTE 1, NOTE 10) | 10-2 | Non-Mission-Critical user plane Push To Talk voice |
| 75 (NOTE 14) |  | 2.5 | 50 ms (NOTE 1) | 10-2 | V2X messages |
| 5 (NOTE 3) |  | 1 | 100 ms (NOTE 1, NOTE 10) | 10-6 | IMS Signalling |
| 6 (NOTE 4) |  | 6 | 300 ms (NOTE 1, NOTE 10) | 10-6 | Video (Buffered Streaming) TCP-based (e.g., www, e-mail, chat, ftp, p2p file sharing, progressive video, etc.) |
| 7 (NOTE 3) | Non-GBR | 7 | 100 ms (NOTE 1, NOTE 10) | 10-3 | Voice, Video (Live Streaming) Interactive Gaming |
| 8 (NOTE 5) |  | 8 | 300 ms (NOTE 1) | 10-6 | Video (Buffered Streaming) TCP-based (e.g., www, e-mail, chat, ftp, p2p file |
| 9 (NOTE 6) |  | 9 |  |  | sharing, progressive video, etc.) |
| 69 (NOTE 3, NOTE 9, NOTE 12) |  | 0.5 | 60 ms (NOTE 7, NOTE 8) | 10-6 | Mission Critical delay sensitive signalling (e.g., MC-PTT signalling) |
| 70 (NOTE 4, NOTE 12) |  | 5.5 | 200 ms (NOTE 7, NOTE 10) | 10-6 | Mission Critical Data (e.g. example services are the same as QCI 6/8/9) |
| 79 (NOTE 14) |  | 6.5 | 50 ms (NOTE 1, NOTE 10) | 10-2 | V2X messages |

According to the Table 1 or modified Table 1 with QoS characteristics for advanced V2X services, UE can transmit V2X packets which require low PER using Tx carrier which supports high reliability requirement.

Proposal 2. There needs other factor (e.g., packet error rate) for Tx carrier selection to meet reliability requirement.

- Conclusion

Proposal 1. The Tx carrier selection should consider required reliability

Proposal 2. There needs other factor (e.g., packet error rate) for Tx carrier selection to meet reliability requirement.

4. References

[1] 3GPP TSG-RAN WG2 Meeting #99bis Chairman’s note: eV2X LTE breakout session

[2] RP-171489, Ultra Reliable Low Latency Communication for LTE.

[3]	R2-1707748, Draft TS 38.300 v060, Rapporteur (Nokia).

[4]	3GPP TS 22.186, Enhancement of 3GPP Support for V2X scenarios.

[5] 3GPP TS 23.285, Architecture enhancements for V2X services (Release 14)

[6] 3GPP TS 23.203, Policy and charging control architecture (Release 14)