3GPP TSG-RAN WG2 Meeting #100                                                                    R2-1714071

Reno, USA, 27thNovember-1stDecember 2017

Source:	vivo

Title:	offline discussion for BI value

Agenda Item:	10.3.1.3

Document for:	Discussion and Decision

- Introduction

RAN2 gave the guidelines for BI value [1]:

6: Guidelines for BI table design: The values for LTE are re-used, FFS if value of zero is removed, at least two reserved values are kept. [CB 514]

This document summarizes the following offline discussion.

- Discussion

After check LTE and NR MAC specificaition in [2][3], we confirm that the network can reset the BI value to zero by not including the BI in RAR.

  - Zero value of BI

Companies are invited to give your answer and comments for the below question in the Table 1.

Question1: Can we remove the zero value from the NR BI table?

Table 1: zero value of BI

| Companies | Answer (i.e., yes or not) | Comments |
| --- | --- | --- |
| vivo | Yes | We cannot find any usages for the zero BI value. |
| LG | Yes | If the BI field is not included in the RAR, the PREAMBLE_BACKOFF is set to 0 ms. Thus, backoff parameter value 0ms is redundant. |
| Ericsson | Yes | We share the view from LG. |
| ASUSTeK | No | BI MAC subheader consists of two R fields, which may be future usage. If the gNB wants to indicate something by these two R fields but is not wish the UE to perform backoff, it could set the BI value as 0. |
| Qualcomm | Yes | We are OK with replacing zero by 5ms |
| Samsung | Yes | Ok to replace zero value by 5 ms |
| Intel | Yes | Same view as LG |

Summary for the question: Can we remove the zero value from the NR BI table?

6 companies think that the zero value from the NR BI table can be removed.

1 company think that the zero value from the NR BI table is useful.

Proposal 1: to remove the zero value from the NR BI table.

  - Other values of BI

Companies are invited to give your answer and comments for the below question in the Table 2.

Question2: Whether are the other values of BI useful for NR in addition to LTE value?

Table 2: other values of BI

| Companies | Answer (i.e., yes or not) | Comments (i.e., 5ms …. If the answer is useful) |
| --- | --- | --- |
| vivo | Yes | We can add the 5ms and 2000ms values for NR BI value and two reserved indexes are kept after removing the zero BI value. 5ms is for URLLC service. 2000ms is for MTC service. |
| LG | Yes | A value shorter than 10ms is needed. Maybe 5ms is a good choice. A value larger than 960ms is also needed. 2000ms or 1920ms is a good choice. |
| Ericsson | Yes | In our contribution R2-1713478 we propose the following table. New values compared to LTE are highlighted. Table 7.2-1: Backoff Parameter values. Index; Backoff Parameter value (ms). 0; 5. 1; 10. 2; 20. 3; 30. 4; 40. 5; 60. 6; 80. 7; 120. 8; 160. 9; 240. 10; 320. 11; 480. 12; 960. 13; 1920. 14; Reserved. 15; Reserved. |
| ASUSTeK | Yes | In our view, URLLC service may be not expected to perform backoff, so the minor value is not needed. |
| Qualcomm | Yes | We are ok with adding the 1920ms to the table. |
| Samsung | Yes | We are ok with adding both the 5ms and 1920ms to the table. |
| Intel | Yes | Some smaller value e.g. 5 ms is useful. However, as long as we have some reserved values, more values can always be added later as needed. |

Summary for the question: Whether are the other values of BI useful?

6 companies think 5 ms and 1920 ms are useful.

1 company think that the minor value is not needed.

Proposal 2: 5 ms and 1920 ms are added in addition to LTE value.

- Conclusion

According to the summaries given above, we have the following proposals:

Proposal 1: to remove the zero value from the NR BI table.

Proposal 2: 5 ms and 1920 ms are added in addition to LTE value.

References

- NR UP_LTE_chair_minutes_RAN2-100-Reno_11-28-20-30

- TS 36.321

- TS 38.321

Annex

TS 36.321：

-     if the Random Access Response contains a Backoff Indicator subheader:

-     set the backoff parameter value as indicated by the BI field of the Backoff Indicator subheader and Table 7.2-1, except for NB-IoT where the value from Table 7.2-2 is used.

-     else, set the backoff parameter value to 0 ms.

TS 38.321：  Draft TS 38.321 v1.1.0

1>  if a downlink assignment for this NR-UNIT has been received on the PDCCH for the RA-RNTI and the received TB is successfully decoded:

2>  if the Random Access Response contains a Backoff Indicator subheader:

3>  set the PREAMBLE_BACKOFF to value of the BI field of the Backoff Indicator subheader using Table 7.2-1.

2>  else:

3>  set the PREAMBLE_BACKOFF to 0 ms.