3GPP TSG-RAN WG2 #100	R2-1714090

Reno, USA, November 27th - December 1st 2017

Agenda item:	10.3.1.6

Source:	Ericsson

Title:	TP on BSR formats

Document for:	Discussion and Decision

# 5	MAC procedures

## 5.4	UL-SCH data transfer

### 5.4.5	Buffer Status Reporting

The Buffer Status reporting (BSR) procedure is used to provide the serving gNB with information about UL data volume in the MAC entity.

RRC configures the following parameters to control the BSR:

-	periodicBSR-Timer;

-	retxBSR-Timer;

-	logicalChannelSR-ProhibitTimer;

-	logicalChannelGroup.

Each logical channel may be allocated to an LCG using the logicalChannelGroup. The maximum number of LCGs is eight.

The MAC entity determines the amount of UL data available for a logical channel according to the data volume calculation procedure in TSs 38.322 and 38.323 [3] [4].

A BSR shall be triggered if any of the following events occur:

-	the MAC entity has new UL data available for a logical channel which belongs to an LCG; and either

-	the new UL data belongs to a logical channel with higher priority than the priority of any logical channel containing available UL data which belong to any LCG; or

-	none of the logical channels which belong to an LCG contains any available UL data;

in which case the BSR is referred below to as 'Regular BSR';

-	UL resources are allocated and number of padding bits is equal to or larger than the size of the Buffer Status Report MAC CE plus its subheader, in which case the BSR is referred below to as 'Padding BSR';

-	retxBSR-Timer expires, and at least one of the logical channels which belong to an LCG contains UL data, in which case the BSR is referred below to as 'Regular BSR';

-	periodicBSR-Timer expires, in which case the BSR is referred below to as 'Periodic BSR'.

For Regular BSR, the MAC entity shall:

1>	if the BSR is triggered for a logical channel for which logicalChannelSR-ProhibitTimer is configured by upper layers:

2>	start or restart the logicalChannelSR-ProhibitTimer;

1>	else:

2>	if running, stop the logicalChannelSR-ProhibitTimer.

For Regular and Periodic BSR, the MAC entity shall:

1>	if more than one LCG has data available for transmission in the NR-UNIT where the BSR is transmitted:

2>	report Long BSR for all LCGs which have data available for transmission;

1>	else:

2>	report Short BSR.

Editor's note: as a baseline, short BSR is reported when a single LCG has data available.

For Padding BSR:

1>	if the number of padding bits is equal to or larger than the size of the Short BSR plus its subheader but smaller than the size of the Long BSR plus its subheader:

2>	if more than one LCG has data available for transmission in the NR-UNIT where the BSR is transmitted:

3>	if the number of padding bits is equal to the size of the Short BSR plus its subheader:

4>	report Short Truncated BSR of the LCG with the highest priority logical channel with data available for transmission;

3>	else:

4>	report Long Truncated BSR of the LCG with the logical channels having data available for transmission following a decreasing order of priority;

2>	else:

3>	report Short BSR;

1>	else if the number of padding bits is equal to or larger than the size of the Long BSR plus its subheader:

2>	report Long BSR.

Editor's note: as a baseline, short BSR (including padding) is reported when a single LCG has data available. The details on padding procedures can also be discussed by RAN2.

The MAC entity shall:

1>	if the Buffer Status reporting procedure determines that at least one BSR has been triggered and not cancelled:

2>	if UL-SCH resources are available for a new transmission in this NR-UNIT:

Editor's note: The term NR-UNIT is used tentatively instead of TTI.

3>	instruct the Multiplexing and Assembly procedure to generate the BSR MAC CE(s);

3>	start or restart periodicBSR-Timer;

Editor's note: additional condition to not (re)start the periodic timer in LTE (i.e. truncated BSR) is not captured, and RAN2 should discuss detailed further.

3>	start or restart retxBSR-Timer.

2>	else if a Regular BSR has been triggered and logicalChannelSR-ProhibitTimer is not running:

3>	if an uplink grant is not configured (i.e. Semi-Persistent Scheduling); or

3>	if the Regular BSR was not triggered for a logical channel for which logical channel SR masking (logicalChannelSR-Mask) is setup by upper layers:

4>	trigger a Scheduling Request.

A MAC PDU shall contain at most one BSR MAC CE, even when multiple events have triggered a BSR by the time. The Regular BSR and the Periodic BSR shall have precedence over the padding BSR.

The MAC entity shall restart retxBSR-Timer upon reception of a grant for transmission of new data on any UL-SCH.

Editor's note: The BSR cancellation part needs to be captured.

# 6	Protocol Data Units, formats and parameters

## 6.1	Protocol Data Units

### 6.1.3	MAC Control Elements (CEs)

#### 6.1.3.1	Buffer Status Report MAC CEs

Buffer Status Report (BSR) MAC CEs consist of either:

-	Short BSR format (fixed size); or

-	Long BSR format; or

-	Short Truncated BSR format (fixed size); or

-	Long Truncated BSR format.

Editor's Note: RAN2 can discuss whether the MAC subheader for Long BSR should be optimized further.

The BSR formats are identified by MAC PDU subheaders with LCIDs as specified in Table 6.2.1-2.

The fields in the BSR MAC CE are defined as follows:

-	LCG ID: The Logical Channel Group ID field identifies the group of logical channel(s) whose buffer status is being reported. The length of the field is 3 bits.

-	LCGi: For the Long BSR format, this field indicates the presence of the Buffer Size field for the logical channel group i. The LCGi field set to "1" indicates that the Buffer Size field for the logical channel group i is reported. The LCGi field set to "0" indicates that the Buffer Size field for the logical channel group i is not reported. For the Long Truncated BSR format, this field indicates whether logical channel group i has data available. The LCGi field set to "1" indicates that logical channel group i has data available. The LCGi field set to "0" indicates that logical channel group i does not have data available;

-	Buffer Size: The Buffer Size field identifies the total amount of data available according to the data volume calculation procedure in TSs 38.322 and 38.323 [3] [4] across all logical channels of a logical channel group after all MAC PDUs for the NR-UNIT have been built (i.e. after the logical channel prioritization procedure). The amount of data is indicated in number of bytes. The size of the RLC and MAC headers are not considered in the buffer size computation. The length of this field for the Short BSR format and the Short Truncated BSR format is 5 bits. The length of this field for the Long BSR format and the Long Truncated BSR format is 8 bits. The values for the 5-bit and 8-bit Buffer Size fields are shown in Tables 6.1.3.1-1 and 6.1.3.1-2, respectively. For the Long BSR format and the Long Truncated BSR format, the Buffer Size fields are included in ascending order based on the LCGi. For the Long Truncated BSR format the number of Buffer Size fields included is maximised, while not exceeding the number of padding bits.

NOTE: The number of BS fields in the Long Truncated BSR format can be zero.

Figure 6.1.3.1-1: Short BSR and Short Truncated BSR MAC CE

Figure 6.1.3.1-2: Long BSR and Long Truncated BSR MAC CE

Table 6.1.3.1-1: Buffer size levels (in bytes) for 5-bit Buffer Size field

| Index | BS value | Index | BS value | Index | BS value | Index | BS value |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 0 | 0 | 8 |  | 16 |  | 24 |  |
| 1 |  | 9 |  | 17 |  | 25 |  |
| 2 |  | 10 |  | 18 |  | 26 |  |
| 3 |  | 11 |  | 19 |  | 27 |  |
| 4 |  | 12 |  | 20 |  | 28 |  |
| 5 |  | 13 |  | 21 |  | 29 |  |
| 6 |  | 14 |  | 22 |  | 30 |  |
| 7 |  | 15 |  | 23 |  | 31 |  |

Table 6.1.3.1-2: Buffer size levels (in bytes) for 8-bit Buffer Size field

| Index | BS value | Index | BS value | Index | BS value | Index | BS value |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 0 | 0 | 64 |  | 128 |  | 192 |  |
| 1 |  | 65 |  | 129 |  | 193 |  |
| 2 |  | 66 |  | 130 |  | 194 |  |
| 3 |  | 67 |  | 131 |  | 195 |  |
| 4 |  | 68 |  | 132 |  | 196 |  |
| 5 |  | 69 |  | 133 |  | 197 |  |
| 6 |  | 70 |  | 134 |  | 198 |  |
| 7 |  | 71 |  | 135 |  | 199 |  |
| 8 |  | 72 |  | 136 |  | 200 |  |
| 9 |  | 73 |  | 137 |  | 201 |  |
| 10 |  | 74 |  | 138 |  | 202 |  |
| 11 |  | 75 |  | 139 |  | 203 |  |
| 12 |  | 76 |  | 140 |  | 204 |  |
| 13 |  | 77 |  | 141 |  | 205 |  |
| 14 |  | 78 |  | 142 |  | 206 |  |
| 15 |  | 79 |  | 143 |  | 207 |  |
| 16 |  | 80 |  | 144 |  | 208 |  |
| 17 |  | 81 |  | 145 |  | 209 |  |
| 18 |  | 82 |  | 146 |  | 210 |  |
| 19 |  | 83 |  | 147 |  | 211 |  |
| 20 |  | 84 |  | 148 |  | 212 |  |
| 21 |  | 85 |  | 149 |  | 213 |  |
| 22 |  | 86 |  | 150 |  | 214 |  |
| 23 |  | 87 |  | 151 |  | 215 |  |
| 24 |  | 88 |  | 152 |  | 216 |  |
| 25 |  | 89 |  | 153 |  | 217 |  |
| 26 |  | 90 |  | 154 |  | 218 |  |
| 27 |  | 91 |  | 155 |  | 219 |  |
| 28 |  | 92 |  | 156 |  | 220 |  |
| 29 |  | 93 |  | 157 |  | 221 |  |
| 30 |  | 94 |  | 158 |  | 222 |  |
| 31 |  | 95 |  | 159 |  | 223 |  |
| 32 |  | 96 |  | 160 |  | 224 |  |
| 33 |  | 97 |  | 161 |  | 225 |  |
| 34 |  | 98 |  | 162 |  | 226 |  |
| 35 |  | 99 |  | 163 |  | 227 |  |
| 36 |  | 100 |  | 164 |  | 228 |  |
| 37 |  | 101 |  | 165 |  | 229 |  |
| 38 |  | 102 |  | 166 |  | 230 |  |
| 39 |  | 103 |  | 167 |  | 231 |  |
| 40 |  | 104 |  | 168 |  | 232 |  |
| 41 |  | 105 |  | 169 |  | 233 |  |
| 42 |  | 106 |  | 170 |  | 234 |  |
| 43 |  | 107 |  | 171 |  | 235 |  |
| 44 |  | 108 |  | 172 |  | 236 |  |
| 45 |  | 109 |  | 173 |  | 237 |  |
| 46 |  | 110 |  | 174 |  | 238 |  |
| 47 |  | 111 |  | 175 |  | 239 |  |
| 48 |  | 112 |  | 176 |  | 240 |  |
| 49 |  | 113 |  | 177 |  | 241 |  |
| 50 |  | 114 |  | 178 |  | 242 |  |
| 51 |  | 115 |  | 179 |  | 243 |  |
| 52 |  | 116 |  | 180 |  | 244 |  |
| 53 |  | 117 |  | 181 |  | 245 |  |
| 54 |  | 118 |  | 182 |  | 246 |  |
| 55 |  | 119 |  | 183 |  | 247 |  |
| 56 |  | 120 |  | 184 |  | 248 |  |
| 57 |  | 121 |  | 185 |  | 249 |  |
| 58 |  | 122 |  | 186 |  | 250 |  |
| 59 |  | 123 |  | 187 |  | 251 |  |
| 60 |  | 124 |  | 188 |  | 252 |  |
| 61 |  | 125 |  | 189 |  | 253 |  |
| 62 |  | 126 |  | 190 |  | 254 |  |
| 63 |  | 127 |  | 191 |  | 255 |  |

Editor's Note: whether to have two BS tables or one BS table should also be discussed further.