3GPP TSG-RAN WG2 #100	R2-1714153

Reno, Nevada, USA, 27th November – 1st December 2017

Source:	NTT DOCOMO, INC.,

Title:	Text Proposal for UE Identification

Agenda Item:	9.4.5

Document for:	Discussion and Decision

Introduction

This document proposes the TP for Aerial UE Identification section.

TP

**************start************************

- 7.X	Potential enhancements for aerial UE Identification

Editor’s note: This chapter is a place holder to capture potential solution for identifying aerial UE in airborne condition and for identification of certification/license of aerial UE.

- 7.x.y Aerial UE Identification based on certification/license

A UE indicates support for the aerial features in the 3GPP network using radio capability indication to the network.  The network may use this capability to identify an aerial UE, apply the relevant functions and perform the necessary control.

Permission for a user to function as an aerial UE in the 3GPP network can be identified via subscription information. This subscription information is passed onto RAN via S1 signalling from the MME. The eNB may use this information together with other indication as described in this section to perform the necessary control.

The actual “aerial usage” certification/license/limitation of a UE and how it is reflected in the subscription information is outside of RAN2 scope. The actual certification/license/other limitation may be provided from (non)-3GPP node(s) to a 3GPP node.

*******************end*****************************