3GPP TSG-RAN WG2 #100	Tdoc R2-1714165

Reno, USA, 27th November – 1st December 2017

Agenda Item:	10.2.2.1

Source:	Intel Corporation (rapporteur)

Title:	Offline discussion on Bearer type redefinitions

Document for:	Discussion, Decision

# Introduction

It is proposed that RAN2 conclude the following definitions:

Proposal #1: it is proposed to agree on Option 1: From R2-1712669 and R3-1713906:

- MCG bearer: in MR-DC, a bearer with logical channel configuration only in the MCG.

- SCG bearer: in MR-DC, a bearer with logical channel configuration only in the SCG.

- Split bearer: in MR-DC, a bearer with logical channel configuration in both SCG and MCG.

- MN terminated bearer: in MR-DC, a bearer for which PDCP is located in the MN.

- SN terminated bearer: in MR-DC, a bearer for which PDCP is located in the SN.

Proposal #2: 2x/2c (i.e., MN terminated SCG bearer and SN terminated MCG bearer) are supported in stage 2.

Proposal #3: Discuss further to define a term for “logical channel configuration” in the above definitions.