3GPP TSG-RAN WG2 #100	Tdoc R2-1714188

Reno, Nevada, USA, 27th November – 1st December 2017

Agenda Item:	10.4.1.4.1

Source:	Ericsson

Title:	Summary of offline#40 (neighbour cells on serving frequencies)

Document for:	Discussion, Decision

# Introduction

In RAN2#100, a contribution on measurement reporting was discussed [1] and, based on that, the following offline discussion was triggered:

=>	Offline discussion to conclude on reporting of RSType(s) and measurement quantities and beam level measurements for best neighbour cell reporting. (Offline discussion #40, Ericsson)

This contribution summarizes the discussions and propose a way forward.

# Summary

In RAN2#99-bis meeting in Prague, the following was agreed:

Agreements:

9    Network can configure the UE to report the best neighbour cells in the serving frequencies.

In the email discussion #20, it has been identified by several companies that there are still some remaining details related to the reporting of best neighbour cells on serving frequencies. This is captured in the section 5.5.5 of the TP:

Editor’s Note: FFS Details of the information to be reported concerning best neighbouring cells in the serving frequencies e.g. which RS type, which quantities, whether beam reporting is supported, etc.

- Two options have been discussed:

- Option a/ Report all available measurements of best neighbours on serving frequencies.

- Cell and beam for all available measurement quantities.

- Option b/ Report available measurements of best neighbours on serving frequencies as in reportConfig:

- Include available beam measurement information if in reportConfig. If configured, same beam info is reported as in reportConfig (e.g. only indexes, same quantity(ies), same rsType, etc.)

- Include the quantities indicated in reportConfig.

- Include only available measurements associated to the same RS type indicated in reportConfig (e.g. if report config associated to the report has rsType set to SSB, report only SSB. If the report config associated to the report has rsType set to CSI-RS, report only CSI-RS).

- Based on the discussions, there seemed to be a reasonable consensus for option b/ as a way forward.

# Conclusion

Based on the summary in section 2 we propose the following:

Proposal 1	Report available measurements of best neighbours on serving frequencies as in reportConfig:

- Include available beam measurement information if in reportConfig. If configured, same beam info is reported as in reportConfig (e.g. only indexes, same quantity(ies), same rsType, etc.)

- Include the quantities indicated in reportConfig.

- Include only available measurements associated to the same RS type indicated in reportConfig (e.g. if report config associated to the report has rsType set to SSB, report only SSB. If the report config associated to the report has rsType set to CSI-RS, report only CSI-RS).

# References

R2-1713598, Remaining issues of email discussion # 20 on measurement reporting, Ericsson, RAN2#100 Reno