3GPP TSG RAN WG2 Meeting 100 			R2-1714243

Reno, USA, 27th Nov – 1st Dec 2017

Title:	DRAFT LS on MAC CEs for beam management and CSI

Release:	Rel-15

Work Item:	NR_newRAT-Core

Source:	RAN WG2

To:	RAN WG1

Cc:	-

Contact Person:

#### Name:	Helka-Liina Määttänen

E-mail Address:	helka-liina.maattanen@ericsson.com

1. Overall Description:

RAN2 discussed the MAC CEs based on R1-1721661 and identified the following questions:

Activating SP DL RS and CSI:

| Semi-persistent CSI-RS / CSI-IM | Activates/deactivates a SP CSI-RS resource set and a SP CSI-IM resource set. Provides the QCL relationship (if activated) | SP CSI-RS Resoruce Set Id ¦ SP CSI-IM Resource Set Id ¦ TCI_State_Id |
| --- | --- | --- |

Question 1: Is a TCI_State_Id an ID of of the M configured TCI states or of the N^2 down selected TCI states?

Question 2: Is M fixed or can it be variable size? What is the (maximum) value of M?

Question 3: As QCL for SP CSI-RS can only be SSB, can SSB index used instead especially if M>>64?

Question 4: Is SP CSI-IM always activated with SP CSI-RS or can it be optionally present?

Agreement:

The following P/SP/AP CSI-RS signaling options are supported:

| QCL parameter | Reference RS | Target RS | Signalling mode | Reference RS and Target RS should belong to the same CC/BWP or not |
| --- | --- | --- | --- | --- |
| Spatial | SSB | P CSI-RS | RRC | Can be on different CCs/BWPs |
| Spatial | SSB | SP CSI-RS | SP CSI-RS activation signal | Can be on different CCs/BWPs |
| Spatial | P CSI-RS | Another P CSI-RS | RRC | Can be on different CCs/BWPs |

Activating SP UL RS:

| Semi-persistent SRS activation | Activates a SP SRS resource set and provides the spatial QCL relationship (if activated) | SP SRS Resoruce Set Id ¦ SSB ID/SRS resource ID/CSI-RS resource ID |
| --- | --- | --- |

Question 5: For each SP SRS resource set, is the QCL  an RS ID type (SSB/SRS resource/CSI-RS resource) and RS ID value or is it a TCI state?

Question 6: If TCI state is used, is it a TCI state out of any of the M configured TCI states or one of the N^2 down selected TCI states?

2. Actions:

RAN2 would like to ask RAN1 clarify the above questions. Additionally, for all MAC CEs listed in R1-1721661, please provide information whether the MAC CE has fixed size or variable size. Please give size of all the subfields as well as for the total field. Further, please indicate which L1 parameters are related to the MAC CE using exact parameter names from R1-1719429 .

RAN2 would also like RAN1 to consider to minimize the requested size of the MAC CE.

3. Date of Next TSG-RAN WG2 Meetings:

TSG RAN WG2 Meeting NR Ad hoc 1801	22nd – 26th January 2018		Vancouver, Canada

TSG RAN WG2 Meeting 101	26th February – 2nd March 2018	Athens, Greece