3GPP TSG-RAN2#101bis                                             R2-1804294

Sanya, China, 16th-20th Apr. 2018

Title: 	Interaction between PDCP and RLC Entities for duplication in NR-NR DC

Source: 	TCL

Agenda item:	10.3.3.5

Document for:	Discussion and Decision

# Introduction

In RAN2 #99 meeting, it was agreed that in DC scenario, UE will fall back to the split bearer operation when DRB is deactivated.

Agreements: 1. For DC, when DRB duplication is deactivated via MAC CE, the UE falls back to the split bearer operation. Once de-activated we rely on split bearer operation and configuration. 2. 1 byte bitmap could be used as duplication activation/deactivation MAC CE 3. The mapping between DRB and the MAC bitmap is based on order of DRB ID(s) of the duplicate configured DRB(s)

In RAN2#AH-1801 meeting, it was agreed that the UE shall discard packets that have been acknowledged by RLC in the other RLC leg.

Agreements:

1.  Upon packet duplication activation, only PDCP SDUs/PDUs not submitted to lower layers are duplicated.

2.  Baseline is that packed duplication is support for data PDUs

3.  For packet duplication, when to submit PDCP PDUs to lower layers is up to UE implementation.  FFS on UE behaviour when duplication is deactivated and what PDCP data volume is used.

4.  After packet duplication is activated, for DC duplication, PDCP data volume is indicated to both the MAC entity associated with the primary RLC entity and the MAC entity associated with the secondary RLC entity

5.  After packet duplication is activated, for CA duplication, PDCP data volume is included in both the LCG associated with the primary RLC entity and the LCG associated with the secondary RLC entity.

6.  Packet duplication does not impact RLC data volume

7.  The UE shall discard packets that have been acknowledged by RLC in the other RLC leg.   PDCP should indicate to the other associated RLC entity to discard the corresponding PDCP PDU.  RLC procedures and PDCP discard procedures are not impacted by this agreement.

8.  The deactivated RLC entity is not re-established

9.  For CA and DC upon deactivation of PDCP data duplication, the UE transmitting PDCP entity should indicate to lower layers to discard all PDCP PDUs provided for duplicate transmission to the secondary RLC entity

10.  When configuring duplication, RRC can also set the initial state (active or inactive) for DRBs.

11.  If SRB is configured to use duplication, the state is always active

12.  FFS Duplication is supported for SRBs for CA

Packet duplication was introduced for NR during the study item, where it was agreed to be implemented in the PDCP sub-layer, associated with two independent RLC entities [1]. However, the PDCP operation upon activation of packet duplication has not been discussed yet. Such as, when the PDCP entity has been acknowledged (e.g. by RLC/PDCP status report) that a PDCP PDU has been successfully delivered by a leg, whether its duplicate needs to be transmitted in the other leg. To avoid the unnecessary waste of resources, some companies have been proposed that the UE stop the transmission of the duplicated PDCP PDU in the other leg [2]. This contribution will discuss the details of PDCP operation for packet duplication.

# Discussion

Fig.1 PDCP duplication in two RLC entities for DC case

Duplication is activated when more reliability is needed for the transmission, in case for example of bad radio conditions. For a DRB/SRB which may need packet duplication to improve reliability, the gNB should first configure two RLC entities for this radio bearer. The duplicate operation can be activated/deactivated by a MAC CE. As can be seen, when a DRB is configured to support packet duplication, and the duplication function is activated, PDCP duplicates any generated PDCP PDU and delivers each instance to one RLC entity. Both RLC entities are assumed running independently from each other.

## Duplication Success

For NR-NR DC. One of the key motivations of the duplication feature is to take profit of multiples legs to deliver to upper layers packets from the always fastest leg to meet the latency requirement. However, An issue is, when the PDCP entity has been acknowledged (e.g. by RLC/PDCP status report) that a PDCP PDU has been successfully delivered by a leg, whether its duplicate needs to be transmitted in the other leg. Moreover the DC architecture results in two potential issues, RLC Tx buffer overflow and RLC AM ARQ window mismatch with actual RLC SDU receiving status[3]. To avoid the unnecessary waste of resources, we prefer the “slower” packets need to be discarded at some point and the slower route must somehow be aware of this. Thus, we propose that upon indication from any of the leg that a PDCP PDU has been correctly transmitted, PDCP should instruct the other legs not to transmit the other duplicates. The easiest way to perform such an action would be to rely on PDCP discard, which currently informs the corresponding RLC entity not to transmit the packet. In other words, indication of correct transmission via any of the legs, will trigger PDCP to signal a PDCP PDU discard to all the other legs.

Proposal 1: when PDCP is informed of the reception of a duplicate via one leg, the corresponding outdated duplicates are discarded on the other legs via discard notification.

## RLC ARQ window update

In the same slow leg / fast leg scenario, when RLC is configured in AM mode, the lower bound of the ARQ window will be outdated in the slow leg with respect to the fast leg. This will result in triggering unnecessary status reports in the slow leg ARQ as well as unnecessary retransmissions resulting in further increasing the slow leg congestion. In case the leg experiences deep channel degradation, the missing PDU may be re-transmitted a large number of times in this leg although it has been received on the other leg. Therefore, the ARQ window in the slow leg should be updated to allow it moving up its lower bound, thus reflecting the overall reception status of the duplicated bearer.

Observation 1: Packet duplication may lead to RLC ARQ windows mismatch between slow and fast legs resulting in slow leg triggering unnecessary status reports and retransmissions thus congesting further the slow leg.

## 2.3 Potential enhancement

We suggest solving the above issues with the below procedure to avoid Tx buffer overflow and AM ARQ window mismatch with actual RLC SDU receiving status when one leg works fine and the other is stuck.

The proposed solution includes the following parts:

Step1: PDCP status report and data conformance report.

  - Rx PDCP send PDCP status report to Tx PDCP reflecting successful PDCP SN receptions, Similar to RLC, these can be triggered periodically or when receiving polling from PDCP peer based on PDU number or PDU bytes.

  - At the same time, PDCP Rx sends the data conformance report to the SCG RLC Rx and MCG RLC Rx over Xn or intra cross-layer interactions. The data conformance report is used to inform RLC of slower leg some RLC SDUs has been received.

Step 2: After receiving the status report from Rx PDCP, Tx PDCP notifies Tx RLC of the successful PDCP SNs in all Tx RLC entities of active legs for this RB.

Step 3: Tx RLC identifies RLC SNs associated with ACK’ed PDCP SNs(from a mapping table between PDCP and RLC SNs), and discards pending RLC SDUs not yet transmitted.

Step 4: After receiving the data conformance report from Rx PDCP, Rx RLC identifies RLC SNs associated with PDCP SNs from data conformance report (from a mapping table between PDCP and RLC SNs) and moves its Rx ARQ window accordingly.

Step 5: After update the Tx buffer and Rx ARQ windows, Rx RLC need to update the mapping table between PDCP and RLC SNs in RLC Rx.

(1) UL DRB duplication process

In UL DRB duplication process which is illustrated in Fig.2. Assume the RLC1 Tx is the main RLC corresponded to MCG RLC Rx, and the RLC2 Tx is the duplication RLC corresponded to SCG RLC Rx. If the RLC1 Tx is the faster leg. The PDCP Rx needs to send the data conformance report to SCG RLC Rx over Xn.

Fig.2 UL DRB duplication process

(2) DL DRB duplication process

In DL DRB duplication process which is illustrated in Fig.3. Assume the MCG RLC Tx is the main RLC corresponded to RLC1 Rx, and the SCG RLC Tx is the duplication RLC corresponded to RLC2 Rx. If the MCG RLC Tx is the faster leg. The PDCP Rx needs to send the data conformance report to RLC2 Rx over intra cross-layer interactions.

Fig.3 DL DRB duplication process

Rx PDCP needs to send data conformance report to Rx RLC reflecting successful PDCP SN receptions, these can be sent simultaneously with the PDCP status report , which can be triggered periodically or when receiving polling from PDCP peer based on PDU number or PDU bytes.

Proposal 2: Rx PDCP needs to send data conformance report to Rx RLC reflecting successful PDCP SN receptions.

In Rx RLC layer, when received data conformance report from Rx PDCP, Rx RLC have to get RLC SDU SN from the PDCP SN which is get from the data conformance report. So Rx RLC has to maintain the PDCP-RLC SNs mapping table to update the ARQ window.

Proposal 3: Tx and Rx RLC entities need to maintain and update the PDCP-RLC SNs mapping table.

# Conclusion and Proposals

Proposal 1: when PDCP is informed of the reception of a duplicate via one leg, the corresponding outdated duplicates are discarded on the other legs via discard notification.

Observation 1: Packet duplication may lead to RLC ARQ windows mismatch between slow and fast legs resulting in slow leg triggering unnecessary status reports and retransmissions thus congesting further the slow leg.

Proposal 2: Rx PDCP needs to send data conformance report to Rx RLC reflecting successful PDCP SN receptions.

Proposal 3: Tx and Rx RLC entities need to maintain and update the PDCP-RLC SNs mapping table.

# Acknowledgement

This document is also contributed in part by SARI (Shanghai Advanced Research Institute, Chinese Academy of Sciences).

# Reference

- 3GPP TR 38.804 v14.0.0, “Study on New Radio Access Technology; Radio Interface Protocol Aspects” ETSI;

- R2-1707719 PDCP operation for packet duplication, Huawei, ASUSTeK, HiSilicon

- R2-1704249 PDCP and RLC enhancements in support of duplication，CATT