3GPP TSG-RAN WG2 #101bis	R2-1804395

16th – 20th April 2018

Sanya, China

Source:                    	NTT DOCOMO, INC.

Title:  	Multiple NS/P-Max for NR

Document for:        	Discussion and decision

Agenda Item:         	10.4.1.6.8	System information - other

- Introduction

RAN4 asked RAN2 to introduce multiple NS/P-Max as shown below [1].

Multiple NS/P-Max;

  - Although RAN4 informed in R2-1711987 that multiple NS value signaling is to be supported, RAN2 would like to confirm if multiple P-Max signaling should also be supported together with multiple NS value signaling and the corresponding cell selection mechanism.

Similar to the LS (R4-152410) for LTE, multiple P-Max together with the corresponding cell selection mechanism is also beneficial for NR to avoid the situation that when the existing P-max is used to make the legacy UEs satisfy regulatory requirements, the P-max affects the new UEs as well. The new P-Max value should only be used in conjunction with the new/modified NS value.

This paper discusses how to implement multiple NS/P-Max in accordance with RAN4 feedback.

- Discussion

In v15.1.0 of TS 38.331, AdditionalSpectrumEmission (a.k.a. NS value) and P-Max are included in FrequencyInfoUL as shown below (unrelated fields are removed).

FrequencyInfoUL ::= 				SEQUENCE {

-- The additional spectrum emission requirements to be applied by the UE on this uplink.

-- If the field is absent, the UE applies the value FFS_RAN4. (see FFS_section, section FFS_Section)

additionalSpectrumEmission			AdditionalSpectrumEmission		OPTIONAL,	-- Need S

-- FFS_Definition. Corresponds to parameter FFS_RAN4. (see FFS_Spec, section FFS_Section)

-- If the field is absent, the UE applies the value FFS_RAN4.

p-Max								P-Max							OPTIONAL,	-- Need S

...

}

FrequencyInfoUL is included in ServingCellConfigCommon as a field of UplinkConfigCommon. Given that ServingCellConfigCommon is supposed to be present in SIB1, it is intuitive that the list of multiple NS and P-Max is added into FrequencyInfoUL in a backward compatible manner.

On the other hand, RAN4 also asked to support Multiple Frequency Band Indicator (MFBI) for NR [1]. In fact, MFBI has already been ready to support in v15.1.0 of TS 38.331 as shown below. At RAN2 #101, RAN2 agreed that only one band is present in case of HO and S(p)Cell addition.

FrequencyInfoDL ::= 				SEQUENCE {

-- List of one or multiple frequency bands to which this carrier(s) belongs. Multiple values are only supported in

-- system information but not when the FrequencyInfoDL is provided in dedicated signalling (HO or S(p)Cell addition).

frequencyBandList					MultiFrequencyBandListNR,

...

}

To make the structure consistent with MFBI, non-backward compatible approach could be considered to include all NS values and P-Max into a new list. Nonetheless, by recalling the background of introducing multiple NS/P-Max for LTE, it is noted that:

-	When a new frequency band is started to use in a network, only one NS value (together with P-Max if needed) is needed and broadcast.

-	After the initial launch, Additional NS value(s) and associated P-Max are introduced due to regulation change, improvement of UR RF characteristics, etc.

It is not likely that multiple NS and P-Max are used at the initial service for a new frequency band. In that sense, non-backward compatible change seems not justified for introducing multiple NS/P-Max. The following is proposed:

Proposal 1:	The list of multiple NS/P-Max is introduced into FrequencyInfoUL in a backward compatible manner.

Multiple NS/P-Max in LTE affects the cell selection mechanism, i.e. Pcompensation as defined in TS 36.304. In fact, it was FFS at the January AH meeting as shown below.

-	From RAN2 perspective compensation parameter(s) is needed to S criterion. Details can be FFS and depending on the input from RAN1/4.

As RAN4 answered in their LS, “the corresponding cell selection mechanism is also beneficial for NR to avoid the situation that when the existing P-max is used to make the legacy UEs satisfy regulatory requirements, the P-max affects the new UEs as well.”, the same formula needs to be defined for Pcompensation. The following is proposed:

Proposal 2:	For cell selection criterion, the formula of Pcompensation covers the multiple NS/P-Max case as in LTE.

- Summary and proposal

In summary, the followings were proposed:

Proposal 1:	The list of multiple NS/P-Max is introduced into FrequencyInfoUL in a backward compatible manner.

Proposal 2:	For cell selection criterion, the formula of Pcompensation covers the multiple NS/P-Max case as in LTE.

TP to introduce multiple NS/P-Max is provided in the Annex section in this paper.

- References

[1] R2-1804231, “LS reply on UE RF related parameters, capabilities and features for NR standalone,” RAN4.

Annex A:	Text proposals to introduce multiple NS/P-Max

A1:	Text proposal to 38.331 (based on v15.1.0)

Editor’s note:	Procedure text on multiple NS/P-Max will be added on top of TP discussed by email discussion [101#38][NR] SI procedure text.

### 6.3.2	Radio resource control information elements

<< skip unchanged part >>

#### –	FrequencyInfoUL

The IE FrequencyInfoUL provides basic parameters of an uplink carrier and transmission thereon.

FrequencyInfoUL information element

-- ASN1START

-- TAG-FREQUENCY-INFO-UL-START

FrequencyInfoUL ::= 				SEQUENCE {

-- List of one or multiple frequency bands to which this carrier(s) belongs. Multiple values are only supported in

-- system information but not when the FrequencyInfoDL is provided in dedicated signalling (HO or S(p)Cell addition).

frequencyBandList					MultiFrequencyBandListNR										OPTIONAL,	-- Cond FDD-OrSUL

-- Absolute frequency of the reference resource block (Common RB 0). Its lowest subcarrier is also known as Point A.

-- Corresponds to L1 parameter 'offset-ref-low-scs-ref-PRB' (see 38.211, section FFS_Section)

absoluteFrequencyPointA						ARFCN-ValueNR					OPTIONAL,	-- Cond FDD-OrSUL

-- A set of virtual carriers for different subcarrier spacings (numerologies). Defined in relation to Point A.

-- Note that the lower edge of the actual carrier is not defined by this field but rather in the scs-SpecificCarrierList.

-- Corresponds to L1 parameter 'offset-pointA-set' (see 38.211, section FFS_Section)

scs-SpecificCarriers				SEQUENCE (SIZE (1..maxSCSs)) OF SCS-SpecificCarrier,

-- The additional spectrum emission requirements to be applied by the UE on this uplink.

-- If the field is absent, the UE applies the value FFS_RAN4. (see FFS_section, section FFS_Section)

additionalSpectrumEmission			AdditionalSpectrumEmission										OPTIONAL,	-- Need S

-- FFS_Definition. Corresponds to parameter FFS_RAN4. (see FFS_Spec, section FFS_Section)

-- If the field is absent, the UE applies the value FFS_RAN4.

p-Max								P-Max															OPTIONAL,	-- Need S

-- Enable the NR UL transmission with a 7.5KHz shift to the LTE raster. If the field is absent, the frequency shift is disabled.

frequencyShift7p5khz				ENUMERATED {true}												OPTIONAL,	-- Cond FDD-OrSUL-Optional

...,

[[	additionalNS-PmaxList			MultiNS-PmaxListNR					OPTIONAL	-- Need R

]]

}

-- TAG-FREQUENCY-INFO-UL-STOP

-- ASN1STOP

| Conditional Presence | Explanation |
| --- | --- |
| FDD-OrSUL | The field is mandatory present if this FrequencyInfoUL is for the paired UL for a DL (defined in a FrequencyInfoDL) or if this FrequencyInfoUL is for a supplementary uplink (SUL). It is absent otherwise (if this FrequencyInfoUL is for an unpaired UL (TDD). |
| FDD-OrSUL-Optional | The field is optionally present, Need R, if this FrequencyInfoUL is for the paired UL for a DL (defined in a FrequencyInfoDL) or if this FrequencyInfoUL is for a supplementary uplink (SUL). It is absent otherwise. |

#### –	MultiNS-PmaxListNR

The IE MultiNS-PmaxListNR is used to configure a list of multiple additionalSpectrumEmission and P-Max.

MultiNS-PmaxListNR information element

-- ASN1START

-- TAG-MULTINS-PMAXLISTNR-START

MultiNS-PmaxListNR ::= 		SEQUENCE (SIZE (1..maxNrofNS-Pmax)) OF NS-PmaxValueNR

NS-PmaxValueNR ::=	SEQUENCE {

additionalSpectrumEmission		AdditionalSpectrumEmission,

p-Max							P-Max								OPTIONAL	-- Need S

}

-- TAG-MULTINS-PMAXLISTNR-STOP

-- ASN1STOP

A2:	Text proposal to 38.304 (based on v1.0.0)

#### 5.2.3.2	Cell Selection Criterion

The cell selection criterion S in normal coverage is fulfilled when:

Srxlev > 0 AND Squal > 0

where:

Srxlev = Qrxlevmeas – (Qrxlevmin + Qrxlevminoffset )– Pcompensation Squal = Qqualmeas – (Qqualmin + Qqualminoffset)

where:

| Srxlev | Cell selection RX level value (dB) |
| --- | --- |
| Squal | Cell selection quality value (dB) |
| Qrxlevmeas | Measured cell RX level value (RSRP) |
| Qqualmeas | Measured cell quality value (RSRQ) |
| Qrxlevmin | Minimum required RX level in the cell (dBm) |
| Qqualmin | Minimum required quality level in the cell (dB) |
| Qrxlevminoffset | Offset to the signalled Qrxlevmin taken into account in the Srxlev evaluation as a result of a periodic search for a higher priority PLMN while camped normally in a VPLMN [9] |
| Qqualminoffset | Offset to the signalled Qqualmin taken into account in the Squal evaluation as a result of a periodic search for a higher priority PLMN while camped normally in a VPLMN [9] |
| Pcompensation | If the UE supports the p-Max in the MultiNS-PmaxListNR, if present, in SIBX [TBD]: max(PEMAX1 –PPowerClass, 0) – (min(PEMAX2, PPowerClass) – min(PEMAX1, PPowerClass)) (dB); else: max(PEMAX1 –PPowerClass, 0) (dB); |
| PEMAX1, PEMAX2 | Maximum TX power level an UE may use when transmitting on the uplink in the cell (dBm) defined as PEMAX in TS 36.101 [x]. PEMAX1 and PEMAX2 are obtained from the p-Max and the MultiNS-PmaxListNR respectively in SIBX [TBD] as specified in TS 38.331 [x]. |
| PPowerClass | Maximum RF output power of the UE (dBm) according to the UE power class as defined in TS 38.101 [x] |

The signalled values Qrxlevminoffset and Qqualminoffset are only applied when a cell is evaluated for cell selection as a result of a periodic search for a higher priority PLMN while camped normally in a VPLMN [23.122]. During this periodic search for higher priority PLMN the UE may check the S criteria of a cell using parameter values stored from a different cell of this higher priority PLMN.