3GPP TSG-RAN WG2 Meeting #101bis	R2-1804601

Sanya, China, 16th April - 20nd April 2018

Source:	vivo

Title:	Discussion on the IDC report of NR frequency in EN-DC

Agenda Item:	10.2.10

Document for:	Discussion and Decision

# Introduction

According to the LS from RAN4 on the IDC issue for EN-DC [3], RAN4 confirms that “IDC problems not only for other systems already identified in LTE CA but also for mmWave ISM bands are expected with EN-DC”, and suggests to reuse the current LTE IDC solution frame work. In the Annex we listed the potential NR bands including both FR1 and FR2 which could have the IDC problem. In this contribution, we provide the frequency-domain IDC solution for the NR band.

# Discussion

## Down-selection of the IDC solution for affected NR frequency

Although the IDC issue of the NR frequency is present for both NR SA and NR NSA according to the Annex A, we consider that the IDC reporting of the NR problematic frequency is only for the EN-DC case in Rel-15 due to the limited time in Rel-15.

Proposal 1: The IDC solution for the affected NR frequency is only for the EN-DC in Rel-15.

According to the LS from RAN4 [6], RAN4’s preference is to reuse the framework of the LTE IDC solutions. Regarding the LTE IDC solutions [4] [5], the UE encountering IDC issue can report some assistance information of TDM and/or FDM via RRC message, and the UE is also allowed to autonomously deny LTE uplink transmission according to the network configuration.  Then the solutions can be divided into the following two categories:

- Cat.1 (FDM): UE reporting of the victim frequency and system

- Cat.2 (TDM)

  - Cat.2.1: UE reporting for the desired DRX/subframe pattern

  - Cat.2.2: Allowed subframe pattern in which the UE is allowed to deny any UL transmission

According to the analysis given in [6] [7], to re-use the framework of the LTE IDC solutions, some further analysis/discussion/modifications are required for both FDM and TDM of the IDC solutions. For example, in order to allow the UE reporting for the NR band, the NR should be included as a victim system, and the reporting of the NR frequency should be allowed. For the TDM solutions, due to the flexible numerology (e.g. flexible time range of slot/OFDM symbol) configured for the UE, more discussions are required on how to allow the TDM solutions (e.g. detailed TDM pattern for both UE assistance information and the autonomous deny). Thus we consider that only the FDM solution (i.e. UE reporting of the NR frequency) is used in Rel-15. According to the EN-DC architecture, the NR SCG could be configured with SRB3 which can be used to report the assistance IDC information. However as NR has not defined any IDC assistance message in the NR RRC specification and allowing the report of the NR IDC issue via SRB3 will cause more specification impacts, we consider that the NR IDC assistance information is only reported via the LTE IDC assistance information.

Proposal 2: Only the IDC reporting of the affected NR frequency is specified in Rel-15.

Proposal 3: The IDC reporting of the affected NR frequency is via the LTE InDeviceCoexIndication message.

## Reporting content of NR problematic frequency

According to the LTE IDC solution framework for frequency reporting [4], the reported frequency can be either serving or non-serving frequencies, and the UE can report the following assistance information to the network:

- affectedCarrierFreqList: The affected LTE frequency band which is identified by MeasObjectId.

- interferenceDirection: To identify if the E-UTRAN or other RAT or both are victim RAT for the LTE frequency band indicated in affectedCarrierFreqList.

- affectedCarrierFreqCombList: To “Indicates a list of E-UTRA carrier frequencies that are affected by IDC problems due to Inter-Modulation Distortion and harmonics from E-UTRA when configured with UL CA”. The frequency are indicated by MeasObjectId.

- victimSystemType: To “Indicate the list of victim system types to which IDC interference is caused from E-UTRA when configured with UL CA”.

Regarding the sidelobe interference, we consider that the NR frequency should be reported in the affectedCarrierFreqList. According to the TS 38.331, one MeasObjectNR configured by the NR SN can include two frequencies (e.g. ssbFrequency and refFreqCSI-RS). Then the MeasObjectId for MeasObjectNR configured by the NR SN cannot uniquely identify and NR frequency. According to the TS 36.331, one MeasObjectNR configured by the LTE MN can only include one frequency (i.e. for SSB).

Observation 1: One MeasObjectNR configured by the LTE MN can only indicate one frequency.

Observation 2: One MeasObjectNR configured by the NR SN can indicate two frequencies.

To report the NR frequency configured by the LTE MN, the current affectedCarrierFreqList can be reused. This means that the MeasObjectId of current affectedCarrierFreqList is reused to report the NR frequency configured by the LTE MN. We only needs to add NR RAT as new type (which can use the spare value) in the interferenceDirection of affectedCarrierFreqList.

Proposal 4: The MeasObjectId of current affectedCarrierFreqList is reused to report the affected NR frequency configured by the LTE MN.

Proposal 5: The interferenceDirection of affectedCarrierFreqList adds “nr” as a new RAT type.

To report the NR frequency configured by the NR SN, we consider that a new affectedCarrierFreqListNR is needed as the MeasObjectId configured by the NR SN is independent from the MeasObjectId configured by the NR MN. This means that the MeasObjectId(s) configured by MN and SN could be collided with each other.

Proposal 6: The affected NR frequency configured by the NR SN is reported in a new list of affectedCarrierFreqListNR.

Proposal 7: Each reported NR frequency in affectedCarrierFreqListNR includes the following information:

- measObjectId-NR: The measurement object ID configured by the NR SN.

- rsType: Indicate if the affected frequency of the measurement object is SSB frequency or CSI-RS frequency.

- interferenceDirectionNR: Indicate if the NR or other RAT or both are victim RAT for the NR frequency indicated in affectedCarrierFreqListNR.

Regarding the inter-modulation interference due to UL CA, we consider that both NR frequency and LTE frequency should be included in the same band combination to report the affected EN-DC band combination. According to the analysis given above, as the measurement object are configured independently by LTE MN and NR SN, a new list of AffectedCarrierFreqCombListNR is needed to report the EN-DC band combination.

Proposal 8: The affected EN-DC band combination is reported in a new list of AffectedCarrierFreqCombListNR.

Regarding the reporting content of the each EN-DC band combination, as the analysis given above, we consider that the MeasObjectId configured by the MN can be used to uniquely identify either the LTE frequency or the NR frequency, and the reference signal type is needed to uniquely identify the affected NR frequency measurement object configured by the NR SN.

Proposal 9: Each frequency information of an affected EN-DC band combination in AffectedCarrierFreqCombListNR includes:

- measObjectId: The measurement object ID configured by either LTE MN or NR SN.

- measObjectType: Indicate if the reported measObjectId is configured by LTE MN or NR SN.

- rsType: Indicate if the affected frequency of the measurement object is SSB frequency or CSI-RS frequency when the measObjectId is configured by NR SN.

By using the new list of AffectedCarrierFreqCombListNR to report the affected band combination, both NR CA combination and the En-DC band combination can be reported.

For the victim system affected by the EN-DC band combination, we consider that the victimSystemType used in LTE can be also used for the EN-DC band combination.

Proposal 10: The LTE victimSystemType is reused for the AffectedCarrierFreqCombListNR.

## Coordination between MN and SN

As the affected NR frequency(s) are reported via the LTE InDeviceCoexIndication message of MCG, to resolve the IDC issue of the NR frequency configured by the SN, the MN needs to forward the report of the affected NR frequency to the SN by using the NR ASN.1 of CG-ConfigInfo message. From our understanding, only the frequency configured by the SN needs to be forwarded.  Regarding the sidelobe interference, the affectedCarrierFreqListNR can be forwarded. Regarding the inter-modulation interference, the AffectedCarrierFreqCombListNR including the NR frequency configured by the SN can be forwarded. The details on which frequency band or band combination can be forwarded can be left to the network implementation.

Proposal 11: The NR CG-ConfigInfo message includes the following IDC assistance information:

- affectedCarrierFreqListNR

- AffectedCarrierFreqCombListNR

- victimSystemType

As the AffectedCarrierFreqCombListNR reported by the UE includes the measurement object configured by both LTE MN and NR SN. From our understanding, only the measurement object configured by the NR SN needs to be forwarded, as the NR SN cannot control the LTE MCG configuration of the measurement object.

Proposal 12: The AffectedCarrierFreqCombListNR included in the NR CG-ConfigInfo message only includes the NR measurement object configured by the NR SN.

Regarding the assistance information from the NR SN to the LTE MN, we consider that the current SCG information included in CG-Config [8] can be reused in Rel-15. More assistance information from SN to MN can be discussed in the future release.

# Conclusion

According to the analysis given above, we have the following Observations and Proposals:

Observation 1: One MeasObjectNR configured by the LTE MN can only indicate one frequency.

Observation 2: One MeasObjectNR configured by the NR SN can indicate two frequencies.

Proposal 1: The IDC solution for the affected NR frequency is only for the EN-DC in Rel-15.

Proposal 2: Only the IDC reporting of the affected NR frequency is specified in Rel-15.

Proposal 3: The IDC reporting of the affected NR frequency is via the LTE InDeviceCoexIndication message.

Proposal 4: The MeasObjectId of current affectedCarrierFreqList is reused to report the affected NR frequency configured by the LTE MN.

Proposal 5: The interferenceDirection of affectedCarrierFreqList adds “nr” as a new RAT type.

Proposal 6: The affected NR frequency configured by the NR SN is reported in a new list of affectedCarrierFreqListNR.

Proposal 7: Each reported NR frequency in affectedCarrierFreqListNR includes the following information:

- measObjectId-NR: The measurement object ID configured by the NR SN.

- rsType: Indicate if the affected frequency of the measurement object is SSB frequency or CSI-RS frequency.

- interferenceDirection: Indicate if the NR or other RAT or both are victim RAT for the NR frequency indicated in affectedCarrierFreqListNR.

Proposal 8: The affected EN-DC band combination is reported in a new list of AffectedCarrierFreqCombListNR.

Proposal 9: Each frequency information of an affected EN-DC band combination in AffectedCarrierFreqCombListNR includes:

- measObjectId: The measurement object ID configured by either LTE MN or NR SN.

- measObjectType: Indicate if the reported measObjectId is configured by LTE MN or NR SN.

- rsType: Indicate if the affected frequency of the measurement object is SSB frequency or CSI-RS frequency when the measObjectId is configured by NR SN.

Proposal 10: The LTE victimSystemType is reused for the AffectedCarrierFreqCombListNR.

Proposal 11: The NR CG-ConfigInfo message includes the following IDC assistance information:

- affectedCarrierFreqListNR

- AffectedCarrierFreqCombListNR

- victimSystemType

Proposal 12: The AffectedCarrierFreqCombListNR included in the NR CG-ConfigInfo message only includes the NR measurement object configured by the NR SN.

# Reference

- 3GPP TS 38.101-1, “User Equipment (UE) radio transmission and reception; Part 1: Range 1 Standalone”.

- 3GPP TS 38.101-2, “User Equipment (UE) radio transmission and reception; Part 2: Range 2 Standalone”.

- R4-1803254, LSout, From RAN4, To RAN2, “LS reply on In-Device Coexistence solution for EN-DC”.

- 3GPP TS 36.331, “Radio Resource Control (RRC); Protocol specification”

- 3GPP TS 36.300, “Evolved Universal Terrestrial Radio Access (E-UTRA) and Evolved Universal Terrestrial Radio Access Network (E-UTRAN); Overall description; Stage 2”

- R2-1802609, IDC indication for EN-DC, Qualcomm Incorporated	discussion

- R2-1802101, Discussion on the IDC report of NR frequency in EN-DC, vivo	discussion

- 3GPP TS 38.331, “NR; Radio Resource Control (RRC); Protocol specification”

# Annex A

The following table gives the potential victim frequency range between NR band(s) and other systems:

| Victim Systems | Frequency range [MHz] |  |  | Regions | Side-lobe | NR second harmonic order | NR third harmonic order |
| --- | --- | --- | --- | --- | --- | --- | --- |
| COMPASS (Beidou) | 1559 | - | 1591 |  | n50(UL)/n74(UL) |  |  |
| Galileo | 1559 | - | 1591 |  | n50(UL)/n74(UL) |  |  |
| GLONASS | 1591 | - | 1610 |  | n50(UL)/n70(UL) |  |  |
| GPS | 1563 | - | 1587 |  | n50(UL)/n74(UL) |  |  |
| ISM band (2.4GHz) | 2400 | - | 2483.5 | US/Europe | n7/n38/n41 |  | n5/n20/n28/n82 |
|  | 2400 | - | 2494 | Asia | n7/n38/n41 |  | n5/n20/n28/n82 |
| ISM band (5GHz) | 5150 | - | 5925 | US | n79 | n7/n38/n41 | n1/n2/n3/n66/n80/n84 |
|  | 5150 | - | 5350 | Europe | n79 | n7/n38/n41 | n1/n2/n3/n66/n80/n84 |
|  | 5470 | - | 5725 |  |  |  |  |
|  | 5150 | - | 5825 | Asia | n79 | n7/n38/n41 | n1/n2/n3/n66/n80/n84 |
| ISM band (45GHz) | 42300 | - | 47000 | China |  | n258 |  |
|  | 47200 | - | 48400 |  |  |  |  |
| ISM band (60GHz) | 57000 | - | 66000 | Europe |  | n257 |  |
|  | 57050 | - | 64000 | USA - Canada |  | n257 |  |
|  | 57000 | - | 64000 | South Korea |  | n257 |  |
|  | 59000 | - | 66000 | Japan |  | n257 |  |
|  | 59000 | - | 64000 | China |  | n257 |  |
|  | 59400 | - | 62900 | Australia |  | n257 |  |

# Annex B

The following table for FR1 is extracted from 38.101-1 [1]:

Table 5.2-1: NR operating bands in FR1

| NR Operating Band | Uplink (UL) operating band BS receive UE transmit |  |  | Downlink (DL) operating band BS transmit UE receive |  |  | Duplex Mode |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| n1 | 1920 MHz | – | 1980 MHz | 2110 MHz | – | 2170 MHz | FDD |
| n2 | 1850 MHz | – | 1910 MHz | 1930 MHz | – | 1990 MHz | FDD |
| n3 | 1710 MHz | – | 1785 MHz | 1805 MHz | – | 1880 MHz | FDD |
| n5 | 824 MHz | – | 849 MHz | 869 MHz | – | 894MHz | FDD |
| n7 | 2500 MHz | – | 2570 MHz | 2620 MHz | – | 2690 MHz | FDD |
| n8 | 880 MHz | – | 915 MHz | 925 MHz | – | 960 MHz | FDD |
| n20 | 832 MHz | – | 862 MHz | 791 MHz | – | 821 MHz | FDD |
| n28 | 703 MHz | – | 748 MHz | 758 MHz | – | 803 MHz | FDD |
| n38 | 2570 MHz | – | 2620 MHz | 2570 MHz | – | 2620 MHz | TDD |
| n41 | 2496 MHz | – | 2690 MHz | 2496 MHz | – | 2690 MHz | TDD |
| n50 | 1432 MHz | – | 1517 MHz | 1432 MHz | – | 1517 MHz | TDD |
| n51 | 1427 MHz | – | 1432 MHz | 1427 MHz | – | 1432 MHz | TDD |
| n66 | 1710 MHz | – | 1780 MHz | 2110 MHz | – | 2200 MHz | FDD |
| n70 | 1695 MHz | – | 1710 MHz | 1995 MHz | – | 2020 MHz | FDD |
| n71 | 663 MHz | – | 698 MHz | 617 MHz | – | 652 MHz | FDD |
| n74 | 1427 MHz | – | 1470 MHz | 1475 MHz | – | 1518 MHz | FDD |
| n75 | N/A |  |  | 1432 MHz | – | 1517 MHz | SDL |
| n76 | N/A |  |  | 1427 MHz | – | 1432 MHz | SDL |
| n78 | 3300 MHz | – | 3800 MHz | 3300 MHz | – | 3800 MHz | TDD |
| n77 | 3300 MHz | – | 4200 MHz | 3300 MHz | – | 4200 MHz | TDD |
| n79 | 4400 MHz | – | 5000 MHz | 4400 MHz | – | 5000 MHz | TDD |
| n80 | 1710 MHz | – | 1785 MHz | N/A |  |  | SUL |
| n81 | 880 MHz | – | 915 MHz | N/A |  |  | SUL |
| n82 | 832 MHz | – | 862 MHz | N/A |  |  | SUL |
| n83 | 703 MHz | – | 748 MHz | N/A |  |  | SUL |
| n84 | 1920 MHz | – | 1980 MHz | N/A |  |  | SUL |

# Annex C

The following table for FR2 is extracted from 38.101-2 [2]:

Table 5.2-1: NR operating bands in FR2

| NR Operating Band |  | Uplink (UL) operating band BS receive UE transmit |  |  | Downlink (DL) operating band BS transmit UE receive |  |  | Duplex Mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| n257 | 26500 MHz |  | – | 29500 MHz | 26500 MHz | – | 29500 MHz | TDD |
| n258 | 24250 MHz |  | – | 27500 MHz | 24250 MHz | – | 27500 MHz | TDD |
| n260 | 37000 MHz |  | – | 40000 MHz | 37000 MHz | – | 40000 MHz | TDD |