3GPP TSG-RAN WG2 Meeting #101bis								R2-1804654

Sanya, CHINA, 16 - 20 Apr 2018                                          			revision on R2-1802657

Agenda item:	9.9.2

Source: 	Huawei, HiSilicon

Title: 	Remaining issues of temporary CQI reporting

Document for:	Discussion and decision

1	Introduction

In RAN2#100 meeting, some agreements related to enhancing the existing SCell activation were reached [1].

Agreements:

1	The short period of the CQI reporting resource could be only available upon receiving the SCell activation command. FFS when the UE fall back to longer periodicity.

If a UE receives the SCell activation command it can enable a short period of CQI reporting configuration, which can shorten the scheduling delay, but when the UE falls back to normal periodicity is still a FFS. In this document, several candidate solutions are proposed, and the analysis and comparison of the alternatives of timing are also provided. The proposal had been changed, compared to previous document.

2	Discussion

2.1	Timing Issue

When UE receives the SCell activation command it starts to perform CQI reporting in temporary short period. It can report downlink channel quality fast to enable a much quicker scheduling. After initial scheduling is completed UE can fall back to normal and longer periodicity for saving energy and system resource. Although the timing issue has been discussed in the last meeting there is still a FFS left for further clarification.

Figure 1 illustration of falling back timing

2.2	Candidate Solutions

To solve the timing issue, several solutions can be considered and proposed below.

Option1: temporary reporting will be disabled upon the first downlink scheduling.

If fast channel feedback has been received by eNB it may start to schedule the UE as soon as possible, and once the first downlink scheduling is completed it can fall back to longer CQI reporting configuration, especially when the radio channel doesn’t vary often. However, it may introduce complexity of the software implementation.

Figure 1 switch point by first downlink scheduling

Option2: a timer can be set to maintain a short period of temporary CQI configuration.

When radio channel fluctuates dramaticly and the quantity of user data is large enough to be transmitted completely for several times, it may benefit form short period of CQI reporting. So a timer can be set in UE side to maintain the fast CQI reporting. When activation command is received the timer starts, and when the timer expires it falls back to longer period CQI reporting. Optionally, if first downlink scheduling indication is received the timer should be stopped.

Figure 2 switch point by timer

Option3: based on an explicit indication, e.g. MAC CE. An new signalling can be considered to indicate the switch point between short and longer period. It depends on eNB to decide when to fall back to normal periodicity considering the quantity of offload data.

Figure 3 switch point by dedicated signalling

Both the advantages and disadvantages of the options above are listed in table 1. Option 2 and option 3 can achieve more flexibility of network via the more flexible control on the available time of frequent CQI reporting, and at the same time it will consume more uplink resource or signalling overhead. For option 1 there is no signalling overhead, without resource waste, and the fast CQI reporting gain can only be derived for the first downlink scheduling.

Table 1 Solutions Comparison

| Options | UL Resource occupation | Signalling overhead | Network Flexibility |
| --- | --- | --- | --- |
| Option 1: switch point by first downlink scheduling. | Little | None | Low, and with introducing complexity of the software implementation |
| Option 2: switch point by timer | Flexible | None | Medium |
| Option 3: switch point by dedicated signalling | Flexible | Medium | High |

Hence, we propose:

Proposal: a timer can be set to maintain the short period of CQI reporting as a complement.

3	Conclusions

In this document, we provide several solutions to solve the timing issue of when the UE falls back to normal CQI reporing periodicity, and the comparison between them are mead for a clear understanding. Based on these ananlysis, we have the following proposal:

Proposal: a timer can be introduced to ensure the timely release of the CQI reporting resource of short period as a complement.

4	References

- Chairman notes, 3GPP TSG-RAN WG2 Meeting #100, Reno, Nevada, 27 November- 1 December 2017