3GPP TSG-RAN WG2 Meeting #101bis                                                               R2-1804680

Sanya, China, 16th April- 20th April 2018                                     (Revision of R2-1802002)

Source:	vivo

Title:	MAC CE for the new SCell state

Agenda Item:	9.9.2

Document for:	Discussion and Decision

- Introduction

In RAN2#100 meeting, RAN2 has agreed  to introduce   new fast SCell activation state (new state) in order to reduce latency for SCell activation from legacy SCell deactivated state to legacy SCell activated state. And in RAN2#101 meeting, some agreements on the name of the new state and the state transition were achieved:

Agreements:

1	Use “Dormant SCell state” as the name of the new Scell state and use ”Hibernate” as the action for moving from Activated SCell state to Dormant Scell state if necessary.

2	Dormant Scell follows PCell DRX for CQI/RRM measurement report triggering.

3	Additional new MAC CE is needed for state transition in/out of the dormant state. Legacy MAC CE is still used as in Rel-10 CA.

4	Define a MAC CE for state transition from activated SCell state to the dormant SCell state.

5    It is supported that UE transits from dormant state to deactivate state.

In this contribution, we discuss the MAC CE for the new SCell state based on the above agreements in more details.

- Discussion

In the legacy case, there are two states for the SCell, activated and deactivated. And the network activates and deactivates the SCell(s) by sending the Activation/Deactivation MAC control element. The Activation/Deactivation MAC control element of one/four octet is identified by a MAC PDU subheader with LCID. It has a fixed size and consists of a single/four octet containing seven/31 C-fields and one R-field.

Since the new state is introduced, there are three SCell states to be configured or transitted between any two of the states.

The legacy Activation/Deactivation MAC control element cannot work to support the three states. If a new MAC CE is used only for state transition in/out of the dormant state, while three SCells will transit to  dormant state and four SCell will transit from dormant state, a legacy MAC CE is needed to indicate which state(activated or deactivated) the SCell will be after transiting from the dormant state. And when the legacy MAC CE is applied, which value should be set for the three dormant SCell? Even though some solutions can indicate the state transition with the legacy MAC CE and the new MAC CE, a single octet containing seven C-filds to indicate three states is difficult.

Observation 1: It is a complex mechanism to use a new MAC CE only for the state transition in/out of the dormant state with the consideration of three possible SCell states.

On the other side, it is straightforward to indicate the three possible SCell states with two bits for one SCell comparing with the legacy one bit for two states. Therefore, A new MAC control element is needed to indicate the three possible states of the SCell(s) by the network.

Proposal 1: A new MAC control element is needed to indicate the three possible states of the SCell(s) by the network.

The new MAC CE is defined as Figure 1 and Figure 2.

Figure 1 the state MAC CE of two octets

Figure 2 the state MAC CE of eight octets

It has a fixed size and consists of  two/eight octets containing seven/31 C-fields and two R-fields. An example for Ci: the bit indexes are set to 11 to indicate that the SCell with SCellIndex i shall be activated. The bit indexes are set to 01 to indicate that the SCell with SCellIndex i shall be the new state. The bit indexes are set to 00 to indicate that the SCell with SCellIndex i shall be deactivated. The bit indexes 10 are reserved. Also, two new LCIDs are needed to identify the new MAC control element of two octets and eight octets.

Proposal 2: The state MAC control element of two octets is identified by a MAC PDU subheader with LCID. It has a fixed size and consists of two octets containing seven C-fields and two R-fields. Each C-field consists of two bits.

Proposal 3: The state MAC control element of eight octets is identified by a MAC PDU subheader with LCID. It has a fixed size and consists of eight octets containing 31 C-fields and two R-fields. Each C-field consists of two bits.

- Conclusion

In this contribution, we discuss the MAC CE for the new SCell state with the following observation and proposals:

Observation 1: It is a complex mechanism to use a new MAC CE only for the state transition in/out of the dormant state with the consideration of three possible SCell states.

Proposal 1: A new MAC control element is needed to indicate the three possible states of the SCell(s) by the network.

Proposal 2: The state MAC control element of two octets is identified by a MAC PDU subheader with LCID. It has a fixed size and consists of two octets containing seven C-fields and two R-fields. Each C-field consists of two bits.

Proposal 3: The state MAC control element of eight octets is identified by a MAC PDU subheader with LCID. It has a fixed size and consists of eight octets containing 31 C-fields and two R-fields. Each C-field consists of two bits.

References

- RAN2 #100 Chairman’s Notes.

- RAN2 #101 Chairman’s Notes.