3GPP TSG-RAN WG2 Meeting #101	R2-1804770

Sanya, China, 16th - 20th April 2018	R2-1802756

Agenda item:	9.9.2

Source:	Nokia, Nokia Shanghai Bell

Title:	Remaining details on “CQI burst” after SCell activation

WID/SID:	LTE_euCA-Core - Release 15

Document for:	Discussion and Decision

# 1	Introduction

The topic of allowing temporary CQI reporting after SCell activation was discussed, with the following agreed in RAN2#100:

Agreements:

1	The short period of the CQI reporting resource could be only available upon receiving the SCell activation command. FFS when the UE fall back to longer periodicity.

In this contribution, we discuss what is still needed to make the agreement work in specifications.

# 2	SCell “burst” CQI reporting after SCell activation

## 2.1	Terminology of “SCell CQI burst” reporting after SCell activation

The so-called temporary CQI reporting has not been denoted with a clear name so far: Originally it was called “short CQI reporting”, with “temporary CQI reporting” being used most recently. However, RAN2 hasn’t agreed to any official term to this. Considering that the feature can be characterized with a “burst” of CQI reporting, we would propose to use “SCell CQI burst” as the term for this feature.

Proposal 0: Use the term “SCell CQI burst” to denote the feature where SCell CQI reporting periodicity temporarily changes after Scell activation.

## 2.2	Description of Scell CQI burst

As defined in R2-1713336, the SCell CQI burst consists of the following:

- Network configures SCell to use SCell CQI burst reporting configuration.

- Once UE receives MAC CE to activate the SCell, its CQI reporting configuration uses the CQI burst reporting configuration.

- The CQI burst reporting configuration has a set fixed duration, after which UE reverts to using the CQI configuration in legacy SCell configuration.

To ensure common understanding, we propose that RAN2 confirms the above as the description of how the agreement is intended to operate.

Proposal 1: RAN2 to confirm the description of SCell CQI burst as shown here:

- Network configures SCell to use SCell CQI burst reporting configuration.

- Once UE receives MAC CE to activate the SCell, its CQI reporting configuration uses the CQI burst reporting configuration.

- The CQI burst reporting configuration has a set fixed duration, after which UE reverts to using the CQI configuration in legacy SCell configuration.

We note that in addition to the FFS in the agreements, the following open questions can be immediately noted from the description:

- What kind of CQI periodicities are supported?

- Is only periodic CQI reporting supported, or also aperiodic?

- Are all CQI reporting enhancements introduced after Rel-8 (e.g. FD-MIMO) usable?

- Is the configuration one-shot, i.e. used only once and then discarded, or does UE store the configuration and is always use it when SCell is activated?

- How long should the CQI burst be possible?

- Can the CQI burst work together with direct SCell activation?

We discuss these questions in the following sections.

## 2.3	Type of reporting and reporting periodicity

RAN1 specification currently allows CQI reporting periodicities as defined in TS36.213, Table 7.2.2-1A-L (see Annex A). In nutshell, RAN1 allows for 1 or 2 ms reporting periodicity a minimum (depending on whether TDD or FDD is used). To avoid unnecessary RAN1 changes, it seems most reasonable that only the existing reporting periodicities would be used.

Proposal 2: Use existing CQI reporting periodicities defined in TS36.213 for SCell CQI burst.

The question of which reporting type should be used is also quite simple: To get the benefits from the scheme, eNB should receive the reports as soon as possible but also be aware of when the reports could arrive. Therefore, it seems that only periodic CQI reporting would need to be supported.

Proposal 3: Only periodic CQI reporting is supported for the SCell CQI burst.

Finally, the question of which types of configurations should be possible needs to be resolved: The easiest could be to just use what is allowed for legacy and be done with it, as that would likely require only minimal or no modifications to RAN1 specifications. We would propose to do that and indicate to RAN1 in an LS that they should consider if something else is required in their specifications.

Proposal 4: Send LS to RAN1 to request them to allow SCell CQI burst with existing CQI reporting periodicities.

## 2.4	Reporting duration

The duration of the CQI burst was also left FFS in RAN2#100. The duration is not so big a problem as such, as any value would do, but both UE and eNB should have the same understanding of it. The alternatives are:

- Continue until SCell is assumed to be activated, i.e. n+24/n+34

- Continue for fixed time, e.g. n+34

- Allow eNB to configure how long the reporting continues

All of the options have some benefits: 1) and 2) would require no new signalling, whereas 3) would give eNB most flexibility. But 1) is not so clear to eNB as to how long to expect (as eNB doesn’t always know if the cell was known or unknown), 2) might continue longer than necessary (causing extra overhead and UL activity), and 3) requires some signalling. However, considering the pros and cons, we think 2) seems the most reasonable as it follows the spirit of the original agreement while still being very clear (we assume the value would be fixed in specifications like n+8 is).

Proposal 5: UE continues to use the CQI burst configuration until n+34.

We also think that the configuration would NOT be a one-shot configuration, but something that eNB can continue to choose to use for every SCell activation (i.e. based on SCell configuration). However, we also consider that the configuration might not be used for all SCells.

Proposal 6: When configured for an SCell, the CQI burst configuration is always used when the SCell is activated.

## 2.5	CQI burst with direct SCell activation

Since the direct Scell activation is also introduced at configuration, it is obvious to ask whether the CQI burst could be used also at that time to allow UEs to indicate as fast as possible when they have a valid CQI. Considering that the direct SCell activation just changes when SCell is considered as activated and the PUCCH format change time, there doesn’t seem to be any overlap that could cause incompatibility with the features. Therefore, we would propose to allow using both features together.

Proposal 7: SCell CQI Burst can be used together with direct SCell activation.

## 2.6	UE capabilities

Since there are only two meetings for the WID left, UE capabilities should be discussed for each agreed feature. Considering the CQI burst seems like a stand-alone feature with rather simple characteristics, requiring IOT testing, it seems a single UE capability bit could be introduced. The bit could cover the following:

- Acting according to the RRC configuration for CQI burst: Support of signalling and procedures for UE to send CQI according to the CQI burst configuration

- RAN4 requirements for the CQI: Should follow existing CQI reporting accuracy when valid CQI is available (like with legacy), so no new UE requirements are needed. However, this should be verified by RAN4.

Proposal 8: Indicate the decisions on CQI burst to RAN1/4 in LS.

# 3	Conclusions

We have discussed the remaining details of SCell CQI burst, and proposed the following:

Proposal 0: Use the term “SCell CQI burst” to denote the feature where SCell CQI reporting periodicity temporarily changes after Scell activation.

Proposal 1: RAN2 to confirm the description of SCell CQI burst as shown here:

- Network configures SCell to use SCell CQI burst reporting configuration.

- Once UE receives MAC CE to activate the SCell, its CQI reporting configuration uses the CQI burst reporting configuration.

- The Scell CQI burst reporting configuration has a set fixed duration, after which UE reverts to using the CQI configuration in legacy SCell configuration.

Proposal 2: Use existing CQI reporting periodicities defined in TS36.213 for SCell CQI burst.

Proposal 3: Only periodic CQI reporting is supported for the SCell CQI burst.

Proposal 4: Send LS to RAN1 to request them to allow SCell CQI burst with existing CQI reporting periodicities.

Proposal 5: UE continues to use the CQI burst configuration until n+34.

Proposal 6: When configured for an SCell, the CQI burst configuration is always used when the SCell is activated.

Proposal 7: Scell CQI Burst can be used together with direct SCell activation.

Proposal 8: Indicate the decisions on CQI burst to RAN1/4 in LS.

# Annex A: CQI reporting periodicities in TS36.213

Table 7.2.2-1A: Mapping of  to  and  for FDD or for FDD-TDD and primary cell frame structure type 1

|  | Value of | Value of |
| --- | --- | --- |
| 0 1 | 2 |  |
| 2 6 | 5 | – 2 |
| 7 16 | 10 | – 7 |
| 17 36 | 20 | – 17 |
| 37 76 | 40 | – 37 |
| 77 156 | 80 | – 77 |
| 157 316 | 160 | – 157 |
| = 317 | Reserved |  |
| 318 349 | 32 | – 318 |
| 350 413 | 64 | – 350 |
| 414 541 | 128 | – 414 |
| 542 601 | 60 | – 542 |
| 602 1023 | Reserved |  |

Table 7.2.2-1B: Mapping of  to  and .

|  | Value of | Value of |
| --- | --- | --- |
| 0 160 | 1 | − |
| 161 321 | 2 | − ( – 161) |
| 322 482 | 4 | − ( – 322) |
| 483 643 | 8 | − ( – 483) |
| 644 804 | 16 | − ( – 644) |
| 805 965 | 32 | − ( – 805) |
| 966 1023 | Reserved |  |

Table 7.2.2-1C: Mapping of  to  and  for TDD or for FDD-TDD and primary cell frame structure type 2

|  | Value of | Value of |
| --- | --- | --- |
| = 0 | 1 |  |
| 1 5 | 5 | – 1 |
| 6 15 | 10 | – 6 |
| 16 35 | 20 | – 16 |
| 36 75 | 40 | – 36 |
| 76 155 | 80 | – 76 |
| 156 315 | 160 | – 156 |
| 316 375 | 60 | – 316 |
| 376 1023 | Reserved |  |

Table 7.2.2-1J: Mapping of ICRIto MCRI when RI reporting is configured

|  | Value of MCRI |
| --- | --- |
| 0 | 1 |
| 1 | 2 |
| 2 | 4 |
| 3 | 8 |
| 4 | 16 |
| 5 | 32 |
| 6 | 64 |
| 7 | 128 |
| 7<1023 | Reserved |

Table 7.2.2-1K: Mapping of MCRI to MCRIand NOFFSET,CRI when the number of antenna ports in each configured CSI-RS resource is one

|  | Value of MCRI | Value of |
| --- | --- | --- |
| 0 ICRI 160 | 1 | −ICRI |
| 161 ICRI 321 | 2 | − (ICRI – 161) |
| 322 ICRI 482 | 4 | − (ICRI– 322) |
| 483 ICRI 643 | 8 | − (ICRI – 483) |
| 644 ICRI 804 | 16 | − (ICRI – 644) |
| 805 ICRI 965 | 32 | − (ICRI– 805) |
| 966 ICRI 1023 | Reserved |  |

Table 7.2.2-1L: Mapping of MCRI to MCRIand NOFFSET,CRI

|  | Value of MCRI | Value of |
| --- | --- | --- |
| 0 ICRI 160 | 1 | −ICRI |
| 161 ICRI 321 | 2 | − (ICRI – 161) |
| 322 ICRI 482 | 4 | − (ICRI– 322) |
| 483 ICRI 643 | 8 | − (ICRI – 483) |
| 644 ICRI 804 | 16 | − (ICRI – 644) |
| 805 ICRI 965 | 32 | − (ICRI– 805) |
| 966 ICRI 1023 | Reserved |  |