3GPP TSG-RAN WG2 #101bis	R2-1804802

Sanya, P.R. of China, 16th – 20th April 2018	(Revision of R2-1802595)

Agenda Item:	10.2.5

Source:	Ericsson

Title:	UE context handling during inter RAT handover

Document for:	Discussion

# Introduction

It has been agreed to support fast mobility between LTE and NR. Furthermore, it is a working assumption in RAN3 that LTE should connect to the NextGen CN using the same NG interface as NR.

This contribution is addressing UE context handling at inter-RAT mobility between LTE and NR when the UE is attached to NextGen CN. A text proposal is also provided to TS 38.300.

# Architecture Overview

Below shows an overview architecture when both LTE eNBs and NR gNBs are connected to the Next Gen CN.

Inter-RAT mobility would occur when a UE that is connected to one of the RATs later moves over to the other RAT. The move could be as part of an inter-RAT hard handover, or as MeNB handover for UEs in DC, or it could occur as part of context fetch procedure e.g. for UEs returning from inactive state. During the mobility the NG termination point will be moved from the source node to the target node.

# Context to be transferred

Given that the lower layers of the radio stack most likely are a bit different between LTE and NR, it is expected that the context transfer would be limited to higher layer parameters which are common for both RATs (e.g. related to the NG termination in RAN and not related to detailed AS configuration).

Below is a list of context to be considered:

- UE radio access and security capabilities

  - Most likely the UE radio access capabilities will be separate sets of LTE and NR capabilities

  - Currently UE RAC is transferred as part of RRC container

- UE security context (could include KeNB* and NCC)

- UE QoS related parameters (including active Flow ID and mapping between Flow ID and RBs)

- UE NG CN identifier used over NG interface (e.g. for a subsequent “Path Switch” message to Next Gen CN)

- Other UE related or subscription related info to be decided (UE history, handover restrictions, tracing info, masked IMEISV, etc.)

- Packet forwarding related info (e.g. endpoints, UL/DL PDCP sequence number status)

- UE bearer configuration (SDAP, PDCP layer configuration)

Proposal 1:		It is proposed to capture the parameters below as an initial list of identified parameters to be transferred between the source and target RAN node for inter-RAT handover between LTE and NR (in either direction) connected to Next Gen CN.

- UE radio access and security capabilities

- UE security context (could include KeNB* and NCC)

- UE QoS related parameters (including active Flow ID and mapping between Flow ID and RBs)

- UE NG CN identifier used over NG interface (e.g. for a subsequent “Path Switch” message to Next Gen CN)

- Other UE related or subscription related info to be decided (UE history, handover restrictions, tracing info, masked IMEISV, etc.)

- Packet forwarding related info (e.g. endpoints, UL/DL PDCP sequence number status)

- UE bearer configuration (SDAP, PDCP layer configuration)

In addition to the parameters above, it is also expected for inter-RAT handover that the source node includes an indication of the “target cell” to the target node. In this scenario, it is further expected that the target RAN node generates an RRC handover command which is sent in a container to the UE via the source RAN node.

Proposal 2: 	For inter-RAT handover, the source RAN node will indicate to the target RAN node which is the “target cell”.

Proposal 3: 	For inter-RAT handover, the target RAN node will generate a Full RRC handover command message which is sent to the UE via the source RAN node.

Proposal 4:		It is proposed to capture this in TS 38.300.

# Conclusion

It is proposed to agree to the proposals below and capture the principles in the TR.

Proposal 1:		It is proposed to capture the parameters below as an initial list of identified parameters to be transferred between the source and target RAN node for inter-RAT handover between LTE and NR (in either direction) connected to Next Gen CN.

- UE radio access and security capabilities

- UE security context (could include KeNB* and NCC)

- UE QoS related parameters (including active Flow ID and mapping between Flow ID and RBs)

- UE NG CN identifier used over NG interface (e.g. for a subsequent “Path Switch” message to Next Gen CN)

- Other UE related or subscription related info to be decided (UE history, handover restrictions, tracing info, masked IMEISV, etc.)

- Packet forwarding related info (e.g. endpoints, UL/DL PDCP sequence number status)

- UE bearer configuration (SDAP, PDCP layer configuration)

Proposal 2: 	For inter-RAT handover the source RAN node will indicate to the target RAN node which is the “target cell”.

Proposal 3: 	For inter-RAT handover the target RAN node will generate a Full RRC handover command message which is sent to the UE via the source RAN node.

Proposal 4:		It is proposed to capture this in TS 38.300.

Below is a text proposal to 38.300..

# Text proposal to TS 38.300

## 9.3	Inter RAT

### 9.3.1	Intra 5GC

#### 9.3.1.1	Cell Reselection

Cell reselection is characterised by the following:

-	Cell reselection between NR RRC_IDLE and E-UTRA RRC_IDLE is supported;

#### 9.3.1.2	Handover

Inter RAT mobility is characterised by the following:

-	Source RAT should be able to support and configure Target RAT measurement and reporting.

-	The in-sequence and lossless handover is supported for the handover between gNB and ng-eNB.

-	Both Xn and NG based inter-RAT handover between NG-RAN nodes is supported. Whether the handover is over Xn or CN is transparent to the UE.

-	The target RAT receives the UE NG-C context information and based on this information configures the UE with a complete RRC message and Full configuration (not delta).

-	The source NG-RAN node provides the following parameters to the target NG-RAN node:

-	UE radio access and security capabilities

-	UE security context (could include KeNB* and NCC)

-	UE QoS related parameters (including active Flow ID and mapping between Flow ID and RBs)

-	UE NG CN identifier used over NG interface (e.g. for a subsequent “Path Switch” message to Next Gen CN)

-	Other UE related or subscription related info to be decided (UE history, handover restrictions, tracing info, masked IMEISV, etc.)

-	Packet forwarding related info (e.g. endpoints, UL/DL PDCP sequence number status)

-	Target cell ID

-	UE bearer configuration (SDAP, PDCP layer configuration)