3GPP TSG-RAN WG2 #101Bis	Tdoc R2-1804855

Sanya, P.R. of China, 16th – 20th April 2018

Agenda Item:	9.7.2

Source:	Ericsson

Title:	Principles of E-UTRA Handover involving CN change

Document: 	Discussion, Decision

# Introduction

In the work on 5GS, it has been agreed that NG-RAN will consist of both NR gNBs and E-UTRA ng-eNBs connected to 5GC. As these systems will gradually be deployed, there will be cases where a UE need to move between a legacy E-UTRA cell connected to EPC and a NG-RAN E-UTRA cell connected to 5GC. This contribution discusses the main principles for the RRC signaling required to enable this inter-system intra-RAT handover in E-UTRA between EPC and 5GC.

# Discussion

In LTE, when a UE should perform a handover from one eNB to another, the target eNB prepares a RRCConnectionReconfiguration message which it sends to the source eNB via either S1 or X2. As the source and target cell use the same RAT, it is possible to perform a delta configuration of the UE to only signal the parameters which need to be updated. Similarly, for the security context, the UE can perform a horizontal or vertical key derivation based in the received NextHopChainingCount. On the contrary, for inter-RAT handover from UTRAN/GERAN to E-UTRAN, the UE needs to map the source context to an E-UTRAN context.

The principle for inter-RAT handover is to minimize the impact on the legacy system, while still enable a loss-less handover. This is achieved by the target RAT generating the configurations and transparently transport it through the source RAT to the UE.

Legacy inter-RAT handover is achieved by transparently transporting the target reconfigurations through the source RAT with minimal modification to legacy RAT.

For the handover case of LTE inter-RAT inter-system, it was agreed in RAN2#99bis that [1]:

Agreements:

1	RAN2 understand that the source eNB/ng-eNB decides handover procedure to trigger (e.g. via the same CN type or to the other CN type)

2	UE has to know the target CN type from the handover command during intra-LTE inter-system HO, intra-LTE intra-system HO

FFS: Stage 3 detail whether this is an explicit indication or can be inferred from other information.

During RAN2#101 it was agreed that [2]:

4	Security procedures for inter-system handover to E-UTRA/EPC should follow legacy inter-RAT handover procedures i.e. be configured with 36.331 SecurityConfigHO.

Although this agreement was reached during discussion for inter-RAT handover from NR to E-UTRA/EPC, it is also applicable to inter-system handovers from E-UTRAN/5GC to E-UTRA/EPC, since CN change will imply in different security procedures.

For the UE to distinguish between an intra-system intra-LTE handover and an inter-system intra-LTE handover, the UE need to be instructed to change the CN. Although this could be achieved by extending the RRCConnectionReconfiguration with a new IE indicating this, the most straightforward way would be to reuse the MobilityFromEUTRACommand also for the inter-system intra-LTE handover.

Inter-system handover between LTE EPC and LTE 5GC should be treated as an inter-RAT handover by using the MobilityFromEUTRACommand

To achieve this, the only change required would be to add eutra as a target RAT in the MobilityFromEUTRACommand and to allow the LTE RRCConnectionReconfiguration to be tunneled in the targetRAT-messageContainer.

MobilityFromEUTRACommand should be changed to include eutra in the targetRAT parameter and to allow the LTE RRCConnectionReconfiguration in the targetRAT-MessageContainer.

For UE handover from E-UTRA/EPC to E-UTRA/5GC or from E-UTRA/5GC to E-UTRA/EPC, the UE would receive the MobilityFromEUTRACommand containing the RRCConnectionReconfiguration and the target RAT set as eutra. Since the UE would know if the source RAT was E-UTRA/EPC or E-UTRA/5GC, the UE would know that it should treat the message as an inter-system intra-LTE handover and apply the received configurations as a full configuration.

In case the target RAT is E-UTRA/5GC, the NR PDCP, SDAP and security parameters would be tunnelled inside the RRCConnectionReconfiguration and the UE applies the configurations contained therein, since it was agreed during RAN2#101 that the handling of PDCP/SDAP and security parameters in all intra-5GC handover scenarios should be the same [2] . Further discussion on security parameters for 5GS are performed in [3].

If the UE handover from E-UTRA/EPC to E-UTRA/5GC, the security parameters are included in the nr-Config IE instead of the SecurityConfigHO IE.

If the target RAT is E-UTRA/EPC, the full configurations would be contained in the RRCConnectionReconfiguration, but the UE would utilize the inter-system configurations in the securityConfigHO IE (i.e. securityAlgorithmConfig and nas-SecurityParamToEUTRA). In this manner, legacy RRC procedures related to security in inter-RAT mobility could be reused without the need for changes.

If the UE handover from E-UTRA/5GC to E-UTRA/EPC, the security parameters are included in the SecurityConfigHO IE and treated as inter-RAT handover.

# Conclusion

In section 2 we made the following observations:

Observation 1	Legacy inter-RAT handover is achieved by transparently transporting the target reconfigurations through the source RAT with minimal modification to legacy RAT.

Based on the discussion in section 2 we propose the following:

Proposal 1	Inter-system handover between LTE EPC and LTE 5GC should be treated as an inter-RAT handover by using the MobilityFromEUTRACommand

Proposal 2	MobilityFromEUTRACommand should be changed to include eutra in the targetRAT parameter and to allow the LTE RRCConnectionReconfiguration in the targetRAT-MessageContainer.

Proposal 3	If the UE handover from E-UTRA/EPC to E-UTRA/5GC, the security parameters are included in the nr-Config IE instead of the SecurityConfigHO IE.

Proposal 4	If the UE handover from E-UTRA/5GC to E-UTRA/EPC, the security parameters are included in the SecurityConfigHO IE and treated as inter-RAT handover.

# References

RAN 2 chair notes, 3GPP TSG-RAN WG2 #99bis, Prague, Czech Republic, 9th - 13th Oct, 2017

RAN 2 chair notes, 3GPP TSG-RAN WG2 #101, Athens, Greece, 26th Feb – 2nd March, 2018

R2-1804798, Security key update during handover, Ericsson, 3GPP TSG-RAN WG2 #101Bis, Sanya, P.R. of China, 16th – 20th April 2018.