3GPP TSG-RAN WG2 #101-Bis	Tdoc R2-1804861

Sanya, P.R. of China, 16th – 20th April 2018

Title:	[DRAFT] Reply LS on 5G-S-TMSI

Release:	Rel-15

Work Item:	NR_newRAT-Core

Source:	Ericsson (to be RAN2)

To:	CT1

Cc:	SA2, RAN3, CT4

Contact Person:

#### Name:	Wahaj Arshad

E-mail Address:	wahaj.arshad @ericsson.com

1. Overall Description:

This is a reply LS to an LS from SA2 (R2-1804256/S2-182964) including questions on length / code space of 5G-S-TMSI.  SA2 is considering not restricting the 5G-S-TMSI to 40 bits.

Question from SA2:

SA2 would kindly like to ask RAN WGs whether it is possible to extend the encoding space for the 5G-S-TMSI from 40 bits to e.g. 48 bits?

Answer:

RAN2 has analysed impact of an extended 5G-S-TMSI to 48 bits instead of 40 bits.

RAN2 conclude that extending the 5G-S-TMSI to 48 bits is straightforward from the perspective of NR.

For LTE connected to 5GC, it is not possible to fit a longer 5G-S-TMSI into message 3. It is however possible to include the 5G-S-TMSI in message 5 instead of message 3 in connection request procedures and then an extension with 8 bits is feasible.

3. Date of Next RAN2 Meetings:

RAN2#102	21st – 25th May 2018, Busan, South Korea

RAN2-AH-1807	2nd – 6th July 2018, Montreal, Canada