3GPP TSG-RAN WG2 #101bis	Tdoc R2-1804862

Sanya, China, 16 – 20 April 2018			(Revision of R2-1802357)

Agenda Item:	10.2.10

Source:	Ericsson

Title:	Emergency/Voice Fall back from NR to LTE

Document for:	Discussion, Decision

# Introduction

The following agreement were made during RAN2#101 on support of emergency fall back in NR [1]

Agreements

1:	EPS fall back to E-UTRAN for voice by HO and redirect procedure are supported. For the HO case no change to the HO procedure is needed compared to any other handover from NR to E-UTRA/EPC.

An offline discussion was triggered during RAN2#101 to progress the need for a CN indication in the redirection to LTE for emergency purposes but some companies requested more time on the topic. This contribution is a revision of R2-1802357, where we elaborate the topic discussed during offline and as a baseline for further discussion during RAN2 101 bis meeting.

# Discussion

## Implications with release with redirect for emergency purposes

The potential problem with using release with redirect could be lack of information on which CN supports emergency services in case of multi core network support in target cell. If the UE is redirected for emergency purposes to a cell that only supports emergency services on one of the supported core network, the UE might fail to camp on a suitable cell, leading to failure of emergency call setup.

If the UE is redirected for emergency purposes to a cell that only supports emergency services on one of the supported cores network, the UE might miss to camp on a suitable cell, leading to failure of emergency call setup.

A solution to handle this situation is to add target core network in release with redirect message meant for emergency services. As indicated earlier, currently RAN is not aware about emergency support in neighbouring cells so this information should either be exchanged from CN or neighbour cells via X2 interface. The UE would then use the core network information to camp on a suitable core network after it is released for emergency fall back.

Another simpler solution is a RAN based approach where the N2 procedure triggered by AMF towards RAN also includes the core network to connect after release with redirect. The RAN can then include this information in the release with redirect command towards the UE.

To ensure that the UE camp on a suitable core network for setting up emergency services after a release with redirect, the N2 procedure triggered by AMF towards RAN also includes the core network to connect to after release with redirect. The RAN can then include this information in the release with redirect command towards the UE.

# Conclusion

In this contribution, the handling of the new N2 based mobility trigger for emergency fall back from NR to LTE is analyzed. Also, the implications associated with using release with redirect procedure for emergency fall back is also explained along with identifying potential solution to handle this issue.

In section ‎2 we made the following observations:

Observation 1	If the UE is redirected for emergency purposes to a cell that only supports emergency services on one of the supported cores network, the UE might miss to camp on a suitable cell, leading to failure of emergency call setup.

Based on the discussion in section ‎2 we propose the following:

Proposal 1	To ensure that the UE camp on a suitable core network for setting up emergency services after a release with redirect, the N2 procedure triggered by AMF towards RAN also includes the core network to connect to after release with redirect. The RAN can then include this information in the release with redirect command towards the UE.

# References

RAN 2 chair notes, 3GPP TSG-RAN WG2 #101, Athens, Greece, 26th Feb – 2nd March, 2018