3GPP TSG-RAN WG2 Meeting #101bis	R2-1804908

Sanya, China, 16th - 20th April 2018

Agenda item:	10.2.6

Source:	Nokia, Nokia Shanghai Bell

Title:	Security Principles for SA

WID/SID:	NR_newRAT-Core - Release 15

Document for:	Discussion and Decision

# 1	Introduction

SA3 has achieved some significant progress on the security framework for NG-RAN connected to 5GC [33.501]. This contribution outlines the main mechanisms while a companion CR [R2-1804909] suggests reflecting them in the Stage 2 [38.300].

# 2	Security Framework

The security framework builds upon the well-established principles of LTE/EPC and can be summarized as follows:

-	The gNB (AS) keys are cryptographically separated from the 5GC (NAS) keys;

-	Separate AS and NAS level security mode command procedures are used;

-	After a primary AKA, KSEAF is created and KAMF is derived;

-	Based on KAMF, KNASint and KNASenc are derived when running a successful NAS SMC procedure;

-	For AS security between UE and gNB, AMF and the UE derive a KgNB when transiting from IDLE to CONNECTED. KRRCint, KRRCenc, KUPint and KUPenc are then derived based on KgNB after a new KgNB is available;

-	NCC controls whether horizontal (KgNB*) or vertical (NH) key derivation is used during mobility events.

-	The key derivation provides both backward and forward security at NG-RAN level:

-	backward security means that a gNB cannot compute a key previously used in another gNB (property of the key derivation operation);

-	forward security means that a gNB cannot compute a key after n hops (ensured by using vertical key derivation during which the basis – NH – is only known to the UE and to the AMF).

-	When transiting to IDLE, all keys must be deleted in the gNB;

-	PCI and ARFCN-DL are used to derive KgNB;

-	Integrity protection is now also available for the user plane.