3GPP TSG-RAN WG2 #101Bis	R2-1805179

Sanya, P.R. of China, 16th – 20th April 2018	Revision of R2-1803082

Agenda Item:	9.14.6

Source:	Ericsson

Title:	Uplink HARQ-ACK feedback for MTC

Document for:	Discussion, Decision

# Introduction

The Rel-15 WI on “Even further enhanced MTC for LTE” [1] has the following WI objective for machine-type communications for BL/CE UEs:

- Power consumption reduction for physical channels [RAN1 lead, RAN2, RAN4]

  - Study and, if found beneficial for connected mode, specify physical signal/channel/DCI for HARQ-ACK feedback in DL for data transmission in UL.

RAN1#91 made the following agreements:

- For the explicit HARQ-ACK feedback signaling,

  - For CE mode A, unused state(s) in the “resource assignment” field in DCI format 6-0A is used to indicate explicit HARQ-ACK feedback for a single UE.

    - FFS: If other fields are set to predetermined values

  - For CE mode B, unused state(s) in the “MCS” field in DCI format 6-0B is used to indicate explicit HARQ-ACK feedback for a single UE.

    - FFS: If other fields are set to predetermined values

- Working assumption:

  - One unused DCI state in each CE mode is used for indicating:

    - Early termination of MPDCCH monitoring and early termination of any ongoing PUSCH transmission

  - Another unused DCI state in each CE mode is used for indicating:

    - Early termination of any ongoing PUSCH transmission (without early termination of MPDCCH monitoring)

  - It can be left up to RAN2 whether the new feedback signaling should also be used for termination of MPDCCH monitoring not related to UL HARQ (re)transmissions (e.g. MPDCCH monitoring related to DL transmissions).

- An LS was sent to RAN2 in R1-1721254 asking for feedback on the above working assumption.

RAN2#101 LS reply to RAN1

- RAN2 assumes that with early termination of MPDCCH monitoring, RAN1 intends to refer to early termination of MPDCCH monitoring and early termination of any ongoing PUSCH transmission.

- For case 1; RAN2 thinks that UL explicit HARQ-ACK for early termination of MPDCCH monitoring and early termination of any ongoing PUSCH transmission can be used to acknowledge the receipt of the RRCConnectionRelease message. RAN2 assumes no changes in RAN2 specification to capture this agreement.

- For case 2; RAN2 thinks that using an explicit HARQ-ACK feedback to acknowledge all pending UL HARQ processes is not efficient with respect to UE power consumption. RAN2 agreed that this use case is not supported.

# Discussion

RAN1 #91 agreed that explicit HARQ-ACK feedback signal is transmitted on MPDCCH and can be used for early termination of PUSCH transmission, as well as for terminating MPDCCH monitoring.

The uplink HARQ operation for BL/CE UEs is asynchronous [3] and there is no explicit positive HARQ-ACK feedback sent from eNB to let the UE know whether an uplink data transmission on PUSCH was successfully received or not, i.e., the absence of an UL retransmission grant can implicitly indicate successful uplink transmission after expiration of drx-ULRetransmissionTimer.

When the uplink traffic is relatively frequent, the UE receives such an implicit positive HARQ-ACK feedback for the previous uplink data transmission when it is scheduled with a new uplink data transmission using the same PUSCH HARQ process with the new data indicator (NDI) bit toggled.

However, when the uplink traffic is relatively infrequent, the UE may not receive any implicit positive HARQ-ACK feedback for an uplink data transmission since there is no immediate next uplink data transmission with the NDI bit toggled. The UE will wait until the uplink HARQ retransmission timers (UL HARQ RTT timer and drx-ULRetransmissionTimer) expire [3], which could take a long time compared to the time scale of the physical layer procedures.

This is relevant for the case when BL/CE UE is released from RRC connected mode to RRC idle mode. When the UE receives an RRCConnectionRelease message, it sends a corresponding RLC-ACK on PUSCH and waits for ”1.25 seconds from the moment the RRCConnectionRelease message was received or optionally when lower layers indicate that the receipt of the RRCConnectionRelease message has been successfully acknowledged, whichever is earlier” [4]. It may be possible to reduce the waiting time if the lower layers can indicate the receipt of the message earlier through the reception of a positive HARQ-ACK feedback from eNB.

## Early termination of PUSCH transmission

Beside for early termination of MPDCCH monitoring, the introduction of positive HARQ-ACK feedback to the UE during an ongoing PUSCH transmission can enable early termination of PUSCH transmission, which can be used to reduce device power consumption, uplink interference and improve system capacity by allowing eNB to use saved system resources for another UE. It is then possible to optimize the number of repetitions in the TTI bundle for a certain UE to match what is required for successful decoding to avoid wasted resources from retransmissions.

If a low BLER is targeted for a PUSCH transmission, a relatively high number of PUSCH repetitions per HARQ transmission must be used to guarantee a high chance of successful reception. This means that the amount of “excessive” repetition may be relatively large in many of the transmissions, and the potential gain from introducing a possibility to do early termination may be significant.

Since the used number of PUSCH repetitions is estimated based on UE’s received signal in downlink and may contain imprecision in measured results, the actual number of PUSCH repetitions required for successful decoding may be less in some cases than initially configured number of PUSCH repetitions.

For uplink transmission, a BL/CE UE would follow procedure described below: if the BL/CE UE is capable of “HARQ-ACK feedback for early termination of PUSCH”, and the eNB has been able to decode ongoing PUSCH transmission, the eNB can schedule new UL grant triggering UE to terminate ongoing PUSCH transmission and starting a new PUSCH transmission. If there is no more UL data to be scheduled, the eNB should not send a new UL grant but instead use DCI to indicate explicit positive HARQ-ACK feedback which would stop the UE from sending further repetitions and hence save power.

In addition, it is desired that the UE indicates with capability signaling whether it supports the Explicit HARQ-ACK feedback and Early PUSCH termination, so that the network can make an informed decision regarding whether to spend downlink resources on transmitting the feedback signal or not [8].

Explicit HARQ-ACK feedback in connected mode and early PUSCH termination in connected mode is an optional UE capability named as ce-UL-HARQ-ACK-Feedback-r15 [9].

## Uplink HARQ-ACK feedback signaling using MPDCCH

In addition, it has been discussed that an indication of explicit positive HARQ-ACK feedback in unused field in DCI could also be used for termination of MPDCCH monitoring when there is no ongoing PUSCH transmission.

After data on PUSCH is transmitted, UL HARQ RTT timer and drx-InactivityTimer are started for the corresponding UL HARQ process. While UL HARQ RTT timer is running UE is not monitoring for DCI of the corresponding UL HARQ process, however BL/CE UE can monitor MPDCCH for possible DCIs of other UL/DL HARQ processes. Once UL HARQ RTT timer expires, the BL/CE UE starts drx-ULRetransmissionTimer for the corresponding UL HARQ process. While drx-ULRetransmissionTimer timer is running, UE monitors MPDCCH for possible retransmission grants. During this time, if no PUSCH retransmission is granted, drx-ULRetransmissionTimer and drx-InactivityTimer expires, the BL/CE UE stops monitoring MPDCCH and is not reachable until the next valid onDuration period of the configured DRX cycle [3].

However, if the BL/CE UE is capable of ce-UL-HARQ-ACK-Feedback-r15[8] to terminate MPDCCH monitoring during drx-ULRetransmissionTimer, possible reduction in monitoring of MPDCCH seem to rather insignificant when taking whole signaling account. In addition, possible reduction of monitoring MPDCCH using UL HARQ-ACK feedback is dependent on configured DRX timer values and the number of MPDCCH repetitions.

RAN2#101 also discussed scenario where MPDCCH indicates an explicit HARQ-ACK for the last HARQ process and then MAC entity stops drx-InactivityTimer. Our view is that early termination of drx-InactivityTimer would break inacitivity timer’s intended functionality to allow the eNB to schedule the UE further. Therefore, we are proposing to use explicit HARQ-ACK feedback signaling to stop drx-ULRetransmissionTimer for the corresponding HARQ process to reduce UE’s power consumption and keep functionality of the drx-InactivityTimer as it is currently specified.

Field for HARQ-ACK feedback configuration for early termination of MPDCCH monitoring and early termination of PUSCH transmission is named as mpdcch-UL-HARQ-ACK-FeedbackConfig.

Explicit HARQ-ACK feedback signaling is not used to terminate drx-InactivityTimer.

# Conclusion

- Explicit HARQ-ACK feedback in connected mode and early PUSCH termination in connected mode is an optional UE capability.

Field for HARQ-ACK feedback configuration for early termination of MPDCCH monitoring and early termination of PUSCH transmission is named as mpdcch-UL-HARQ-ACK-FeedbackConfig.

Explicit HARQ-ACK feedback signaling is not used to terminate drx-InactivityTimer.

# References

- RP-172811, “Revised WID on Even further enhanced MTC for LTE

- R1-1706805, “Way forward on Uplink HARQ-ACK feedback”, Ericsson

TS 36.321 V14.5.0, “Evolved Universal Terrestrial Radio Access (E-UTRA); Medium Access Control (MAC) protocol specification (Release 14)”, March 2017.

TS 36.331, “RRC protocol specification”, v15.0.1.

R1-1721254, “LS on HARQ-ACK feedback for eFeMTC”

R2-1803886, “Reply LS on HARQ-ACK feedback for efeMTC”

TS 36.306 V14.4.0, “Evolved Universal Terrestrial Radio Access (E-UTRA); User Equipment (UE) radio access capabilities v14.4.0.

R2-1805182 “CR for 36.331: Uplink HARQ ACK feedback for BL UEs and UEs in CE “

R2-1805180 “CR for 36.306: Uplink HARQ ACK feedback for BL UEs and UEs in CE ”