3GPP TSG RAN2 Meeting #101-Bis	R2-1805208

Sanya, China, 16 – 20 April 2018

| CR-Form-v11.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.355 | CR | CRNum | rev | - | Current version: | 14.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network | - X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Introduction of motion states over LPP |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Fraunhofer IIS |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R2 |  |  |  |  |  |  |  |  |  |
| Work item code: | LCS_LTE_acc_enh-Core |  |  |  |  |  |  | Date: |  | 2018-04-05 |
| Category: | B |  |  |  |  |  |  | Release: |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | To support the signalling of UE motion state information over LPP. Rel 15 will support signalling information derrived from IMU measurements. Together with this information, the UE should also report the motion state information. |  |  |  |  |  |  |  |  |
| Summary of change: |  | The changes includes: - Support of motion state information report with low overhead - Informing the location server about the source of the derived motion states. Impact on IMU signaling: - The change does not impact or replace the signaling of IMU measurements. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | If not included, motion state signaling is not available to enhance positioning accuracy while making efficient use of IMU measurements. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.5.5 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  | TS/TR ... CR ... |  |  |  |
| affected: |  |  | x | Test specifications |  |  | TS/TR ... CR ... |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  | TS/TR ... CR ... |  |  |  |
| Other comments: |  | The intention of this CR is to be merged in to the running CR. |  |  |  |  |  |  |  |  |

Beginning of change

### 6.5.5	Sensor based Positioning

#### 6.5.5.1	Sensor Location Information

#### –	Sensor-ProvideLocationInformation

The IE Sensor-ProvideLocationInformation is used by the target device to provide location information for sensor-based methods to the location server. It may also be used to provide sensor specific error reason.

-- ASN1START

Sensor-ProvideLocationInformation-r13 ::= SEQUENCE {

sensor-MeasurementInformation-r13		Sensor-MeasurementInformation-r13		OPTIONAL,

sensor-Error-r13						Sensor-Error-r13						OPTIONAL,

...

}

Sensor-ProvideMotionStateInformation-r15 ::= SEQUENCE {

sensor-MotionStateInformation-r15		Sensor-MotionStateInformation-r15		OPTIONAL,

...

}

-- ASN1STOP

Next change

#### 6.5.5.2	Sensor Location Information Elements

#### –	Sensor-MeasurementInformation

The IE Sensor-MeasurementInformation is used by the target device to provide UE sensor measurements to the location server.

-- ASN1START

Sensor-MeasurementInformation-r13 ::= SEQUENCE {

measurementReferenceTime-r13			UTCTime						OPTIONAL,

uncompensatedBarometricPressure-r13 	INTEGER (30000..115000)		OPTIONAL, -- Cond Barometer

...,

[[

uncertainty-r14					SEQUENCE {

range-r14		INTEGER (0..1000),

confidence-r14	INTEGER (1..100)

}									OPTIONAL

]]

}

-- ASN1STOP

| Conditional presence | Explanation |
| --- | --- |
| Barometer | The field is mandatory present if the Sensor-MeasurementInformation is provided for barometric pressure; otherwise it is not present. |

Sensor-MeasurementInformation field descriptions

measurementReferenceTime This field provides the UTC time when the sensor measurements are performed and should take the form of YYMMDDhhmmssZ.

uncompensatedBarometricPressure This field provides the uncompensated barometric pressure as measured by the UE sensor, in units of Pa.

uncertainty This field provides the expected range for the pressure measurement in units of Pa and the confidence as a percentage that the true pressure lies in a range of (measurement – range) to (measurement + range).

-- ASN1START

Sensor-MotionStateInformation-r15::= SEQUENCE {

motionStateSource-r15				 ENUMERATED{

Baramoter,

IMU,

...

},						OPTIONAL

motionStateList-r15					Motion-State-List-r15		OPTIONAL, -- Cond MotionState

quality-r15							INTEGER (1 .. 100) 			OPTIONAL, -- Cond MotionState

...

}

-- ASN1STOP

Sensor-MotionStateInformation field descriptions

MotionStateSource This field contains the source technology for the estimation of the motion states.

MotionStateList This field specifies one or more elements for the motion state list detected by the target device.

Quality This field specifies a measure derived by the UE on the quality of the obtained motion state information.

-- ASN1START

Motion-State-List-r15	::= SEQUENCE (SIZE(1..4)) OF Motion-State-Element-r15

Motion-State-Element-r15::= SEQUENCE {

MotionState		              ENUMERATED{

unknown,

stationary,

pedestrian,

running,

cycling,

vehicleHighway,

vehicleUrban,

drone,

train,

...

},

Probabilty                     INTEGER(1..100),

}

-- ASN1STOP

Sensor-MotionState-Element field descriptions

MotionState This field contains the identified motion state.

Probabilty This field provides an estimation probability for the motion state to handle estimation ambiguity in percent. The total sum of probabilities in one motion state list has to be 100 percent.

End of changes