3GPP TSG-RAN WG2 Meeting #101bis	R2-1805242

Sanya, China, 16th - 20th April 2018

Agenda item:	10.4.4.2

Source:	Nokia, Nokia Shanghai Bell

Title:	Alternative proposal for signalling differentiation of xDD and FRx capabilities

WID/SID:	NR_newRAT-Core - Release 15

Document for:	Discussion and Decision

# 1	Introduction

In this contribution, we attempt to clarify a few aspects on the UE capability signalling.

# 2	Background

FRX/XDD separation of capabilities

Looking at the latest CR, we have this structure (again with some highlighting to illustrate the points):

-- ASN1START

-- TAG-UE-NR-CAPABILITY-START

UE-NR-Capability ::= SEQUENCE {

pdcp-Parameters					PDCP-Parameters,

rlc-Parameters					RLC-Parameters						OPTIONAL,

mac-Parameters					MAC-Parameters						OPTIONAL,

phy-Parameters					Phy-Parameters,

rf-Parameters					RF-Parameters,

measParameters					MeasParameters						OPTIONAL,

fdd-Add-UE-NR-Capabilities		UE-NR-CapabilityAddXDD-Mode			OPTIONAL,

tdd-Add-UE-NR-Capabilities		UE-NR-CapabilityAddXDD-Mode			OPTIONAL,

fr1-Add-UE-NR-Capabilities		UE-NR-CapabilityAddFRX-Mode			OPTIONAL,

fr2-Add-UE-NR-Capabilities		UE-NR-CapabilityAddFRX-Mode			OPTIONAL,

lateNonCriticalExtension		OCTET STRING						OPTIONAL,

nonCriticalExtension			SEQUENCE {}							OPTIONAL

}

UE-NR-CapabilityAddXDD-Mode ::=	SEQUENCE {

phy-ParametersXDD-Diff			Phy-ParametersXDD-Diff			OPTIONAL,

mac-ParametersXDD-Diff			MAC-ParametersXDD-Diff			OPTIONAL,

measParametersXDD-Diff			MeasParametersXDD-Diff			OPTIONAL

}

UE-NR-CapabilityAddFRX-Mode ::=	SEQUENCE {

phy-ParametersFRX-Diff			Phy-ParametersFRX-Diff			OPTIONAL,

measParametersFRX-Diff			MeasParametersFRX-Diff			OPTIONAL

}

Phy-Parameters ::=	SEQUENCE {

phy-ParametersCommon			Phy-ParametersCommon				OPTIONAL,

phy-ParametersXDD-Diff			Phy-ParametersXDD-Diff				OPTIONAL,

phy-ParametersFRX-Diff			Phy-ParametersFRX-Diff				OPTIONAL,

phy-ParametersFR1				Phy-ParametersFR1					OPTIONAL,

phy-ParametersFR2				Phy-ParametersFR2					OPTIONAL,

supportedBasebandProcessingCombination		SupportedBasebandProcessingCombination,

basebandCombinationParametersUL-List		BasebandCombinationParametersUL-List

}

Now, the green and yellow parts (at the lowest level) are using the exact same IE definitions, so can UE include all parameters in both parts? If it can, what does that mean: Does UE have to be consistent with these, i.e. if the yellow parts are meant for the FRX+XDD differences, shouldn’t the green parts also include the same indications? And if that’s the case, then why do we ever even need the yellow parts at all, since the green parts contain exactly the same information?

Proposal 1:  Clarify why the Phy-ParametersXDD-Diff is repeated in 2 places; one within UE-NR-CapabilityAddXDD-Mode and Phy-Parameters

Proposal 2:  Clarify why the Phy-ParametersFRX-Diff is repeated in 2 places; one within UE-NR-CapabilityAddFRX-Mode and Phy-Parameters

FRX-specific parameters

In the above ASN.1, there are FR1/FR2-specific parameters, meaning parameters that are only relevant for FR1 or FR2. If we have those, could we then also have FDD/TDD-specific parameters in the same way, e.g. as shown below?

Phy-Parameters ::=	SEQUENCE {

phy-ParametersCommon			Phy-ParametersCommon				OPTIONAL,

phy-ParametersXDD-Diff			Phy-ParametersXDD-Diff				OPTIONAL,

phy-ParametersFRX-Diff			Phy-ParametersFRX-Diff				OPTIONAL,

phy-ParametersFR1				Phy-ParametersFR1					OPTIONAL,

phy-ParametersFR2				Phy-ParametersFR2					OPTIONAL,

phy-ParametersFDD				Phy-ParametersFDD					OPTIONAL,

phy-ParametersTDD				Phy-ParametersTDD					OPTIONAL,

supportedBasebandProcessingCombination		SupportedBasebandProcessingCombination

-- FFS on other parameters

}

Proposal 3: Clarify if we will also need FDD/TDD-specific parameters in NR. If yes, consider the proposal below to have FR1/FR2 and TDD/FDD specific capabilities with the other FR1/FR2 capabilities as shown.

UE-NR-Capability ::= SEQUENCE {

pdcp-Parameters					PDCP-Parameters,

rlc-Parameters					RLC-Parameters,						-- FFS OPTIONAL

mac-Parameters					MAC-Parameters,						-- FFS OPTIONAL

phy-Parameters					Phy-Parameters,

rf-Parameters					RF-Parameters,

measParameters					MeasParameters						OPTIONAL,

fdd-Add-UE-NR-Capabilities		UE-NR-CapabilityAddFDD-Mode			OPTIONAL,

tdd-Add-UE-NR-Capabilities		UE-NR-CapabilityAddTDD-Mode			OPTIONAL,

fr1-Add-UE-NR-Capabilities		UE-NR-CapabilityAddFR1-Mode			OPTIONAL,

fr2-Add-UE-NR-Capabilities		UE-NR-CapabilityAddFR2-Mode			OPTIONAL,

nonCriticalExtension			SEQUENCE {}							OPTIONAL

}

UE-NR-CapabilityAddFDD-Mode ::=	SEQUENCE {

fdd-Capabilities			FDD-Parameters						OPTIONAL,

fdd-Add-UE-NR-Capabilities		UE-NR-CapabilityAddXDD-Mode			OPTIONAL

}

UE-NR-CapabilityAddTDD-Mode ::=	SEQUENCE {

tdd-Capabilities			TDD-Parameters						OPTIONAL,

tdd-Add-UE-NR-Capabilities		UE-NR-CapabilityAddXDD-Mode			OPTIONAL

}

UE-NR-CapabilityAddFR1-Mode ::=	SEQUENCE {

phy-ParametersFR1				Phy-ParametersFR1					OPTIONAL,

fr1-Add-UE-NR-Capabilities		UE-NR-CapabilityAddFRX-Mode			OPTIONAL

}

UE-NR-CapabilityAddFR2-Mode ::=	SEQUENCE {

phy-ParametersFR2				Phy-ParametersFR2					OPTIONAL,

fr2-Add-UE-NR-Capabilities		UE-NR-CapabilityAddFRX-Mode			OPTIONAL

}

# 3	Recommendation

Overall picture

If we combine the above, we get the picture that looks like this:

UE-NR-Capability ::= SEQUENCE {

pdcp-Parameters					PDCP-Parameters,

rlc-Parameters					RLC-Parameters,						-- FFS OPTIONAL

mac-Parameters					MAC-Parameters,						-- FFS OPTIONAL

phy-Parameters					Phy-Parameters,

rf-Parameters					RF-Parameters,

measParameters					MeasParameters						OPTIONAL,

fdd-Add-UE-NR-Capabilities		UE-NR-CapabilityAddFDD-Mode			OPTIONAL,

tdd-Add-UE-NR-Capabilities		UE-NR-CapabilityAddTDD-Mode			OPTIONAL,

fr1-Add-UE-NR-Capabilities		UE-NR-CapabilityAddFR1-Mode			OPTIONAL,

fr2-Add-UE-NR-Capabilities		UE-NR-CapabilityAddFR2-Mode			OPTIONAL,

nonCriticalExtension			SEQUENCE {}							OPTIONAL

}

UE-NR-CapabilityAddTDD-Mode ::=	SEQUENCE {

tdd-Capabilities			TDD-Parameters						OPTIONAL,

tdd-Add-UE-NR-Capabilities		UE-NR-CapabilityAddXDD-Mode			OPTIONAL

}

UE-NR-CapabilityAddfDD-Mode ::=	SEQUENCE {

fdd-Capabilities			FDD-Parameters						OPTIONAL,

fdd-Add-UE-NR-Capabilities		UE-NR-CapabilityAddXDD-Mode			OPTIONAL

}

UE-NR-CapabilityAddXDD-Mode ::=	SEQUENCE {

phy-ParametersXDD-Diff			Phy-ParametersXDD-Diff			OPTIONAL,

mac-ParametersXDD-Diff			MAC-ParametersXDD-Diff			OPTIONAL,

measParametersXDD-Diff			MeasParametersXDD-Diff			OPTIONAL

}

FDD-Parameters ::=	SEQUENCE {

-- Empty for now

}

TDD-Parameters ::=	SEQUENCE {

-- Empty for now

}

UE-NR-CapabilityAddFR1-Mode ::=	SEQUENCE {

fr1-Capabilities			FR1-Parameters						OPTIONAL,

fr1-Add-UE-NR-Capabilities		UE-NR-CapabilityAddFRX-Mode			OPTIONAL

}

UE-NR-CapabilityAddFR2-Mode ::=	SEQUENCE {

fr2-Capabilities			FR2-Parameters						OPTIONAL,

fr2-Add-UE-NR-Capabilities		UE-NR-CapabilityAddFRX-Mode			OPTIONAL

}

UE-NR-CapabilityAddFRX-Mode ::=	SEQUENCE {

phy-ParametersFRX-Diff			Phy-ParametersFRX-Diff			OPTIONAL,

measParametersFRX-Diff			MeasParametersFRX-Diff			OPTIONAL

}

FR1-Parameters ::=	SEQUENCE {

phy-ParametersFR1				Phy-ParametersFR1					OPTIONAL,

}

FR2-Parameters ::=	SEQUENCE {

phy-ParametersFR2				Phy-ParametersFR2					OPTIONAL

}

Phy-Parameters ::=	SEQUENCE {

phy-ParametersCommon			Phy-ParametersCommon				OPTIONAL,

}

The matching colours indicate matching parts.

The benefit that we see in this is that it would be very clear where each capabilities are located: Whatever UE supports for XDD/FRX will be in the corresponding container, and the XDD/FRX-specific capabilities can be indicated separately in different IEs. Whatever is common will ONLY be in the common IE, and would only be added to the FRX/XDD-specific parts if we need to split the capability. If a capability depends on both XDD and FRX, then network will have to read all of the corresponding containers to know the UE capabilities, but that seems like the usual procedure anyway.

An excel file is prepared to compare the overhead on the current signalling v/s the alternative proposal.

|  | Current | Current x3 | Current x6 |
| --- | --- | --- | --- |
| Current | 163 | 453 | 896 |
| Nokia | 135 | 352 | 685 |
| Efficiency | 17 % | 22 % | 24 % |

Table: Signalling size reduction due to alternative proposal

Note: Current=latest ASN.1, x3, x6 = multiplication factor for each field element as it grows in the future

ALTERNATIVE PROPOSAL FOR SIGNALLING

Template (Alt.2)

UE-NR-Capability ::=	SEQUENCE {

phy-Parameters						Phy-Parameters,

fdd-Add-UE-NR-Capabilities			UE-NR-CapabilityAddXDD-Mode			OPTIONAL,

tdd-Add-UE-NR-Capabilities			UE-NR-CapabilityAddXDD-Mode			OPTIONAL,

fr1-Add-UE-NR-Capabilities			UE-NR-CapabilityAddFRX-Mode			OPTIONAL,

fr2-Add-UE-NR-Capabilities			UE-NR-CapabilityAddFRX-Mode			OPTIONAL,

nonCriticalExtension				SEQUENCE {}							OPTIONAL

}

UE-NR-CapabilityAddXDD-Mode ::=	SEQUENCE {

phy-ParametersXDD-Diff			Phy-ParametersXDD-Diff			OPTIONAL,

...

}

UE-NR-CapabilityAddFRX-Mode ::=	SEQUENCE {

phy-ParametersFRX-Diff			Phy-ParametersFRX-Diff			OPTIONAL,

...

}

Phy-Parameters ::=	SEQUENCE {

phy-ParametersCommon			Phy-ParametersCommon					OPTIONAL,

}

Phy-ParametersXDD-Diff ::=	SEQUENCE {

-- FDD/TDD diff. needed

featureA						ENUMERATED {supported}					OPTIONAL,

-- Both FDD/TDD and FR1/FR2 diff. needed

featureC						ENUMERATED {supported}					OPTIONAL

}

Phy-ParametersFRX-Diff ::=	SEQUENCE {

-- FR1/FR2 diff. needed

featureB						ENUMERATED {supported}					OPTIONAL,

-- Both FDD/TDD and FR1/FR2 diff. needed

featureC						ENUMERATED {supported}					OPTIONAL

}

Example in NR RRC with explanations

UE-NR-Capability ::= SEQUENCE {

pdcp-Parameters					PDCP-Parameters,

rlc-Parameters					RLC-Parameters,						-- FFS OPTIONAL

mac-Parameters					MAC-Parameters,						-- FFS OPTIONAL

phy-Parameters					Phy-Parameters,

rf-Parameters					RF-Parameters,

measParameters					MeasParameters						OPTIONAL,

fdd-Add-UE-NR-Capabilities		UE-NR-CapabilityAddFDD-Mode			OPTIONAL,

tdd-Add-UE-NR-Capabilities		UE-NR-CapabilityAddTDD-Mode			OPTIONAL,

fr1-Add-UE-NR-Capabilities		UE-NR-CapabilityAddFR1-Mode			OPTIONAL,

fr2-Add-UE-NR-Capabilities		UE-NR-CapabilityAddFR2-Mode			OPTIONAL,

nonCriticalExtension			SEQUENCE {}							OPTIONAL

}

UE-NR-CapabilityAddfDD-Mode ::=	SEQUENCE {

fdd-Capabilities			FDD-Parameters						OPTIONAL,

fdd-Add-UE-NR-Capabilities		UE-NR-CapabilityAddXDD-Mode			OPTIONAL

}

UE-NR-CapabilityAddTDD-Mode ::=	SEQUENCE {

tdd-Capabilities			TDD-Parameters						OPTIONAL,

tdd-Add-UE-NR-Capabilities		UE-NR-CapabilityAddXDD-Mode			OPTIONAL

}

UE-NR-CapabilityAddXDD-Mode ::=	SEQUENCE {

phy-ParametersXDD-Diff			Phy-ParametersXDD-Diff			OPTIONAL,

mac-ParametersXDD-Diff			MAC-ParametersXDD-Diff			OPTIONAL,

measParametersXDD-Diff			MeasParametersXDD-Diff			OPTIONAL

}

FDD-Parameters ::=	SEQUENCE {

-- Empty for now

}

TDD-Parameters ::=	SEQUENCE {

-- Empty for now

}

UE-NR-CapabilityAddFRX-Mode ::=	SEQUENCE {

phy-ParametersFRX-Diff			Phy-ParametersFRX-Diff			OPTIONAL,

measParametersFRX-Diff			MeasParametersFRX-Diff			OPTIONAL

}

UE-NR-CapabilityAddFR1-Mode ::=	SEQUENCE {

fr1-Capabilities			FR1-Parameters						OPTIONAL,

fr1-Add-UE-NR-Capabilities		UE-NR-CapabilityAddFRX-Mode			OPTIONAL

}

UE-NR-CapabilityAddFR2-Mode ::=	SEQUENCE {

fr2-Capabilities			FR2-Parameters						OPTIONAL,

fr2-Add-UE-NR-Capabilities		UE-NR-CapabilityAddFRX-Mode			OPTIONAL

}

FR1-Parameters ::=	SEQUENCE {

phy-ParametersFR1				Phy-ParametersFR1					OPTIONAL,

}

FR2-Parameters ::=	SEQUENCE {

phy-ParametersFR2				Phy-ParametersFR2					OPTIONAL

}

Phy-Parameters ::=	SEQUENCE {

phy-ParametersCommon			Phy-ParametersCommon				OPTIONAL,

}

Phy-ParametersCommon ::=	SEQUENCE {

-- R1 1-9: CSI-RS based CFRA for HO

csi-RS-CFRA-ForHO					ENUMERATED {supported}						OPTIONAL,

}

Phy-ParametersXDD-Diff ::=	SEQUENCE {

-- R1 4-2: 2 PUCCH of format 0 or 2 in consecutive symbols

twoPUCCH-F0-2-ConsecSymbols			ENUMERATED {supported}						OPTIONAL,

}

Phy-ParametersFRX-Diff ::=	SEQUENCE {

-- R1 2-6 & 2-16b: Support 1+2 DMRS (DL/UL)

oneFL-DMRS-TwoAdditionalDMRS		BIT STRING (SIZE (2))						OPTIONAL,

}

Phy-ParametersFR1 ::=	SEQUENCE {

-- R1 3-2: Unicast PDCCH monitoring following Case 1-2

pdcchMonitoringSingleOccasion		ENUMERATED {supported}						OPTIONAL,

}

Phy-ParametersFR2 ::=	SEQUENCE {

-- R4 2-8: PA calibration gap

calibrationGapPA					ENUMERATED {supported}						OPTIONAL

}

# 4	Heading 1

In this contribution, we have analysed the current structure for FRx and xDD differentiation. Based on the analysis above we would like to propose the following:

Proposal 1:  Clarify why the Phy-ParametersXDD-Diff is repeated in 2 places; one within UE-NR-CapabilityAddXDD-Mode and Phy-Parameters

Proposal 2:  Clarify why the Phy-ParametersFRX-Diff is repeated in 2 places; one within UE-NR-CapabilityAddFRX-Mode and Phy-Parameters

Proposal 3: Clarify if we will also need FDD/TDD-specific parameters in NR. If yes, consider the proposal to have FR1/FR2 and TDD/FDD specific capabilities with the other FR1/FR2 capabilities as shown.

Proposal 4: Discuss the alternative proposal in light of the signalling efficiency it brings in.

# References

- 3GPP TS 21.801 Drafting Rules

- Rx-18xxxxx Title - Company