3GPP TSG RAN2 Meeting #101bis	R2-1805262

# Sanya, China, 16th April – 20th April 2018

| CR-Form-v11.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.355 | CR | 0198 | rev | - | Current version: | 14.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | - X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addition of NR CID support to LPP |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R2 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT-Core |  |  |  |  |  |  | Date: |  | 2018-04-06 |
| Category: | B |  |  |  |  |  |  | Release: |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | To provide NR CID support for NR Rel15 devices |  |  |  |  |  |  |  |  |
| Summary of change: |  | Adding the following items to E-CID support: - NR physical cell ID - NR cell global ID - NR ARFCN - NR SFN - NR RSRP/RSRQ results. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | OTDOA support requires assistance information from the LMF which requires this NR CID information to provide proper assistance information. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.5.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | X |  | Other core specifications |  |  | TS/TR ... CR ... 36.305 |  |  |  |
| affected: |  |  | x | Test specifications |  |  | TS/TR ... CR ... |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  | TS/TR ... CR ... |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

Beginning of Change

# 2	References

[x1]	3GPP TS 38.331: "NR; Radio Resource Control (RRC); Protocol specification".

[x2]	3GPP TS 38.215: "NR; Physical layer measurements".

[x3]	3GPP TS 38.133: "NR; Requirements for support of radio resource management".

Next Change

### 6.5.3	Enhanced Cell ID Positioning

#### 6.5.3.1	E-CID Location Information

#### –	ECID-ProvideLocationInformation

The IE ECID-ProvideLocationInformation is used by the target device to provide E-CID location measurements to the location server. It may also be used to provide ECID positioning specific error reason.

-- ASN1START

# ECID-ProvideLocationInformation ::= SEQUENCE {

ecid-SignalMeasurementInformation	ECID-SignalMeasurementInformation		OPTIONAL,

ecid-Error							ECID-Error								OPTIONAL,

...

}

-- ASN1STOP

#### 6.5.3.2	E-CID Location Information Elements

#### –	ECID-SignalMeasurementInformation

The IE ECID-SignalMeasurementInformation is used by the target device to provide various UE-measurements to the location server.

-- ASN1START

# ECID-SignalMeasurementInformation ::= SEQUENCE {

primaryCellMeasuredResults	MeasuredResultsElement	OPTIONAL,

measuredResultsList			MeasuredResultsList,

...

}

# MeasuredResultsList ::= SEQUENCE (SIZE(1..32)) OF MeasuredResultsElement

# MeasuredResultsElement ::= SEQUENCE {

physCellId		INTEGER (0..503),

cellGlobalId	CellGlobalIdEUTRA-AndUTRA			OPTIONAL,

arfcnEUTRA 		ARFCN-ValueEUTRA,

systemFrameNumber

BIT STRING (SIZE (10))				OPTIONAL,

rsrp-Result		INTEGER (0..97)						OPTIONAL,

rsrq-Result		INTEGER (0..34)						OPTIONAL,

ue-RxTxTimeDiff	INTEGER (0..4095)					OPTIONAL,

...,

[[ arfcnEUTRA-v9a0		ARFCN-ValueEUTRA-v9a0 		OPTIONAL		-- Cond EARFCN-max

]],

[[ nrsrp-Result-r14		INTEGER (0..113)			OPTIONAL,

nrsrq-Result-r14		INTEGER (0..74)				OPTIONAL,

carrierFreqOffsetNB-r14

CarrierFreqOffsetNB-r14		OPTIONAL,		-- Cond NB-IoT

hyperSFN-r14			BIT STRING (SIZE (10))		OPTIONAL

]]

[[ nrPhysCellId-r15		INTEGER (0..1008),

nrCellGlabalId-r15	CellGlabalIdNR		  		OPTIONAL,

arfcnNR-r15			ARFCN-ValueNR,

nrSystemFrameNumber-r15

BIT STRING (SIZE (TBD))		OPTIONAL,

nr-RSRP-Result-r15   INTEGER (0..TBD)			OPTIONAL,

nr-RSRQ-Result-r15	INTEGER (0..TBD)			OPTIONAL

]]

}

-- ASN1STOP

| Conditional presence | Explanation |
| --- | --- |
| EARFCN-max | The field is mandatory present if the corresponding arfcnEUTRA (i.e. without suffix) is set to maxEARFCN. Otherwise the field is not present. |
| NB-IoT | The field is mandatory present if the measured cell is a NB-IoT cell. Otherwise it is not present. |

ECID-SignalMeasurementInformation field descriptions

primaryCellMeasuredResults This field contains measurements for the primary cell, when the target device reports measurements for both primary cell and neighbour cells. This field shall be omitted when the target device reports measurements for the primary cell only, in which case the measurements the primary cell is reported in the measuredResultsList.

measuredResultsList This list contains the E-CID measurements for up to 32 cells.

physCellId This field specifies the physical cell identity of the measured cell.

cellGlobalId This field specifies cell global ID of the measured cell. The target device shall provide this field if it was able to determine the ECGI of the measured cell at the time of measurement.

arfcnEUTRA This field specifies the ARFCN of the measured E-UTRA carrier frequency, as defined in [12]. In case the target device includes arfcnEUTRA-v9a0, the target device shall set the corresponding arfcnEUTRA (i.e. without suffix) to maxEARFCN.

systemFrameNumber This field specifies the system frame number of the measured cell during which the measurements have been performed. The target device shall include this field if it was able to determine the SFN of the cell at the time of measurement.

rsrp-Result This field specifies the reference signal received power (RSRP) measurement, as defined in [12], [17].

rsrq-Result This field specifies the reference signal received quality (RSRQ) measurement, as defined in [12], [17].

ue-RxTxTimeDiff This field specifies the UE Rx–Tx time difference measurement, as defined in [17]. It is provided only for measurements on the UE’s primary cell. Measurement report mapping is according to 3GPP TS 36.133 [18].

nrsrp-Result This field specifies the narrowband reference signal received power (NRSRP) measurement, as defined in [17]. Measurement report mapping is according to TS 36.133 [18].

nrsrq-Result This field specifies the narrowband reference signal received quality (NRSRQ) measurement, as defined in [17]. Measurement report mapping is according to TS 36.133 [18].

carrierFreqOffsetNB This field specifies the offset of the NB-IoT channel number to ARFCN given by arfcnEUTRA as defined in TS 36.101 [21].

hyperSFN This field specifies the hyper-SFN of the measured cell during which the measurements have been performed. The target device shall include this field if it was able to determine the hyper-SFN of the cell at the time of measurement.

nrPhysCellId This field specifies the physical cell identity of the measured NR cell. The value 0 indicates that no measured NR cell is available.

nrCellGlobalId This field specifies cell global ID of the measured NR cell. The target device shall provide this field if it was able to determine the ECGI of the measured NR cell at the time of measurement.

arfcnNR This field specifies the ARFCN of the measured NR carrier frequency, as defined in [x1].

nrSystemFrameNumber This field specifies the system frame number of the measured NR cell during which the measurements have been performed. The target device shall include this field if it was able to determine the SFN of the NR cell at the time of measurement.

nr-RSRP-Result This field specifies the NR reference signal received power (NR RSRP)measurement, as defined in [x2]. Measurement report mapping is according to TS 36.133 [x3].

nr-RSRQ-Result This field specifies the NR reference signal received quality (NR RSRQ) measurement, as defined in [x2]. Measurement report mapping is according to TS 36.133 [x3].

#### 6.5.3.3	E-CID Location Information Request

#### –	ECID-RequestLocationInformation

The IE ECID-RequestLocationInformation is used by the location server to request E-CID location measurements from a target device.

-- ASN1START

# ECID-RequestLocationInformation ::= SEQUENCE {

requestedMeasurements		BIT STRING { 	rsrpReq		(0),

rsrqReq		(1),

ueRxTxReq	(2),

nrsrpReq-r14	(3),

nrsrqReq-r14	(4),

nr-rsrpReq-r15  (5),

nr-rsrqReq-r15  (6)} (SIZE(1..8)),

...

}

-- ASN1STOP

ECID-RequestLocationInformation field descriptions

requestedMeasurements This field specifies the E-CID measurements requested. This is represented by a bit string, with a one-value at the bit position means the particular measurement is requested; a zero-value means not requested.

#### 6.5.3.4	E-CID Capability Information

#### –	ECID-ProvideCapabilities

The IE ECID-ProvideCapabilities is used by the target device to indicate its capability to support E-CID and to provide its E-CID location capabilities to the location server.

-- ASN1START

# ECID-ProvideCapabilities ::= SEQUENCE {

ecid-MeasSupported	BIT STRING { 	rsrpSup		(0),

rsrqSup		(1),

ueRxTxSup	(2),

nrsrpSup-r14	(3),

nrsrqSup-r14	(4),

nr-rsrpReq-r15  (5),

nr-rsrqReq-r15  (6)} (SIZE(1..8)),

...,

[[	ueRxTxSupTDD-r13					ENUMERATED { true }				OPTIONAL

]],

[[	periodicalReporting-r14				ENUMERATED { supported }		OPTIONAL,

triggeredReporting-r14				ENUMERATED { supported }		OPTIONAL,

idleStateForMeasurements-r14		ENUMERATED { required }			OPTIONAL

]]

}

-- ASN1STOP

ECID-Provide-Capabilities field descriptions

ecid-MeasSupported This field specifies the E-CID measurements supported by the target device. This is represented by a bit string, with a one-value at the bit position means the particular measurement is supported; a zero-value means not supported. A zero-value in all bit positions in the bit string means only the basic Cell ID positioning method is supported by the target device. If the UE Rx-Tx time difference measurement is supported by the target device (i.e., ueRxTxSup field is set to one), it means that the UE supports the UE Rx-Tx time difference measurement reporting via both LPP signaling and RRC signalling. If a target device doesn’t support LPP, the E-SMLC may assume the target device can not report the UE Rx-Tx time difference measurement results via RRC signalling.

ueRxTxSupTDD This field, if present, indicates that any UE Rx-Tx time difference measurement reporting for TDD from the target device includes the NTAoffset according to [16], [17] and uses the UE Rx-Tx time difference measurement report mapping for TDD as specified in 3GPP TS 36.133 [18]. This field may only be included if the ueRxTxSup field in ecid-MeasSupported is set to value one.

periodicalReporting This field, if present, indicates that the target device supports periodicalReporting of ECID measurements. If this field is absent, the location server may assume that the target device does not support periodicalReporting in CommonIEsRequestLocationInformation.

triggeredReporting This field, if present, indicates that the target device supports triggeredReporting for the cellChange event. If this field is absent, the location server may assume that the target device does not support triggeredReporting in CommonIEsRequestLocationInformation.

idleStateForMeasurements This field, if present, indicates that the target device requires idle state to perform ECID measurements.

#### 6.5.3.5	E-CID Capability Information Request

#### –	ECID-RequestCapabilities

The IE ECID-RequestCapabilities is used by the location server to request E-CID positioning capabilities from a target device.

-- ASN1START

# ECID-RequestCapabilities ::= SEQUENCE {

...

}

-- ASN1STOP

#### 6.5.3.6	E-CID Error Elements

#### –	ECID-Error

The IE ECID-Error is used by the location server or target device to provide E-CID error reasons to the target device or location server, respectively.

-- ASN1START

# ECID-Error ::= CHOICE {

locationServerErrorCauses		ECID-LocationServerErrorCauses,

targetDeviceErrorCauses			ECID-TargetDeviceErrorCauses,

...

}

-- ASN1STOP

#### –	ECID-LocationServerErrorCauses

The IE ECID-LocationServerErrorCauses is used by the location server to provide E-CID error reasons to the target device.

-- ASN1START

# ECID-LocationServerErrorCauses ::= SEQUENCE {

cause		ENUMERATED	{	undefined,

...

},

...

}

-- ASN1STOP

#### –	ECID-TargetDeviceErrorCauses

The IE ECID-TargetDeviceErrorCauses is used by the target device to provide E-CID error reasons to the location server.

-- ASN1START

# ECID-TargetDeviceErrorCauses ::= SEQUENCE {

cause		ENUMERATED {	undefined,

requestedMeasurementNotAvailable,

notAllrequestedMeasurementsPossible,

...

},

rsrpMeasurementNotPossible				NULL		OPTIONAL,

rsrqMeasurementNotPossible				NULL		OPTIONAL,

ueRxTxMeasurementNotPossible			NULL		OPTIONAL,

...,

[[

nrsrpMeasurementNotPossible-r14		NULL		OPTIONAL,

nrsrqMeasurementNotPossible-r14		NULL		OPTIONAL

]],

[[

nrRSRPMeasurementNotPossible-r15		NULL		OPTIONAL,

nrRSRQMeasurementNotPossible-r15		NULL		OPTIONAL

]]

}

-- ASN1STOP

ECID-TargetDeviceErrorCauses field descriptions

cause This field provides a ECID specific error cause. If the cause value is ‘notAllRequestedMeasurementsPossible’, the target device was not able to provide all requested ECID measurements (but may be able to provide some measurements). In this case, the target device should include any of the rsrpMeasurementNotPossible, rsrqMeasurementNotPossible, ueRxTxMeasurementNotPossible, nrsrpMeasurementNotPossible, or nrsrqMeasurementNotPossible fields, as applicable.

End of Changes