3GPP TSG-RAN WG2 #101bis	R2-1805274

# Sanya, China, 16th – 20th April 2018

Agenda Item:	9.16

Source:	Ericsson

Title:	Update of UDC PDCP Control PDU

Document for:	Discussion, Decision

# Introduction

The UDC PDCP control PDU that has been depicted in running CR [1] is very specific which has been described as error handling scenario rather than a generic UDC Feedback packet similar to ROHC or other feature.

# Discussion

From PDCP Running CR,

### 6.2.P PDCP Control PDU for UDC Feedback Packet Figure 6.2.P.1 shows the format of the PDCP Control PDU for UDC Feedback packet. This format is applicable for DRBs configured with UDC. Figure 6.2.P.1: PDCP Control PDU format for UDC Feedback Packet ### 6.3.8 PDU type Length: 3 bits Table 6.3.8.1: PDU type Bit; Description. 000; PDCP status report. 001; Interspersed ROHC feedback packet. 010; LWA status report. 011; LWA end-marker packet. 100; UDC checksum error notification. 101-111; reserved.

As the PDU type is limited, i.e. the UDC PDCP control PDU seems from the name to only be allowed to be used for “UDC checksum error notification”, but from a forward compatibility point of view it would be wise to have generic description. There has been understanding that if further optimization for UDC is needed then the PDCP control PDU can be further used for enhancing other part of UDC functionalities. Considering above, it would be good to have a generic name of PDCP control PDU similar to ROHC.

Current depiction of UDC control PDU is very specific. This will create problem for further extension/usage of this PDCP control PDU for any other purpose

Use a generic name such as UDC Feedback Packet similar to ROHC feedback packet

To do this we could use an R-bit in the PDCP Control PDU for UDC Feedback Packet to indicate that this particular PDU is for indicating “UDC checksum error notification”.

Use one of the R bits to indicate the UDC checksum error notification.

Below is a text proposal for how to implement this in the PDCP spec (based on the current running CR for UDC).

# Text Proposal

### 6.2.P	PDCP Control PDU for UDC Feedback Packet

Figure 6.2.P.1 shows the format of the PDCP Control PDU for UDC feedback packet. This format is applicable for DRBs configured with UDC.

Figure 6.2.P.1: PDCP Control PDU format for UDC feedback packet

Table 6.2.P.1: FE field

| Bit | Description |
| --- | --- |
| 0 | No Error |
| 1 | Checksum Error Notification |

--------------------------------------the next change-------------------------------------

### 6.3.8	PDU type

Length: 3 bits

Table 6.3.8.1: PDU type

| Bit | Description |
| --- | --- |
| 000 | PDCP status report |
| 001 | Interspersed ROHC feedback packet |
| 010 | LWA status report |
| 011 | LWA end-marker packet |
| 100 | UDC feedback packet |
| 101-111 | reserved |

# Conclusion

In section 2 we made the following observations:

Observation 1	Current depiction of UDC control PDU is very specific. This will create problem for further extention/usage of this PDCP control PDU for any other purpose

Based on the discussion in section 2 we propose the following:

Proposal 1	Use a generic name such as UDC Feedback Packet similar to ROHC feedback packet

Proposal 2	Use one of the R bits to indicate the UDC checksum error notification.

# References

R2-1803822 Running CR, “Introduction of DEFLATE based UDC Solution”, CATT