3GPP TSG-RAN WG2 #101bis	R2-1805280

# Sanya, China, 16th – 20th April 2018

Agenda Item:	9.13.8

Source:	Ericsson

Title:		Impact Analysis on NB-TDD MAC RAR Window Size and Contention Resolution Timers

Document for:	Discussion, Decision

# Introduction

In RAN2#101, an email discussion for various timers that is impacted by introduction of NB-TDD was discussed [1].

In this contribution we discuss, the MAC timers ra-ResponseWindowSize and mac-ContentionResolutionTimer which were marked as FFS in RAN2#101.

It is FFS whether ra-ResponseWindowSize should be extended, e.g. from 10.24sec to 15.36sec or 20.48 sec. It is FFS whether max value of mac-ContentionResolutionTimer should be extended, e.g. from 10.24sec to 15.36sec or 20.48 sec.

# Discussion

## Timers Impact

Currently in NB-IoT FDD, the max value for RAR window size and contention resolution is 10.24s.

ra-ResponseWindowSize Duration of the RA response window in TS 36.321 [6]. Value in PDCCH periods. Value pp2 corresponds to 2 PDDCH periods, pp3 corresponds to 3 PDCCH periods and so on. The value considered by the UE is: ra-ResponseWindowSize = Min (signaled value x PDCCH period, 10.24s).

mac-ContentionResolutionTimer Timer for contention resolution in TS 36.321 [6]. Value in PDCCH periods. Value pp1 corresponds to 1 PDCCH period, pp2 corresponds to 2 PDCCH periods and so on. The value considered by the UE is: mac-ContentionResolutionTimer = Min (signaled value x PDCCH period, 10.24s).

However, TDD mode fundamentally differs from FDD mode in the way UL and DL are transmitted. In FDD mode, dedicated uplink and downlink radio frames each on a separate carrier frequency are used. However, in TDD mode, uplink and downlink resources are allocated within the same radio frame sharing the same carrier frequency. Some of the sub-frames are allocated for uplink whereas some are allocated for downlink. In TDD mode different UL/DL configurations are provided as shown in Table 1 [2].

Table 1

| Uplink-downlink configuration | Downlink-to-Uplink Switch-point periodicity | Subframe number |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 |
| 0 | 5 ms | D | S | U | U | U | D | S | U | U | U |
| 1 | 5 ms | D | S | U | U | D | D | S | U | U | D |
| 2 | 5 ms | D | S | U | D | D | D | S | U | D | D |
| 3 | 10 ms | D | S | U | U | U | D | D | D | D | D |
| 4 | 10 ms | D | S | U | U | D | D | D | D | D | D |
| 5 | 10 ms | D | S | U | D | D | D | D | D | D | D |
| 6 | 5 ms | D | S | U | U | U | D | S | U | U | D |

For TDD configuration 6, there are only 3 downlink subframes available in a frame, which makes NB-IoT TDD deployment very difficult. Similarly, for other configurations there are at least 30% less DL subframe as compared to NB-IoT FDD. For example, TDD config 1 has 60% less DL subframes compared to FDD.

Considering, configuration 1, which has the 4 DL subframes in one frame and this is one of the configurations which is fairly commonly used; we see that the current maximum RAR widow size of 10.24s will not be sufficient. DL scheduler in anchor or non-anchor carrier carrying SIB1-NB message may not be able to schedule the Random response immediately. Besides, as per the RAN1 agreement below:

- For NPSS, NSSS and NPBCH transmission in TDD:

  - NPSS is transmitted on subframe #5 in every radio frame

  - NSSS is transmitted on subframe #0 in every even-numbered radio frame

  - NPBCH is in subframe 9 in every radio frame on the same carrier as NPSS/NSSS.

    - Note: TDD configuration 0 may not be possible due to this.

Thus, in TDD config 1, we have 4 DL subframes, 3 subframes are already occupied by NPSS/NSSS/NPBCH, which implies only one DL subframe is left for DL transmissions. That means to send 2048 NPDCCH repetitions it would take 2048 * 10 ms = 20.48s, i.e. longer than the currently support 10.24 s time of the contention resolution and RAR-window.

For TDD config 1, it would take 20.48s to carry the maximum 2048 NPDCCH repetitions in anchor carrier. Similar duration is also expected in non-anchor carrier carrying the SIB1 and SI messages.

Hence, we would like to propose to remove the restriction of 10.24s for RAR window size duration.

Using the same analogy, we would like to propose to remove the restriction of 10.24s for MAC contention resolution timer.

For NB-TDD; the restriction of 10.24s duration for ra-ResponseWindowSize and mac-ContentionResolutionTimer is not applicable. The timer value is still expressed in seconds via the signaled value x PDCCH period.

RAN2 assumes that PDCCH Period definition is same as FDD and hence it is counted including the UL TDD subframes.

## RA-RNTI

Based upon above discussion, it is obvious that RA-RNTI formula for NB-FDD may not be applicable for NB-TDD because of the SFN wrap around issue. Hence, below RA-RNTI formula based upon HFN is proposed for NB-TDD:

RA RNTI=1+ Floor ((SFN_id+1024*mod(HFN_id,sfnCycles ))/4)+ 256*sfnCycles* carrier_id

For NB-TDD; RA-RNTI formula is based upon HFN in addition to SFN to avoid any SFN wrap around issue.

# Text Proposal

## Timers Impact

The below text proposal is suggested for TS 36.331 section 6.7.3.2 NB-IoT Radio resource control information elements

#### RACH-ConfigCommon-NB

The IE RACH-ConfigCommon-NB is used to specify the generic random access parameters.

RACH-ConfigCommon-NB information element

-- ASN1START

RACH-ConfigCommon-NB-r13 ::=		SEQUENCE {

preambleTransMax-CE-r13				PreambleTransMax,

powerRampingParameters-r13			PowerRampingParameters,

rach-InfoList-r13					RACH-InfoList-NB-r13,

connEstFailOffset-r13				INTEGER (0..15)					OPTIONAL,	-- Need OP

...

}

RACH-InfoList-NB-r13 ::=	SEQUENCE (SIZE (1.. maxNPRACH-Resources-NB-r13)) OF RACH-Info-NB-r13

RACH-Info-NB-r13	::=		SEQUENCE {

ra-ResponseWindowSize-r13			ENUMERATED {

pp2, pp3, pp4, pp5, pp6, pp7, pp8, pp10},

mac-ContentionResolutionTimer-r13	ENUMERATED {

pp1, pp2, pp3, pp4, pp8, pp16, pp32, pp64}

}

-- ASN1STOP

RACH-ConfigCommon-NB field descriptions

connEstFailOffset Parameter “Qoffsettemp” in TS 36.304 [4]. If the field is not present the value of infinity shall be used for “Qoffsettemp”.

mac-ContentionResolutionTimer Timer for contention resolution in TS 36.321 [6]. Value in PDCCH periods. Value pp1 corresponds to 1 PDCCH period, pp2 corresponds to 2 PDCCH periods and so on. The value considered by the UE for NB-FDD is: mac-ContentionResolutionTimer = Min (signaled value x PDCCH period, 10.24s). The value considered by the UE for NB-TDD is: ra-ResponseWindowSize = signaled value x PDCCH period.

powerRampingParameters Power ramping step and preamble initial received target power – same as TS 36.213 [23] and TS 36.321 [6]. If more than one repetition level is configured in the cell, then the UE transmits NPRACH with max power except for the lowest repetition level. Otherwise, the UE uses NPRACH power ramping.

preambleTransMax-CE Maximum number of preamble transmission in TS 36.321 [6]. Value is an integer.

ra-ResponseWindowSize Duration of the RA response window in TS 36.321 [6]. Value in PDCCH periods. Value pp2 corresponds to 2 PDDCH periods, pp3 corresponds to 3 PDCCH periods and so on. The value considered by the UE for NB-FDD is: ra-ResponseWindowSize = Min (signaled value x PDCCH period, 10.24s). The value considered by the UE for NB-TDD is: ra-ResponseWindowSize = signaled value x PDCCH period.

## RA-RNTI

### MAC Impacts

The below text proposal is suggested for TS 36.321 section 5.1.4 Random Access Response reception

## 3.2	Abbreviations

For the purposes of the present document, the abbreviations given in TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in TR 21.905 [1].

BL			Bandwidth reduced Low complexity

BR			Bandwidth Reduced

BSR			Buffer Status Report

C-RNTI			Cell RNTI

CBR			Channel Busy Ratio

CC-RNTI			Common Control RNTI

CQI			Channel Quality Indicator

CRI			CSI-RS Resource Indicator

CSI			Channel State Information

eIMTA			Enhanced Interference Management and Traffic Adaptation

eIMTA-RNTI			Enhanced Interference Management and Traffic Adaptation - RNTI

E-UTRA			Evolved UMTS Terrestrial Radio Access

E-UTRAN			Evolved UMTS Terrestrial Radio Access Network

G-RNTI			Group RNTI

HFN			Hyper Frame Number

MAC			Medium Access Control

MCG			Master Cell Group

M-RNTI			MBMS RNTI

MPDCCH			MTC Physical Downlink Control Channel

LCG			Logical Channel Group

NB-IoT			Narrow Band Internet of Things

NPDCCH			Narrowband Physical Downlink Control Channel

NPDSCH			Narrowband Physical Downlink Shared channel

NPRACH			Narrowband Physical Random Access Control Channel

NPUSCH			Narrowband Physical Uplink Shared channel

PCell			Primary Cell

PSCell			Primary Secondary Cell

PHR			Power Headroom Report

PMI			Precoding Matrix Index

PPPP			ProSe Per-Packet Priority

P-RNTI			Paging RNTI

ProSe			Proximity-based Services

pTAG			Primary Timing Advance Group

PTI			Precoding Type Indicator

RA-RNTI			Random Access RNTI

RAI			Release Assistance Indication

RI			Rank Indicator

RN			Relay Node

RNTI			Radio Network Temporary Identifier

SCell			Secondary Cell

SC-FDM			Single-Carrier Frequency Division Multiplexing

SCG			Secondary Cell Group

SCI			Sidelink Control Information

SC-N-RNTI			Single Cell Notification RNTI

SC-PTM			Single Cell Point to Multipoint

SC-RNTI			Single Cell RNTI

SI-RNTI			System Information RNTI

SL			Sidelink

SL-RNTI			Sidelink RNTI

SL-V-RNTI			Sidelink V2X RNTI

SR			Scheduling Request

SRS			Sounding Reference Symbols

SRS-TPC-RNTI			Sounding Reference Symbols-Transmit Power Control-RNTI

SpCell			Special Cell

sTAG			Secondary Timing Advance Group

TAG			Timing Advance Group

TB			Transport Block

TPC-PUCCH-RNTI	Transmit Power Control-Physical Uplink Control Channel-RNTI

TPC-PUSCH-RNTI	Transmit Power Control-Physical Uplink Shared Channel-RNTI

V2X			Vehicle-to-Everything

### 5.1.4	Random Access Response reception

Once the Random Access Preamble is transmitted and regardless of the possible occurrence of a measurement gap or a Sidelink Discovery Gap for Transmission or a Sidelink Discovery Gap for Reception, and regardless of the prioritization of V2X sidelink communication described in subclause 5.14.1.2.2, the MAC entity shall monitor the PDCCH of the SpCell for Random Access Response(s) identified by the RA-RNTI defined below, in the RA Response window which starts at the subframe that contains the end of the preamble transmission [7] plus three subframes and has length ra-ResponseWindowSize. If the UE is a BL UE or a UE in enhanced coverage, RA Response window starts at the subframe that contains the end of the last preamble repetition plus three subframes and has length ra-ResponseWindowSize for the corresponding enhanced coverage level. If the UE is an NB-IoT UE, in case the number of NPRACH repetitions is greater than or equal to 64, RA Response window starts at the subframe that contains the end of the last preamble repetition plus 41 subframes and has length ra-ResponseWindowSize for the corresponding enhanced coverage level, and in case the number of NPRACH repetitions is less than 64, RA Response window starts at the subframe that contains the end of the last preamble repetition plus 4 subframes and has length ra-ResponseWindowSize for the corresponding enhanced coverage level. The RA-RNTI associated with the PRACH in which the Random Access Preamble is transmitted, is computed as:

RA-RNTI= 1 + t_id + 10*f_id

where t_id is the index of the first subframe of the specified PRACH (0≤ t_id <10), and f_id is the index of the specified PRACH within that subframe, in ascending order of frequency domain (0≤ f_id< 6) except for NB-IoT UEs, BL UEs or UEs in enhanced coverage. If the PRACH resource is on a TDD carrier, the f_id is set to , where  is defined in Section 5.7.1 of [7].

For BL UEs and UEs in enhanced coverage, RA-RNTI associated with the PRACH in which the Random Access Preamble is transmitted, is computed as:

RA-RNTI=1+t_id + 10*f_id + 60*(SFN_id mod (Wmax/10))

where t_id is the index of the first subframe of the specified PRACH (0≤ t_id <10), f_id is the index of the specified PRACH within that subframe, in ascending order of frequency domain (0≤ f_id< 6), SFN_id is the index of the first radio frame of the specified PRACH, and Wmax is 400, maximum possible RAR window size in subframes for BL UEs or UEs in enhanced coverage. If the PRACH resource is on a TDD carrier, the f_id is set to , where  is defined in Section 5.7.1 of [7].

For NB-IoT UEs in case of FDD, the RA-RNTI associated with the PRACH in which the Random Access Preamble is transmitted, is computed as:

RA-RNTI=1 + floor(SFN_id/4) + 256*carrier_id

where SFN_id is the index of the first radio frame of the specified PRACH and carrier_id is the index of the UL carrier associated with the specified PRACH. The carrier_id of the anchor carrier is 0.

For NB-IoT UEs in case of TDD, the RA-RNTI associated with the PRACH in which the Random Access Preamble is transmitted, is computed as:

RA-RNTI=1 + floor ((SFN_id + 1024*mod(HFN_id, sfnCycles))/4) + 256*sfnCycles*carrier_id

where SFN_id and HFN_id are the indices of the first radio frame and Hyper frame number of the specified PRACH, carrier_id is the index of the UL carrier associated with the specified PRACH. sfnCycles is configured by upper layers. The carrier_id of the anchor carrier is 0.

The below text proposal is suggested for TS 36.331 section 6.7.3.2 NB-IoT Radio resource control information elements

### RRC Impacts

#### RACH-ConfigCommon-NB

The IE RACH-ConfigCommon-NB is used to specify the generic random access parameters.

RACH-ConfigCommon-NB information element

-- ASN1START

RACH-ConfigCommon-NB-r13 ::=		SEQUENCE {

preambleTransMax-CE-r13				PreambleTransMax,

powerRampingParameters-r13			PowerRampingParameters,

rach-InfoList-r13					RACH-InfoList-NB-r13,

connEstFailOffset-r13				INTEGER (0..15)					OPTIONAL,	-- Need OP

...,

rach-InfoTDDList-r15				RACH-InfoList-NB-TDD-r15		OPTIONAL	-- cond NB-TDD

}

RACH-InfoList-NB-r13 ::=	SEQUENCE (SIZE (1.. maxNPRACH-Resources-NB-r13)) OF RACH-Info-NB-r13

RACH-Info-NB-r13	::=		SEQUENCE {

ra-ResponseWindowSize-r13			ENUMERATED {

pp2, pp3, pp4, pp5, pp6, pp7, pp8, pp10},

mac-ContentionResolutionTimer-r13	ENUMERATED {

pp1, pp2, pp3, pp4, pp8, pp16, pp32, pp64}

}

RACH-InfoList-NB-TDD-r15 ::=	SEQUENCE (SIZE (1.. maxNPRACH-Resources-NB-r13)) OF RACH-Info-NB-    TDD-r15

RACH-Info-NB-TDD-r15	::=		SEQUENCE {

sfnCycles 							ENUMERATED {n1, n2, n3, n4},

...

}

-- ASN1STOP

RACH-ConfigCommon-NB field descriptions

connEstFailOffset Parameter “Qoffsettemp” in TS 36.304 [4]. If the field is not present the value of infinity shall be used for “Qoffsettemp”.

mac-ContentionResolutionTimer Timer for contention resolution in TS 36.321 [6]. Value in PDCCH periods. Value pp1 corresponds to 1 PDCCH period, pp2 corresponds to 2 PDCCH periods and so on. The value considered by the UE for NB-FDD is: mac-ContentionResolutionTimer = Min (signaled value x PDCCH period, 10.24s). The value considered by the UE for NB-TDD is: ra-ResponseWindowSize = signaled value x PDCCH period.

powerRampingParameters Power ramping step and preamble initial received target power – same as TS 36.213 [23] and TS 36.321 [6]. If more than one repetition level is configured in the cell, then the UE transmits NPRACH with max power except for the lowest repetition level. Otherwise, the UE uses NPRACH power ramping.

preambleTransMax-CE Maximum number of preamble transmission in TS 36.321 [6]. Value is an integer.

ra-ResponseWindowSize Duration of the RA response window in TS 36.321 [6]. Value in PDCCH periods. Value pp2 corresponds to 2 PDDCH periods, pp3 corresponds to 3 PDCCH periods and so on. The value considered by the UE for NB-FDD is: ra-ResponseWindowSize = Min (signaled value x PDCCH period, 10.24s). The value considered by the UE for NB-TDD is: ra-ResponseWindowSize = signaled value x PDCCH period.

sfnCycles Used to provide unique RA-RNTI values across SFN cycles as described in TS 36.321 [6]. Value n1 corresponds to 2^1, value n2 corresponds to 2^2 and so on.

| Conditional presence | Explanation |
| --- | --- |
| NB-TDD | The field is optionally present, need ON, if the NB is based upon TDD mode; otherwise it is not present. |

# Conclusion

In section 2 we made the following observations:

Observation 1	For TDD configuration 6, there are only 3 downlink subframes available in a frame, which makes NB-IoT TDD deployment very difficult. Similarly, for other configurations there are at least 30% less DL subframe as compared to NB-IoT FDD. For example, TDD cnfig 1 has 60% less DL subframes compared to FDD.

Observation 2	For TDD config 1, it would take 20.48s to carry the maximum 2048 NPDCCH repetitions in anchor carrier. Similar duration is also expected in non-anchor carrier carrying the SIB1 and SI messages.

Based on the discussion in section 2 we propose the following:

Proposal 1	For NB-TDD; the restriction of 10.24s duration for ra-ResponseWindowSize and mac-ContentionResolutionTimer is not applicable. The timer value is obtained in seconds from signaled value x PDCCH period.

Proposal 2	RAN2 assumes that PDCCH Period is same as FDD and hence it is counted including the UL TDD subframes.

Proposal 3	For NB-TDD; RA-RNTI formula is based upon HFN instead of SFN to avoid any SFN wrap around issue.

# References

R2-1803008, “Email discussion report on Timer impact of TDD”,	Ericsson

3GPP TS 36.211, “Evolved Universal Terrestrial Radio Access  (E-UTRA); Physical Channels and Modulation”.