3GPP TSG-RAN WG2 Meeting #101bis	R2-1805410

# Sanya, P.R. China, 16th – 20th April 2018

| CR-Form-v11.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.321 | CR | 0066 | rev | - | Current version: | 15.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Scaling factor for prioritized Random Access |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R2 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT-Core |  |  |  |  |  |  | Date: |  | 2018-04-03 |
| Category: | F |  |  |  |  |  |  | Release: |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | At RAN2-101, a scaling factor for prioritized Random Access for HO and BFR was agreed. |  |  |  |  |  |  |  |  |
| Summary of change: |  | The scaling factor is introduced in the Random Access procedure in Section 5.1.1 and 5.1.4. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The scaling factor for prioritized Random Access is not supported properly. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.17 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | Y |  | Other core specifications |  |  | TS 38.331 CR 0043 |  |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

### 5.1.1	Random Access procedure initialization

The Random Access procedure described in this subclause is initiated by a PDCCH order, by the MAC entity itself, or by RRC for the events in accordance with TS 38.300 [2]. There is only one Random Access procedure ongoing at any point in time in a MAC entity. The Random Access procedure on an SCell other than PSCell shall only be initiated by a PDCCH order with ra-PreambleIndex different from 0b000000.

NOTE 1:	If the MAC entity receives a request for a new Random Access procedure while another is already ongoing in the MAC entity, it is up to UE implementation whether to continue with the ongoing procedure or start with the new procedure (e.g. for SI request).

RRC configures the following parameters for the Random Access procedure:

-	prach-ConfigIndex: the available set of PRACH occasions for the transmission of the Random Access Preamble;

-	preambleReceivedTargetPower: initial Random Access Preamble power;

-	rsrp-ThresholdSSB: an RSRP threshold for the selection of the SSB and corresponding Random Access Preamble and/or PRACH occasion;

-	csirs-Threshold: an RSRP threshold for the selection of CSI-RS and corresponding Random Access Preamble and/or PRACH occasion;

-	sul-RSRP-Threshold: an RSRP threshold for the selection between the NUL carrier and the SUL carrier;

-	preamblePowerRampingStep: the power-ramping factor;

-	scalingFactorBI: a scaling factor for prioritized random access procedure

-	ra-PreambleIndex: Random Access Preamble;

-	ra-ssb-OccasionMaskIndex: defines in which PRACH occasion associated with an SSB the MAC entity may transmit a Random Access Preamble (see subclause 7.4);

-	preambleTxMax: the maximum number of Random Access Preamble transmission;

-	ssb-perRACH-OccasionAndCB-PreamblesPerSSB (SpCell only): defines the number of SSBs mapped to each PRACH occasion and the number of Random Access Preambles mapped to each SSB;

-	numberOfRA-PreamblesGroupA (SpCell only): defines the number of Random Access Preambles in Random Access Preamble group A for each SSB, if Random Access Preambles group B is configured;

-	If numberOfRA-PreamblesGroupA is equal to the number of Random Access Preambles per SSB configured by ssb-perRACH-OccasionAndCB-PreamblesPerSSB, there is no Random Access Preambles group B.

-	The Random Access Preambles in Random Access Preamble group A are the Random Access Preambles 0 to numberOfRA-PreamblesGroupA – 1, if Random Access Preambles group B is configured; Otherwise, the Random Access Preambles in Random Access Preamble group A are the Random Access Preambles 0 to the number of Random Access Preamles per SSB configured by ssb-perRACH-OccasionAndCB-PreamblesPerSSB;

-	The Random Access Preambles in Random Access Preamble group B, if exists, are the Random Access Preambles numberOfRA-PreamblesGroupA to the number of Random Access Preamles per SSB configured by ssb-perRACH-OccasionAndCB-PreamblesPerSSB.

NOTE 2:	If random Access Preambles group B is supported by the cell and SSBs are mapped to Random Access Preambles, Random Access Preambles group B is included in each SSB.

-	if Random Access Preambles group B exists:

-	ra-Msg3SizeGroupA (per cell): the threshold to determine the groups of Random Access Preambles;

-	deltaPreambleMsg3: ∆PREAMBLE_Msg3 in TS 38.213 [6];

-	messagePowerOffsetGroupB: the power offset for preamble selection.

-	the set of Random Access Preambles and/or PRACH occasions for SI request, if any;

-	the set of Random Access Preambles and/or PRACH occasions for beam failure recovery request, if any;

-	ra-ResponseWindow: the time window to monitor RA response(s) (SpCell only);

-	ra-ContentionResolutionTimer: the Contention Resolution Timer (SpCell only).

In addition, the following information for related Serving Cell is assumed to be available for UEs:

-	if Random Access Preambles group B exists:

-	if the Serving Cell for the Random Access procedure is configured with supplementaryUplink, and SUL carrier is selected for performing Random Access Procedure:

-	PCMAX,f,c of the SUL carrier as specified in TS 38.101 [10].

-	else:

-	PCMAX,f,c of the NUL carrier as specified in TS 38.101 [10].

The following UE variables are used for the Random Access procedure:

-	PREAMBLE_INDEX;

-	PREAMBLE_TRANSMISSION_COUNTER;

-	PREAMBLE_POWER_RAMPING_COUNTER;

-	PREAMBLE_RECEIVED_TARGET_POWER;

-	PREAMBLE_BACKOFF;

-	PCMAX;

-	TEMPORARY_C-RNTI.

When the Random Access procedure is initiated on a Serving Cell, the MAC entity shall:

1>	flush the Msg3 buffer;

1>	set the PREAMBLE_TRANSMISSION_COUNTER to 1;

1>	set the PREAMBLE_POWER_RAMPING_COUNTER to 1;

1>	set the PREAMBLE_BACKOFF to 0 ms;

1>	if the carrier to use for the Random Access procedure is explicitly signalled:

2>	select the signalled carrier for performing Random Access procedure.

1>	else if the carrier to use for the Random Access procedure is not explicitly signalled; and

1>	if the Serving Cell for the Random Access procedure is configured with supplementaryUplink; and

1>	if the RSRP of the downlink pathloss reference is less than sul-RSRP-Threshold:

2>	select the SUL carrier for performing Random Access procedure;

2>	set the PCMAX to PCMAX,f,c of the SUL carrier.

1>	else:

2>	select the normal carrier for performing Random Access procedure;

2>	set the PCMAX to PCMAX,f,c of the NUL carrier.

- perform the Random Access Resource selection procedure (see subclause 5.1.2).

### NEXT CHANGE

### 5.1.4	Random Access Response reception

Once the Random Access Preamble is transmitted and regardless of the possible occurrence of a measurement gap, the MAC entity shall:

1>	if the contention-free Random Access Preamble for beam failure recovery request was transmitted by the MAC entity:

2>	start the ra-ResponseWindow configured in BeamFailureRecoveryConfig at the first PDCCH occasion as specified in TS 38.213 [6] from the end of the Random Access Preamble transmission;

2>	monitor the PDCCH of the SpCell for response to beam failure recovery request identified by the C-RNTI while ra-ResponseWindow is running.

1>	else:

2>	start the ra-ResponseWindow configured in RACH-ConfigCommon at the first PDCCH occasion as specified in TS 38.213 [6] from the end of the Random Access Preamble transmission;

2>	monitor the PDCCH of the SpCell for Random Access Response(s) identified by the RA-RNTI while the ra-ResponseWindow is running.

1>	if notification of a reception of a PDCCH transmission is received from lower layers; and

1>	if PDCCH transmission is addressed to the C-RNTI; and

1>	if the contention-free Random Access Preamble for beam failure recovery request was transmitted by the MAC entity:

2>	consider the Random Access procedure successfully completed.

1>	else if a downlink assignment has been received on the PDCCH for the RA-RNTI and the received TB is successfully decoded:

2>	if the Random Access Response contains a Backoff Indicator subheader:

3> if scalingFactorBI is configured:

4> set the PREAMBLE_BACKOFF to value of the BI field of the Backoff Indicator subheader using Table 7.2-1, multiplied with scalingFactorBI.

3> else:

4>	set the PREAMBLE_BACKOFF to value of the BI field of the Backoff Indicator subheader using Table 7.2-1.

2>	else:

3>	set the PREAMBLE_BACKOFF to 0 ms.

2>	if the Random Access Response contains a Random Access Preamble identifier corresponding to the transmitted PREAMBLE_INDEX (see subclause 5.1.3):

3>	consider this Random Access Response reception successful.

2>	if the Random Access Response reception is considered successful:

3>	if the Random Access Response includes RAPID only:

4>	consider this Random Access procedure successfully completed;

4>	indicate the reception of an acknowledgement for the SI request to upper layers.

3>	else:

4>	apply the following actions for the Serving Cell where the Random Access Preamble was transmitted:

5>	process the received Timing Advance Command (see subclause 5.2);

5>	indicate the preambleReceivedTargetPower and the amount of power ramping applied to the latest Random Access Preamble transmission to lower layers (i.e. (PREAMBLE_POWER_RAMPING_COUNTER – 1) × preamblePowerRampingStep);

5>	if the Serving Cell for the Random Access procedure is SRS-only SCell:

6>	ignore the received UL grant.

5>	else:

6>	process the received UL grant value and indicate it to the lower layers.

4>	if the Random Access Preamble was not selected by the MAC entity among the contention-based Random Access Preamble(s):

5>	consider the Random Access procedure successfully completed.

4>	else:

5>	set the TEMPORARY_C-RNTI to the value received in the Random Access Response;

5>	if this is the first successfully received Random Access Response within this Random Access procedure:

6>	if the transmission is not being made for the CCCH logical channel:

7>	indicate to the Multiplexing and assembly entity to include a C-RNTI MAC CE in the subsequent uplink transmission.

6>	obtain the MAC PDU to transmit from the Multiplexing and assembly entity and store it in the Msg3 buffer.

1>	if ra-ResponseWindow configured in RACH-ConfigCommon expires, and if the Random Access Response containing Random Access Preamble identifiers that matches the transmitted PREAMBLE_INDEX has not been received; or:

1>	if ra-ResponseWindow configured in BeamFailureRecoveryConfig expires and if the PDCCH addressed to the C-RNTI has not been received:

2>	consider the Random Access Response reception not successful;

2>	increment PREAMBLE_TRANSMISSION_COUNTER by 1;

2>	if PREAMBLE_TRANSMISSION_COUNTER = preambleTxMax + 1:

3>	if the Random Access Preamble is transmitted on the SpCell:

4>	indicate a Random Access problem to upper layers.

3>	else if the Random Access Preamble is transmitted on a SCell:

4>	consider the Random Access procedure unsuccessfully completed.

2>	if in this Random Access procedure, the Random Access Preamble was selected by MAC among the contention-based Random Access Preambles:

3>	select a random backoff time according to a uniform distribution between 0 and the PREAMBLE_BACKOFF;

3>	delay the subsequent Random Access Preamble transmission by the backoff time.

2>	perform the Random Access Resource selection procedure (see subclause 5.1.2).

The MAC entity may stop ra-ResponseWindow (and hence monitoring for Random Access Response(s)) after successful reception of a Random Access Response containing Random Access Preamble identifiers that matches the transmitted PREAMBLE_INDEX.

HARQ operation is not applicable to the Random Access Response transmission.