3GPP TSG-RAN WG2 Meeting #101bis	R2-1805423

# Sanya, P.R. China, 16th – 20th April 2018

| CR-Form-v11.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.321 | CR | 0070 | rev | - | Current version: | 15.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CBRA BFR on SpCell and CFRA BFR on SCell |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R2 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT-Core |  |  |  |  |  |  | Date: |  | 2018-04-03 |
| Category: | F |  |  |  |  |  |  | Release: |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | 1. As agreed at RAN2-101, the RA configuration in BeamRecoveryConfig is not applicable for contention based random access. Instead, the UE shall apply the normal RA configuration (in rach-ConfigCommon) for contention based beam failure recovery. However, this is not clear from the current MAC specification. The beam failure recovery procedure should be clarified for the case of contention-based random access. 2. RAN2 has agreed to support CFRA CBFR on SCell. This is not implemented. |  |  |  |  |  |  |  |  |
| Summary of change: |  | 1. The absence of beamRecoveryConfig is interpreted by the UE to perform BFR using CBRA. 2. Support for CFRA BFR on SCell is added. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | 1. CBRA on BFR on SpCell is not supported properly since it remains unclear which RA configuration the UE shall apply in this case. 2. CFRA BFR on SCell is not supported. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.17 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

## 5.17	Beam Failure Detection and Recovery procedure

The MAC entity may be configured by RRC with a beam failure recovery procedure which is used for indicating to the serving gNB of a new SSB or CSI-RS when beam failure is detected on the serving SSB(s)/CSI-RS(s). Beam failure is detected by counting beam failure instance indication from the lower layers to the MAC entity.

RRC configures the following parameters in the BeamFailureRecoveryConfig for the Beam Failure Detection and Recovery procedure:

-	beamFailureInstanceMaxCount for the beam failure detection;

-	beamFailureDetectionTimer for the beam failure detection;

-	beamFailureCandidateBeamThreshold: an RSRP threshold for the beam failure recovery;

-	preamblePowerRampingStep: preamblePowerRampingStep for the beam failure recovery;

-	preambleReceivedTargetPower: preambleReceivedTargetPower for the beam failure recovery;

-	preambleTxMax: preambleTxMax for the beam failure recovery;

-	ra-ResponseWindow: the time window to monitor response(s) for the beam failure recovery using contention-free Random Access Preamble.

The following UE variables are used for the beam failure detection procedure:

-	BFI_COUNTER: counter for beam failure instance indication which is initially set to 0.

The MAC entity shall:

1>	if beam failure instance indication has been received from lower layers:

2>	start or restart the beamFailureDetectionTimer;

2>	increment BFI_COUNTER by 1;

2>	if BFI_COUNTER = beamFailureInstanceMaxCount + 1:

3>	if beamFailureRecoveryConfig is configured for the serving cell that triggered the beam failure:

4>	initiate a Random Access procedure (see subclause 5.1) on the serving cell that triggered the beam failure by applying the parameters configured in beamFailureRecoveryConfig.

3>	else if the serving cell that triggered the beam failure is SpCell:

4>	initiate a Random Access procedure (see subclause 5.1) on the SpCell by applying the parameters configured in rach-ConfigCommon;

1>	if the beamFailureDetectionTimer expires:

2>	set BFI_COUNTER to 0.

1>	if the Random Access procedure is successfully completed (see subclause 5.1):

2>	consider the Beam Failure Recovery procedure successfully completed.