3GPP TSG-RAN WG2 #101Bis	Tdoc R2-1805449

Sanya, P.R. of China, 16th – 20th April 2018

Agenda Item:	10.2.9

Source:	Ericsson

Title:	NR RRC design principles for NE-DC

Document: 	Discussion, Decision

# Introduction

RAN2 has not progressed in Stage-3 details of NR-E-UTRA Dual Connectivity (NE-DC). In order to ensure the progress on the main RRC aspects, in this contribution, we will be discussing the RRC design principles for NE-DC both from the RRC architecture and ASN.1 design perspectives.

# Discussion

## RRC baseline for NE-DC

In EN-DC, as the primarily specified DC scenario, we agreed that the both MN and SN generate ASN.1 [1]. In addition, it was agreed that ASN.1 generated by the secondary node (SN) can be transported by the master node (MN) at least in some cases, e.g., for the first configuration or the configurations requiring capability coordination between MN and SN. Furthermore, while LTE RRC Connection Reconfiguration message is re-used, NR lower layer configurations generated by NR RRC are encapsulated into the LTE RRC message as an NR RRC message. Further, it was decided that separate containers for NR PDCP configurations are used allowing flexible mapping between NR PDCP entities and lower layers (LTE or NR).

Next, we need to specify the RRC design principles for the MR-DC cases. Considering that we have many other DC cases in addition to EN-DC, i.e., NE-DC (Option 4), ng-EN-DC (Option 7) and NN-DC, ideally it would be optimal to have a common RRC baseline for all. This way, the specified RRC architecture and ASN.1 structure could be re-used as much as possible. Further, the efforts needed for both the network and the UE implementations could be significantly reduced.

It would simplify both the network and the UE implementations if ASN.1 design structure follows the same baseline for the MR-DC cases.

In this context, the ASN.1 design of EN-DC can be considered as the baseline in NE-DC where the baseline can be complemented with the necessary changes. Furthermore, the supported bearer types can be harmonized among different DC options as illustrated for EN-DC and NE-DC options in Figures 1 and 2. In case of NE-DC, the PDCP version for the supported bearers are proposed to be the NR PDCP to minimize the options and harmonize the procedures for the other NR/eLTE DC options in the future, whereas in EN-DC LTE PDCP could also be used for MCG bearers.

The ASN.1 design of EN-DC is taken as the baseline for NE-DC where the baseline can be complemented with the necessary changes.

In case of NE-DC, only NR PDCP version(s) is supported, i.e., no LTE PDCP configuration is supported.

Figure 1. Supported bearers in EN-DC.

Figure 2. Supported bearers in NE-DC assuming EN-DC as the baseline.

## Current ASN.1 structure for EN-DC

RAN2 has agreed that NR configuration embedded in the LTE RRCConnectionReconfiguration is RRC PDU. Later on, it was agreed that this is same as NR RRCReconfiguration message. In addition, it was agreed that bearer configuration is given in another transparent container, one used to configure bearers using KeNB and constructed by the MN and another used to configure bearers using S-KgNB and constructed by the SN.

In the latest merged CR for NR RRC specifications, NR RRC Reconfiguration structure is defined as follows:

-- ASN1START

-- TAG-RRCRECONFIGURATION-START

RRCReconfiguration ::= 				SEQUENCE {

rrc-TransactionIdentifier			RRC-TransactionIdentifier,

criticalExtensions					CHOICE {

rrcReconfiguration					RRCReconfiguration-IEs,

criticalExtensionsFuture			SEQUENCE {}

}

}

RRCReconfiguration-IEs ::= 			SEQUENCE {

-- Configuration of Radio Bearers (DRBs, SRBs) including SDAP/PDCP.

-- In EN-DC this field may only be present if the RRCReconfiguration

-- is transmitted over SRB3.

radioBearerConfig						RadioBearerConfig 														OPTIONAL, -- Need M

-- Configuration of secondary cell group (EN-DC):

secondaryCellGroup						OCTET STRING (CONTAINING CellGroupConfig)								OPTIONAL, -- Need M

measConfig								MeasConfig																OPTIONAL, -- Need M

lateNonCriticalExtension				OCTET STRING															OPTIONAL,

nonCriticalExtension					SEQUENCE {}																OPTIONAL

}

-- TAG-RRCRECONFIGURATION-STOP

-- ASN1STOP

In the embedded NR RRC PDU in the LTE RRCConnectionReconfiguration, the fields that can be present are

- secondaryCellGroup to configure SCG

- measConfig to configure NR measurements

It was also agreed in RAN2#100 that, for EN-DC for the Dec -17 drop, the UE behaviour will only be specified for the case that the embedded NR RRC PDU in the LTE RRRCConnectionReconfiguration includes secondaryCellGroup and/or measConfig and nothing else.

Furthermore, RadioBearerConfig structure below can be used to reconfigure PDCP via SRB1 at least for the cases when the relevant parameters are needed to be coordinated with MN. NR PDCP configuration can be sent via SRB3 when the coordination with MN is not needed.

RadioBearerConfig information element

-- ASN1START

-- TAG-RADIO-BEARER-CONFIG-START

RadioBearerConfig ::=					SEQUENCE {

srb-ToAddModList						SRB-ToAddModList										OPTIONAL, -- Need N

srb3-ToRelease							ENUMERATED{true}										OPTIONAL, -- Need N

drb-ToAddModList						DRB-ToAddModList										OPTIONAL, -- Need N

drb-ToReleaseList						DRB-ToReleaseList										OPTIONAL, -- Need N

securityConfig 							SecurityConfig											OPTIONAL, -- Cond M

...

}

SRB-ToAddModList ::=					SEQUENCE (SIZE (1..2)) OF SRB-ToAddMod

SRB-ToAddMod ::=						SEQUENCE {

srb-Identity							SRB-Identity,

-- may only be set if the cell groups of all linked logical channels are reset or released

reestablishPDCP							ENUMERATED{true}										OPTIONAL, 		-- Need N

discardOnPDCP                           ENUMERATED{true}										OPTIONAL,		-- Need N

pdcp-Config								PDCP-Config												OPTIONAL,		-- Cond PDCP

...

}

DRB-ToAddModList ::=					SEQUENCE (SIZE (1..maxDRB)) OF DRB-ToAddMod

DRB-ToAddMod ::=						SEQUENCE {

cnAssociation							CHOICE {

-- The EPS bearer ID determines the EPS bearer when NR connects to EPC using EN-DC

eps-BearerIdentity						INTEGER (0..15),												-- EPS-DRB-Setup

--	The SDAP configuration determines how to map QoS flows to DRBs when NR connects to the 5GC

sdap-Config								SDAP-Config													-- 5GC

} 										OPTIONAL, -- Cond DRBSetup

drb-Identity							DRB-Identity,

-- may only be set if the cell groups of all linked logical channels are reset or released

reestablishPDCP						ENUMERATED{true}										OPTIONAL, 		-- Need N

recoverPDCP							ENUMERATED{true}										OPTIONAL, 		-- Need N

pdcp-Config							PDCP-Config												OPTIONAL,		-- Cond PDCP

...

}

DRB-ToReleaseList ::=					SEQUENCE (SIZE (1..maxDRB)) OF DRB-Identity

SecurityConfig ::=						SEQUENCE {

securityAlgorithmConfig				SecurityAlgorithmConfig									OPTIONAL,	-- Cond RBTermChange

keyToUse							ENUMERATED{keNB, s-KgNB}								OPTIONAL,	-- Cond RBTermChange

...

}

-- TAG-RADIO-BEARER-CONFIG-STOP

-- ASN1STOP

These parameters are for the bearers configured with NR PDCP. Rest of the parameters the UE gets via LTE RRC (SecurityModeCommand and RRCConnectionReconfiguration with MobilityControlInfo).

## Introduction NE-DC

When NE-DC are introduced, there are some open issues expected to be discussed regarding the RRC procedures and ASN.1:

- Is SN configuration constructed from SCG configuration and bearer configuration?

- Is SCG configuration a complete RRC message or IE(s)?

- Where does the RRC signalling procedure terminate?

- How is RLF handled?

- Is SRB3 supported?

- How is measurement configuration handled?

- How is security configured?

In this contribution we address Questions 1, 2 and 3. Questions 4, 5, 6 and 7 are addressed in [2], [3], [4], and [5], respectively.

We consider that it is beneficial to reuse the structure where PDCP configuration and SCG configuration are separated also in NE-DC. This makes it possible to use harmonised bearer concept also in the NE-DC scenario as well as for the split architecture option.

In NE-DC, separate bearer configuration fields (similar to nr-RadioBearerConfig1&2 in LTE RRC) are introduced per termination point in NR RRC.

We consider that SCG configuration does not need to be full RRC message in NE-DC. That is because in case of NE-DC, NR RRC message should not encapsulate any unnecessary LTE RRC fields, which will become a significant overhead. The required SCG configuration can be provided by means of Information Elements (IEs), like RadioBearerConfig, which is generated by NR RRC, in case of EN-DC. The IEs regarding the SN  configuration includes RadioBearerConfigDedicated (with no PDCP configuration present) and MeasConfig since LTE SN should configure the LTE measurements as proposed in [4], minimizing the impact to NR RRC specifications.  In case of NE-DC, RRC signalling terminates at MN even though IE(s) can be generated in either MN or SN.

In NE-DC, SCG configuration is not a separate RRC PDU but IE(s) seen as octet string by the MN RRC.

In NE-DC, the IEs regarding SN configuration is comprised of RadioBearerConfigDedicated (with no PDCP configuration present) and MeasConfig.

In case of NE-DC, “RRC” signalling procedure always terminates at MN even though IE(s) can be generated in either MN or SN.

The ASN.1 example for the NE-DC extension in NR RRC (TS 38.331) is given below:

-- ASN1START

-- TAG-RRCRECONFIGURATION-START

RRCReconfiguration ::= 				SEQUENCE {

rrc-TransactionIdentifier			RRC-TransactionIdentifier,

criticalExtensions					CHOICE {

rrcReconfiguration					RRCReconfiguration-IEs,

criticalExtensionsFuture			SEQUENCE {}

}

}

RRCReconfiguration-IEs ::= 			SEQUENCE {

-- Configuration of Radio Bearers (DRBs, SRBs) including SDAP/PDCP.

-- In EN-DC this field may only be present if the RRCReconfiguration

-- is transmitted over SRB3.

radioBearerConfig						RadioBearerConfig 														OPTIONAL, -- Need M

-- Configuration of secondary cell group (EN-DC):

secondaryCellGroup						OCTET STRING (CONTAINING CellGroupConfig)								OPTIONAL, -- Need M

measConfig								MeasConfig																OPTIONAL, -- Need M

lateNonCriticalExtension				OCTET STRING															OPTIONAL,

nonCriticalExtension					SEQUENCE {}																OPTIONAL

}

RRCReconfiguration-v15x0-IEs ::= SEQUENCE {

lte-Config-r15					CHOICE {

release							NULL,

setup							SEQUENCE {

nedc-ReleaseAndAdd-r15	BOOLEAN,

lte-RadioBearerConfigDedicated -r15	OCTET STRING			OPTIONAL,	-- Need ON

lte-MeasConfig				   -r15	OCTET STRING			OPTIONAL,	-- Need ON

p-Maxlte-r15		    		P-Max						OPTIONAL	-- Need ON

}

}																	    OPTIONAL,	-- Need ON

sk-Counter-r15					INTEGER (0.. 65535)					    OPTIONAL,	-- Need ON

nr-RadioBearerConfig1-r15	OCTET STRING (CONTAINING RadioBearerConfig)	OPTIONAL,	-- Need ON

nr-RadioBearerConfig2-r15	OCTET STRING (CONTAINING RadioBearerConfig)	OPTIONAL,	-- Need ON

nonCriticalExtension			SEQUENCE {}							    OPTIONAL

}

-- TAG-RRCRECONFIGURATION-STOP

-- ASN1STOP

# Conclusion

Based on the discussion in section 2, we have the following observations:

- It would simplify both the network and the UE implementations if ASN.1 design structure follows the same baseline for the MR-DC cases.

Further, we propose the following:

- The ASN.1 design of EN-DC is taken as the baseline for NE-DC where the baseline can be complemented with the necessary changes.

In case of NE-DC, only NR PDCP version(s) is supported, i.e., no LTE PDCP configuration is supported.

In NE-DC, separate bearer configuration fields (similar to nr-RadioBearerConfig1&2 in LTE RRC) are introduced per termination point in NR RRC.

In NE-DC, SCG configuration is not a separate RRC PDU but IE(s) seen as octet string by the MN RRC.

In NE-DC, the IEs regarding SN configuration is comprised of RadioBearerConfigDedicated (with no PDCP configuration present) and MeasConfig.

In case of NE-DC, “RRC” signalling procedure always terminates at MN even though IE(s) can be generated in either MN or SN.

# References

- 3GPP TS 37.340, 3rd Generation Partnership Project; Technical Specification Group Radio Access Network; Evolved Universal Terrestrial Radio Access (E-UTRA) and NR; Multi-connectivity; Stage 2 (Release 15).

- R2-1805449, MCG RLF handling in case of NE-DC (TP to 37.340), Ericsson, 3GPP TSG-RAN WG2 #101Bis, Sanya, P.R. of China, 16th – 20th April 2018.

- R2-1805450, Support of SRB3 for NE-DC, Ericsson, 3GPP TSG-RAN WG2 #101Bis, Sanya, P.R. of China, 16th – 20th April 2018.

- R2-1805451, NE-DC measurement principles, Ericsson, 3GPP TSG-RAN WG2 #101Bis, Sanya, P.R. of China, 16th – 20th April 2018.

- R2-1804798, Security key update during handover, Ericsson, 3GPP TSG-RAN WG2 #101Bis, Sanya, P.R. of China, 16th – 20th April 2018.