3GPP TSG-RAN WG2 #101bis	Tdoc R2-1805507

# Sanya, P. R China, 16 – 20 April 2018

Agenda Item:	10.3.4.3

Source:	Ericsson

Title:	QoS Flow remapping and end marker

Document for:	Discussion, Decision

# Introduction

RAN2#101-Athens agreed:

Agreements:

=>	For DL it is left up to gNB implementation.  FFS if RAN3 signalling is required.

=>	FFS - We define an end/start marker on UE side and how it is used it is up to gNB implementation.   At least for RLC AM the start/end marker solution is used as a baseline.

This contribution discusses stage 3 details of UL end marker solution on QoS Flow remapping.

# Discussion

When a QoS flow is remapped, the remapping is done from a source DRB to a target DRB. In this case the SDAP entity is responsible of making the path switch, which can be made based on RDI indication or RRC signalling. After the path switch, packets which have already been transmitted to source PDCP entity will be transmitted via source path and new SDAP SDUs will be transmitted via new path, i.e. target DRB. Figure 1 illustrates this scenario.

Figure 1: Flow Relocation with PDCP receiver buffering and UE end marker

Upon receiving indication either via RRC signalling or via RDI bit on SDAP header, the SDAP entity shall re-route all new traffic to target DRB.

RAN2 has agreed that the SDAP layer does not have buffers as well as the PDCP layer does not have concept of QoS flows. This principal is important to maintain, in order to keep low complexity and not to introduce unnecessary inter layer violations. If changes to these principles were to be agreed, conscious decision to place SDAP functionality outside PDCP would need to be revisited. For example, in PDCP the SDAP header or information related to each PDU QFI field would need to be known.

As a result of the above, and of remapping through reflective QoS or RRC reconfiguration, it is the SDAP entity that should create the end marker, or request PDCP entity to create the end marker, and it should be up to the PDCP entity to buffer the traffic.

Upon receiving QoS Flow mapping via RRC signalling or RDI indication from SDAP header, the SDAP layer shall re-route all new traffic to target DRB.

As buffers do not exist at the SDAP layer, the target PDCP entity instantiated to serve the remapped QoS flow may start buffering of all the traffic upon receiving indication from SDAP layer. Buffering at lower layers, e.g. RLC would mean introducing the need to propagate the information from SDAP layer to RLC along with creating RLC SDUs with associated SN. This creates additional complexity in handling and interlayer interactions from SDAP to both PDCP and RLC.

Target PDCP entity performs buffering based on SDAP indication.

The PDCP buffering may be located on transmitter or receiver side. The transmitter side buffering requires UE to consume memory resources for buffering, which may end up being expensive w.r.t. resource and processing, especially in high data rate scenarios. On the other hand, receiver side buffering (gNB), relaxes UE requirements on buffering as the UE may directly transmit packets to gNB (source DRB). Also, here a quick release to higher layers not incurring the Uu delay can be achieved. A benefit of a transmitter side solution is that the upon receiving the end marker, the end marker receive indication do not need to be transmitted over the air. Also buffering of other data on a (active) target DRB can be avoided (see section 2.1 on multiple relocations).

Note that as the network is the one originating the Flow relocation request, network side buffering and actions/timing related to end marker can be left for network implementation.

Receiver side buffering relaxes UE requirements

Receiver side buffering results in faster reaction to end marker as the receiver may directly release buffers upon receiving the end marker.

PDCP entity of the target DRB buffers the packets.

Buffering on RX Target PDCP entity is left for network implementation.

As described above, the SDAP layer has two options, either it may create the end marker or request PDCP to create the end marker. The PDCP entity of the source DRB may have multiple DRBs mapped on it. In the case where multiple DRBs are sending traffic meanwhile QoS Flow is remapped, the receiver PDCP entity of the source DRB needs to maintain information about the PDCP SN at the point of remapping (e.g. indication). This SN information will help the RX source PDCP entity to decide whether to indicate to target that the buffering can be finished.

PDCP SN needs to be associated with end marker as there may be multiple QFIs mapped on same DRB while Flow is relocated.

A straightforward option is to use a PDCP control PDU as it is able to associate/include information about the PDCP SN related to the end marker across PDCP PDUs. Using a PDCP control PDU including a SN may also handle cases for where e.g. retransmissions result in end marker arriving earlier than the last SN at the receiver.

PDCP control PDU is used as end marker.

PDCP SN is associated with the end marker.

In a transmitter side buffering solution (UE side buffering); as the end marker (for example as a PDCP control PDU) can be lost over the air, a timer could be used to allow completion of a remapping procedure. The value of this timer can be configured based on the QoS characteristics or other criteria associated with the flows that are being buffered.

## Relocation of multiple QoS Flows

As already described above, the new QoS framework allows mapping of multiple QoS flows to DRBs simultaneously. Additionally, the new QoS Framework supports mapping of large amount of QoS Flow to DRBs (up to 64 with 6-bit QFI). This results in a situation where multiple parallel QoS Flow relocations are on-going can be expected. As the PDCP is not aware of QoS Flows, in order to block traffic from one QoS Flow, the PDCP needs to block traffic of all QoS Flows associated with the DRB.

As the PDCP is not aware of QoS Flows, it needs to block all traffic on DRB meanwhile QFI is being relocated.

The figure below illustrates a problem on relocation of multiple QoS flows simultaneously. For example, when considering a case where two QoS Flows, Flow 1 and Flow 2, are relocated to a new, DRB 1. Flow 1 is associated with an End marker EM1 and QoS Flow 2 is associated with the end marker EM2. Because PDCP/RLC cannot be aware of QoS Flows, there is a need to block all outgoing traffic from DRB 1 until the relocation is completed. In the case where only one flow is relocated at the time, the PDCP could release the traffic from DRB 1 upon receiving any end marker. When multiple QFIs are activated simultaneously, the delay the flow relocation procedures incur depend on the completion of all relocated flows. This would be problematic especially when multiple or all QFIs are mapped to a default DRB when traffic is established. Delaying each activation of RAN QFI remapping procedure by picking one QFI at the time procedure may result in an undefined behaviour as the 5GC and end application assumes that the QFI has already been established. Additionally, long delays on critical QFIs may result that PDB is exceeded. Note that it does not matter for the end user whether packets reach the destination too late or are dropped.

Figure 2: Two QoS Flows  are relocated simultaneously, both are sending end markers to target PDCP entity

With the new QoS framework and Default DRB concept, it does not seem practical to relocate one QFI at the time.

Multiple QoS Flows relocated simultaneously is supported

In order to identify which QoS Flow the end marker is associated to, the SDAP entity needs to provide identification for the QoS Flow. This identification does not necessarily need to be QFI, but the QFI could be used for this purpose. The buffering PDCP entity would get flow identifier from the SDAP entity when it starts buffering as well as when the end marker arrives. This way the PDCP entity is able to know which QFI finished relocation. Signalling the Flow information to the PDCP would not require PDCP entity to peek into SDAP header and therefore it would allow clean protocol design principles. It is worth of noting that signalling the Flow identifier to PDCP would not mean that the PDCP needs to be aware of QoS Flows, the PDCP entity would be just matching this identifier with another identifier to be able to know when the buffering may end and at what SN this would relate to.

SDAP provides target DRB Id and Flow identifier to source PDCP entity

Signalling of QFI and target DRB id can be left for UE implementation

# Conclusion

In section 2 we made the following observations:

- Observation 1	Receiver side buffering relaxes UE requirements

- Observation 2	Receiver side buffering results in faster reaction to end marker as the receiver may directly release buffers upon receiving the end marker.

Observation 3	PDCP SN needs to be associated with end marker as there may be multiple QFIs mapped on same DRB while Flow is relocated.

Observation 4	As the PDCP is not aware of QoS Flows, it needs to block all traffic on DRB meanwhile QFI is being relocated.

Observation 5	With the new QoS framework and Default DRB concept, it does not seem practical to relocate one QFI at the time.

Based on the discussion in section 2 we propose the following:

Proposal 1	Upon receiving QoS Flow mapping via RRC signalling or RDI indication from SDAP header, the SDAP layer shall re-route all new traffic to target DRB.

Proposal 2	Target PDCP entity performs buffering based on SDAP indication

Proposal 3	PDCP entity of the target DRB buffers the packets.

Proposal 4	Buffering on RX Target PDCP entity is left for network implementation

Proposal 5	PDCP control PDU is used as end marker

Proposal 6	PDCP SN is associated with the end marker

Proposal 8	SDAP provides target DRB Id and Flow identifier to source PDCP entity

Proposal 9	Signalling of QFI and target DRB id can be left for UE implementation