3GPP TSG-RAN WG2 Meeting #101bis	R2-1805603

# Sanya, P.R. China, 16th – 20th April 2018

| CR-Form-v11.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.321 | CR | 0073 | rev | - | Current version: | 15.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Corrections to DRX timer operation |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R2 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT-Core |  |  |  |  |  |  | Date: |  | 2018-04-05 |
| Category: | F |  |  |  |  |  |  | Release: |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | The current specification is not precise enough when it comes to starting and stopping of some DRX timers. Additionally RAN2 has agreed to include an Annex describing the DRX timer operation, but there is no annex in the specification. |  |  |  |  |  |  |  |  |
| Summary of change: |  | The starting time for the HARQ RTT timers are clarified and an annex describing the DRX timers is added. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The DRX timer operation remains ambiguous. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.7, Annex X (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

## 5.7	Discontinuous Reception (DRX)

The MAC entity may be configured by RRC with a DRX functionality that controls the UE's PDCCH monitoring activity for the MAC entity's C-RNTI, CS-RNTI, TPC-PUCCH-RNTI, TPC-PUSCH-RNTI, and TPC-SRS-RNTI. When using DRX operation, the MAC entity shall also monitor PDCCH according to requirements found in other subclauses of this specification. When in RRC_CONNECTED, if DRX is configured, the MAC entity may monitor the PDCCH discontinuously using the DRX operation specified in this subclause; otherwise the MAC entity shall monitor the PDCCH continuously.

RRC controls DRX operation by configuring the following parameters:

-	drx-onDurationTimer: the duration at the beginning of a DRX Cycle;

-	drx-SlotOffset: the delay in slots before starting the drx-onDurationTimer;

-	drx-StartOffset: the subframe where the DRX Cycle starts;

-	drx-InactivityTimer: the duration after the PDCCH occasion in which a PDCCH indicates an initial UL or DL user data transmission for the MAC entity;

-	drx-RetransmissionTimerDL (per DL HARQ process): the maximum duration until a DL retransmission is received;

-	drx-RetransmissionTimerUL (per UL HARQ process): the maximum duration until a grant for UL retransmission is received;

-	drx-LongCycle: the Long DRX cycle;

-	drx-ShortCycle (optional): the Short DRX cycle;

-	drx-ShortCycleTimer (optional): the duration the UE shall follow the Short DRX cycle;

-	drx-HARQ-RTT-TimerDL (per DL HARQ process): the minimum duration before a DL assignment for HARQ retransmission is expected by the MAC entity;

-	drx-HARQ-RTT-TimerUL (per UL HARQ process): the minimum duration before a UL HARQ retransmission grant is expected by the MAC entity.

When a DRX cycle is configured, the Active Time includes the time while:

-	drx-onDurationTimer or drx-InactivityTimer or drx-RetransmissionTimerDL or drx-RetransmissionTimerUL or ra-ContentionResolutionTimer (as described in subclause 5.1.5) is running; or

-	a Scheduling Request is sent on PUCCH and is pending (as described in subclause 5.4.4); or

-	a PDCCH indicating a new transmission addressed to the C-RNTI of the MAC entity has not been received after successful reception of a Random Access Response for the Random Access Preamble not selected by the MAC entity among the contention-based Random Access Preamble (as described in subclause 5.1.4).

When DRX is configured, the MAC entity shall:

1>	if a MAC PDU is transmitted in a configured uplink grant:

2>	start the drx-HARQ-RTT-TimerUL for the corresponding HARQ process immediately after the first repetition of the corresponding PUSCH transmission;

2>	stop the drx-RetransmissionTimerUL for the corresponding HARQ process.

1>	if a drx-HARQ-RTT-TimerDL expires:

2>	if the data of the corresponding HARQ process was not successfully decoded:

3>	start the drx-RetransmissionTimerDL for the corresponding HARQ process.

1>	if a drx-HARQ-RTT-TimerUL expires:

2>	start the drx-RetransmissionTimerUL for the corresponding HARQ process.

1>	if a DRX Command MAC CE or a Long DRX Command MAC CE is received:

2>	stop drx-onDurationTimer;

2>	stop drx-InactivityTimer.

1>	if drx-InactivityTimer expires or a DRX Command MAC CE is received:

2>	if the Short DRX cycle is configured:

3>	start or restart drx-ShortCycleTimer;

3>	use the Short DRX Cycle.

2>	else:

3>	use the Long DRX cycle.

1>	if drx-ShortCycleTimer expires:

2>	use the Long DRX cycle.

1>	if a Long DRX Command MAC CE is received:

2>	stop drx-ShortCycleTimer;

2>	use the Long DRX cycle.

1>	if the Short DRX Cycle is used, and [(SFN × 10) + subframe number] modulo (drx-ShortCycle) = (drx-StartOffset) modulo (drx-ShortCycle); or

1>	if the Long DRX Cycle is used, and [(SFN × 10) + subframe number] modulo (drx-LongCycle) = drx-StartOffset:

2>	if drx-SlotOffset is configured:

3>	start drx-onDurationTimer after drx-SlotOffset.

2>	else:

3>	start drx-onDurationTimer.

1>	if the MAC entity is in Active Time:

2>	monitor the PDCCH;

2>	if the PDCCH indicates a DL transmission or if a DL assignment has been configured:

3>	start the drx-HARQ-RTT-TimerDL for the corresponding HARQ process at the first symbol after the end of the corresponding PUCCH transmission;

3>	stop the drx-RetransmissionTimerDL for the corresponding HARQ process.

2>	if the PDCCH indicates a UL transmission:

3>	start the drx-HARQ-RTT-TimerUL for the corresponding HARQ process at the first symbol after the end of the first repetition of the corresponding PUSCH transmission;

3>	stop the drx-RetransmissionTimerUL for the corresponding HARQ process.

2>	if the PDCCH indicates a new transmission (DL or UL):

3>	start or restart drx-InactivityTimer.

1>	else (i.e. not part of the Active Time):

2>	not transmit type-0-triggered SRS defined in TS 38.214 [7].

1>	if CQI masking (cqi-Mask) is setup by upper layers:

2>	if drx-onDurationTimer is not running:

3>	not report CSI on PUCCH.

1>	else:

2>	if the MAC entity is not in Active Time:

3>	not report CSI on PUCCH.

Regardless of whether the MAC entity is monitoring PDCCH or not, the MAC entity transmits HARQ feedback and type-1-triggered SRS defined in TS 38.214 [7] when such is expected.

The MAC entity needs not to monitor the PDCCH if it is not a complete PDCCH occasion (e.g. the Active Time starts or expires in the middle of a PDCCH occasion).

NEXT CHANGE

Annex X (informative):
Intended UE behaviour for DRX Timers

When a DRX timer is set to a value of X, and n denotes the slot in which the related event is triggered according to the subclause 5.7, and k denotes the number of slots per millisecond for the given numerology, the intended behaviours of each DRX timer are presented in the Table X-1 below:

Table X-1: Intended UE behaviour for DRX timers

| DRX Timers | Intended UE behaviour ([x, y] means including slot x and y) |
| --- | --- |
| drx-InactivityTimer | The MAC entity monitors PDCCH for X ms during the slots [n+1, n+k*X]. The timer is started at the slot boundary between n and n+1 The MAC entity starts or restarts drxShortCycleTimer, and uses Short DRX Cycle in the slot n+k*X+1, if configured. |
| ra-ContentionResolutionTimer | The MAC entity monitors PDCCH for X ms during the slots [n+1, n+k*X]. |
| drx-RetransmissionTimerDL or drx-RetransmissionTimerUL | The MAC entity monitors PDCCH during X ms. It is started at the at symbol n+1 if the drx-HARQ-RTT-TimerDL or drx-HARQ-RTT-TimerUL expire in symbol n. |
| drx-onDurationTimer | The MAC entity monitors PDCCH for X ms during starting at slot boundary between n-1 and n. |
| drx-ShortCycleTimer | The MAC entity uses the Short DRX Cycle for X ms during the slot [n, n+k*(X-1)]. The MAC entity starts to use the Long DRX Cycle in the slot n+k*X. |
| drx-HARQ-RTT-TimerDL | The MAC entity starts drx-HARQ-RTT-TimerDL at symbol n+1 if the corresponding PUCCH transmission is performed in symbol n. |
| drx-HARQ-RTT-TimerUL | The MAC entity starts drx-HARQ-RTT-TimerUL at symbol n+1 if the corresponding PUSCH transmission is performed in symbol n. |