3GPP TSG-RAN WG2 #101bis	Tdoc R2-1805609

Sanya, China, 16th – 20th April 2018

Agenda Item:	9.18.4

Source:	Ericsson

Title:	Airborne status/height information upon RRC configuration setup/reestablishement

Document for:	Discussion, Decision

# Introduction

In RAN#78, a new WI for enhanced aerial UE support was agreed [1]. The WID includes the following objectives related to mobility and airborne status indication:

- Specify enhancements to support improved mobility performance and interference detection in the following areas [RAN2]: - Enhancements to existing measurement reporting mechanisms such as definition of new events, enhanced triggering conditions, mechanisms to control the amount of measurement reporting. - Enhancements to adjust handover procedure for Aerial UEs based on information such as location information, UE’s airborne status, etc. - Specify enhancements to support indication of UE’s airborne status and indication of the UE’s support of UAV related functions in LTE network e.g. UE radio capability [RAN2].

In this paper, we discuss airborne status indication upon RRC connection setup.

# Discussion

Agreement from RAN2#101

Agreement:

Provide reference altitude information (including threshold) to UAV UE provided by eNB to assist UE to identify its status (i.e., airborne status).

Airborne status indication has been discussed and proposed in several contributions including [2]. The solution discussed here can be applied to either a binary – ground/flying, or non-binary ones, such as ground/hovering/flying or similar.

## Airborne status indication

The airborne status, either a binary or non-binary indicator, can be very useful for assisting the network for the configuration RRC_IDLE and RRC_CONNECTED state.

In previous contributions, it was commented that the new status would require new reporting procedures and triggering conditions.

Mechanisms for reporting an airborne status or height related feedback should be made as simple as possible and should utilize already existing mechanisms and techniques to the largest extent.

### Upon RRC connection setup/reestablishement

When UE is RRC connected mode there may be more than one way to receive UEs airborne status which can be based on configured threshold. However, UE may take off while idle and initiate RRC connection while airborne. Further, UE may loose RRC connection and attempt reestablishement while flying or during take off/landing.

It would be useful for the UE to be able to let eNB know of its airborne status at an early stage, more specifically at the beginning of the setup of the connection. A natural way of doing this is to embed a field in RRCConnectionSetupComplete and RRCConnectioReestablishement, since other fields like mobilityState are already available to be reported. In

Introduce a field to report the flying status in RRCConnectionSetupComplete and RRCConnectioReestablishement.

# Conclusion

In the previous sections we made the following observations:

Observation 1	Mechanisms for reporting an airborne status or height related feedback should be made as simple as possible and should utilize already existing mechanisms and techniques to the largest extent.

Based on the discussion in the previous sections we propose the following:

Proposal 1	Introduce a field to report the flying status in RRCConnectionSetupComplete and RRCConnectioReestablishement.

# References

TR 36.777, Enhanced LTE support for aerial vehicles

R2-1802789, Measurement reporting enhancement, Ericsson, RAN2#101, Athens, Greece, Feb 2018

TS 36.331, Evolved Universal Terrestrial Radio Access (E-UTRA); Radio Resource Control (RRC); Protocol specification