3GPP TSG-RAN WG2 #101Bis	Tdoc R2-1805652

Sanya, P.R. of China, 16th – 20th April 2018

Agenda Item:	11.3

Source:	Ericsson

Title:	IMT-2020 self-evaluation: Reliability in NR

Document for:	Information

# Introduction

The ITU target for reliability in IMT 2020 has been set to 10-5 error within 1ms for a 32B packet [1]. In this paper, we go through the evaluation steps as defined in [1] and relate the results to the NR Rel-15 capabilities.

# Discussion

## System-level simulations

The assumptions for the SLS are given in Table 1, with values that have been aligned with the calibration campaign for ITU self-evaluation.

The total gain (including antenna gain) is given in Figure 1 for UMa models A and B. The resulting SINR at full load (cell utilization 1) is given in Figure 2. The cell-edge (5th percentile) SINR is found to be -2.3 dB (in DL) and -1.6 dB (in UL) for channel model UMa A, and -2.5 dB (DL)  and -1.7 dB (UL) for channel model UMa B.

Table 1. Assumptions for the system-level simulations.

| Configuration | URLLC configuration B |
| --- | --- |
| Path loss model | UMa A/B with SCM (for ZOD) |
| BS antenna | Sector antenna (8 rows) |
| Tilt | 9 degrees down tilt |
| UE antenna | Isotropic |
| Traffic | Full buffer |
| UL power control | Alpha=1, P0=-106dBm |
| UL allocation | 5PRB (10UEs sharing 50PRBs) |

Figure 1. Total gain for URLLC configuration B.

Figure 2. SINR distribution for URLLC configuration B.

The cell-edge SINR for URLLC Conf. B is approximately -2.3 dB (in DL) and -1.6 dB (in UL) for channel model UMa A, and -2.5 dB (DL)  and -1.7 dB (UL) for channel model UMa B.

## Link-level simulations

The assumptions for LLS are given in Table 2. For PDCCH a compact URLLC DCI of 34 bits including CRC is assumed. For the short PUCCH (1os) we have assumed a payload of 1 bit and repetition after frequency hopping.

The resulting BLER as function of SNR for the control channels is shown in Figure 3, and for LDPC data channels in Figure 4.

Table 2. Assumptions for the link-level simulations.

| Channel model | TDL-C with 300ns delay spread |
| --- | --- |
| Carrier | 700MHz, 20MHz BW |
| Antenna setting | 1TX 2RX |
| Speed | 3km/h |
| Channel estimation | Practical |
| PUCCH | 1os Sequence selection short PUCCH repeated after frequency hopping between band edges |
| PDCCH | Polar code. 34b payload incl. CRC. Distributed CCEs |
| Data | LDPC, BG2 |

Figure 3. Sequence selection Short PUCCH and PDCCH BLER as function of SNR.

Figure 4. Data LDPC BLER for QPSK with different code rates as function of SNR.

## Total reliability

With a few exceptions, the discussion here assumes that the retransmissions are uncorrelated, which is reasonable to assume if e.g. retransmissions are done on a different frequency allocation. In the following we will write the success probabilities on the channel level according to Table 1.

Table 1: Success probabilities for calculating total reliability

| Probability | Description |
| --- | --- |
| p0 | SR |
| p1 | PDCCH |
| p2 | PDSCH/PUSCH |
| p3 | PUCCH NACK detection |
| p4 | PUCCH DTX detection |

DL data, HARQ-based

In the DL we can describe the total reliability after N transmission as:

pt=n=1Np1p2,n1-p1p4+p1p31-p2,nn-1

Where p2,n is the probability of a data block correctly received after n transmissions are soft combined.

DL data, automatic repetitions

For the case of automatic repetitions, we can consider two cases, either with one DL control per DL repetition, or only one DL control. Based on the above expression we derive the following expressions:

Single-DCI repetition:
pt=p1(1-1-p2,1N)

for the assumption of independent repetitions. However, it is more reasonable to see the repetitions as parts of one continuous transmission, also taking soft combining into account, giving instead

pt=p1p2,N

Multi-DCI repetition: 
pt=1-(1-p1p2,1)N

for the assumption of independent repetitions. Again, if we don’t see the transmissions as independent, and taking into account that the DL control is not soft combined we can use the expression

pt=n=1Np1,np2,n

Where p1,n is the probability of correctly decoding n DL control transmissions.

UL data, SR-based

With SR-based UL scheduling the total reliability can be described as:

pt=(1-1-p0M)(1-(1-p1p2,n)N)

for a repetition of M times SR, and N times UL grant and data transmissions/retransmissions.

UL data, SPS-based

With SPS-based scheduling instead we remove the SR step (p0 = 1) and the total reliability can be described as:

pt=1-1-p2,11-p1p2,nN-1

Assuming perfect energy detection performance on the PUSCH resource.

UL data, SPS-based repetition

For the case of SPS-based automatic repetitions, p1 = 1, and we have the following expression:

pt=1-1-p2,1N

which only relies on the data channel performance. As for the case of DL repetition we can instead consider the repetitions as part of a longer transmission:

pt=p2,N

### Reliability estimate URLLC conf. B UMa B

Based on the above expressions for DL and UL data, and the link simulations results, we can evaluate the total reliability. Looking at the lower percentiles for URLLC conf. B UMa B we find the channel BLER at the corresponding UL and DL SINR points. We assume AL16 for PDCCH and 1% D2A level on PUCCH. For UL we assume SPS with a configured resource every TTI. For both DL and UL 1-3 transmission attempts (including HARQ retransmissions) are considered. The transmissions are assumed to be uncorrelated and are not soft combined.

Figure 5. Total reliability for DL data with 1-3 HARQ transmissions at lowest percentiles assuming uncorrelated transmissions.

Figure 6. Total reliability for UL data with 1-3 HARQ transmissions at lowest percentiles with SPS-based scheduling assuming uncorrelated transmissions.

With 3 transmissions of a code rate of 1/10 the reliability target of 10-5 error can be met in DL and in UL with SPS.

## Packet size

The ITU requirement states that a packet of 32B should fulfil the latency and reliability target. With QPSK modulation and a code rate of 1/40 to 1/5 and an overhead of one OFDM symbol, the required #PRBs per such transmission is given in Table 1. Here, the TBS is assumed to be exactly 32B, and CRC is not considered.

Table 4. Required #PRBs for 32B packet and 1OFDM symbol OH, at different code rates.

| #PRBs | 14-os TTI | 7-os TTI | 4-os TTI | 2-os TTI |
| --- | --- | --- | --- | --- |
| Code rate 1/5 | 5 | 9 | 18 | 54 |
| Code rate 1/10 | 9 | 18 | 36 | 107 |
| Code rate 1/20 | 17 | 36 | 72 | 214 |
| Code rate 1/40 | 33 | 72 | 143 | 427 |

With code rate 1/10 and a 4-os mini-slot, 36PRBs are required for a 32B packet.

## Total latency

In a companion paper [2] we evaluate the UP latency for a sequence of transmissions. It is found that 2 HARQ transmissions are possible within the latency bound of 1ms for the 30kHz SCS and 4-os mini-slot configuration, see Table 1. With automatic back-to-back repetitions, 4 transmissions are possible with the same configuration, see Table 3. Thus, the ITU reliability requirement of 10-5 error within 1ms can be met.

Table 5. Maximum #transmissions and HARQ retransmissions in FDD within 1ms.

| #TX within 1ms | 15kHz SCS |  |  |  | 30kHz SCS |  |  |  | 120kHz SCS |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 14-os TTI | 7-os TTI | 4-os TTI | 2-os TTI | 14-os TTI | 7-os TTI | 4-os TTI | 2-os TTI | 14-os TTI | 7-os TTI | 4-os TTI | 2-os TTI |
| DL data | 0 | 0 | 1 | 1 | 0 | 1 | 2 | 2 | 2 | 3 | 4 | 5 |
| UL data (SPS) | 0 | 0 | 1 | 1 | 0 | 1 | 2 | 2 | 2 | 3 | 4 | 5 |

Table 6. Maximum #transmissions and automatic repetitions in FDD within 1ms.

| #TX within 1ms | 15kHz SCS |  |  |  | 30kHz SCS |  |  |  | 120kHz SCS |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 14-os TTI | 7-os TTI | 4-os TTI | 2-os TTI | 14-os TTI | 7-os TTI | 4-os TTI | 2-os TTI | 14-os TTI | 7-os TTI | 4-os TTI | 2-os TTI |
| DL data | 0 | 0 | 1 | 3 | 0 | 2 | 4 | 9 | 5 | 12 | 22 | 46 |
| UL data (SPS) | 0 | 0 | 1 | 3 | 0 | 2 | 4 | 9 | 5 | 12 | 22 | 46 |

With 30kHz SCS and 4-os mini-slot 4 transmissions can be made in FDD within 1ms.

The ITU target on Reliability can be met with NR.

# Conclusion

In section 2 we made the following observations:

- The cell-edge SINR for URLLC Conf. B is approximately -2.3 dB (in DL) and -1.6 dB (in UL) for channel model UMa A, and -2.5 dB (DL) and -1.7 dB (UL) for channel model UMa B.

With 3 transmissions of a code rate of 1/10 the reliability target of 10-5 error can be met in DL and in UL with SPS.

With code rate 1/10 and a 4-os mini-slot, 36PRBs are required for a 32B packet.

With 30kHz SCS and 4-os mini-slot 4 transmissions can be made in FDD within 1ms.

The ITU target on Reliability can be met with NR.

The observations above can be used as a starting point for discussion on reliability in the self-evaluation study item.

# References

ITU-R M.[IMT-2020-TECH PERF REQ] (WP5D-TD-0300)

R2-1805654, IMT2020 Self-evaluation UP Latency in NR