3GPP TSG-RAN WG2 Meeting #101bis	R2-1805689

Sanya, China, April 16 – April 20 2018

Title:	(Draft)LS on UE capability ID based capability transfer

Release:	Rel-15

Work Item:	NR_newRAT-Core

Source:	Intel Corporation

To:	SA2

Contact Person:

#### Name:	Naveen Palle

Tel. Number:	+1 224 715 6778

E-mail Address:	naveen [DOT] palle [AT] intel [DOT] com

# 1	UE Capability ID requirements

RAN2 would like to inform SA2 that RAN2 has agreed to use a new ‘Capability ID’ to retrieve the UE NR SA access stratum capabilities from the 5GC thereby avoiding the need to have the NR SA UE providing these using the air interface.

RAN2 has agreed to the following on the usage and requirements of the Capability ID:

- The ‘Capability ID’ is globally unique and maps to the set of UE NR SA access stratum capabilities stored in 5GC.

- During registration procedure, the UE sends the Capability ID to the NG-RAN. NG-RAN provides Capability ID to 5GC. 5GC retrieves the corresponding UE Access Stratum capabilities and send them back to NG-RAN. How 5GC retrieves UE Access Stratum capabilities from capability ID is left for SA2 design.

- In case the 5GC cannot provide UE AS capabilities based on the Capability ID provided, NG-RAN can request the UE to provide the same using legacy means.

- A Capability ID is only associated with no more than one set of UE AS capabilities.

- RAN2 has not discussed format of Capability ID and has left that to SA2.

# 2	Actions

ACTION to SA2:

RAN2 would like to request SA2 to consider the above information into account and provide system level design considering the below:

- Inputs on the requirements of the definition of “Capability ID”

- Inputs on the requirements of how the capability ID and the corresponding UE capabilities for that ID are updated by the UE manufacturer or operator directly with the 5GC at the UE production time and during SW updates.

- Inputs on what is needed in 5GC to retrieve the “UE radio capability information” based on the “Capability ID” provided by RAN and provides UE radio capability to RAN over N2.

# 3	Date of Next RAN2 Meetings:

RAN2#101bis	16 April - 20 April	Sanya, China

RAN2#102		21 May – 25 May  Busan, South Korea