3GPP TSG-RAN WG2 NR #101b Meeting                                            	R2-1805849

Sanya, China, 16th – 20th April 2018	(Update of R2-1802466)

Agenda item:	10.4.1.4.1

Source:	Samsung

Title:	RRM measurement configuration for BWP

Document for:	Discussion & Decision

# 1. Introduction

RAN#75 in March approved a 5G WID [1] on New Radio access technology, which targets a unified framework for traffic with diverse QoS requirements e.g. enhanced mobile broadband (eMBB), massive machine-type-communications (mMTC), ultra-reliable and low latency communications (URLLC).

During NR SI/WI phases for Rel-15, the following agreements were achieved for NR RRM measurement.

RAN2 Agreements (#100)

Agreements

1	UE determines which MO corresponds to the serving cell frequency from the frequency location of the CD-SSB that is contained within the serving cell configuration.

2	More than one MO with CSI-RS resources for measurement can be associated to the same SSB location in frequency. The SSB is at least used for timing reference.

3	In case that more than one MO with CSI-RS resources for measurement is associated to the same SSB location in frequency the UE is indicated which MO corresponds to the serving carrier.

FFS whether the indication is in MO or serving cell configuration

This contribution investigates design considerations and framework to support wider BW in NR; especially about flexible BW support based on bandwidth part.

# 2. 	RRM Measurement for Bandwidth Part

  - General understanding about bandwidth part

BWP is configured for RRC connected mode UE. The motivation of BWP configuration is to support wider BW compared to maximum BW capability of UE. The configured BWP is regarded as the operating BW of the UE at the instance. Network can configure multiple different BWPs to RRC connected UEs to utilize wider BW efficiently. From UE side, at least one BWP should be configured and multiple BWPs can be configured by network decision based on UE BW capability, and measurement result. Multiple BWPs configuration may be useful for BW adaptation to save UE power consumption or for flexible scheduling mechanism in single CC. The parameters including numerology, location of BWP, BW of BWP, CORESET (Control Resource Set) configuration, DL/UL channels have an association to a BWP.

Figure 1: The concept of bandwidth part

Observation 1: A BWP configuration is per UE and contains BW information (location, bandwidth), numerology, channels, and so on.

Even though different numerology can be configured per BWP, we will assume that UE monitors cell-specific RSs configured in a MO with only single numerology. It is beneficial to avoid diverge discussions to consider complicated problem due to numerology mismatch between network and UE. Since CSI-RS resource is cell-specific, we cannot say that BW of BWP and BW of CSI-RS are always aligned. However it is beneficial to configure UE by the similar attributes (i.e. numerology, BW information) of BWP, to measure CSI-RS resources.

Observation 2: For UE not supporting mixed numerology, we can assume that all CSI-RSs associated to a cell in a measurement object are configured in single numerology.

According to the current 38.331, the RRC structure especially on measurement object supports the assumption as shown in the following MeasObjectNR IE:

MeasObjectNR information element

-- ASN1START

-- TAG-MEAS-OBJECT-NR-START

MeasObjectNR ::=							SEQUENCE {

ssbFrequency								ARFCN-ValueNR															OPTIONAL,

refFreqCSI-RS								ARFCN-ValueNR															OPTIONAL,

--RS configuration (e.g. SMTC window, CSI-RS resource, etc.)

referenceSignalConfig						ReferenceSignalConfig

[…]

}

ReferenceSignalConfig::=     			SEQUENCE {

-- SSB configuration for mobility (nominal SSBs, timing configuration)

ssb-ConfigMobility						SSB-ConfigMobility				OPTIONAL,	-- Need M

-- CSI-RS resources to be used for CSI-RS based RRM measurements

csi-rs-ResourceConfigMobility			SetupRelease { CSI-RS-ResourceConfigMobility } OPTIONAL-- Need M

}

[…]

CSI-RS-ResourceConfigMobility ::= 		SEQUENCE {

-- MO specific values

isServingCellMO						BOOLEAN,

-- Subcarrier spacing of CSI-RS.

-- Only the values 15, 30 or 60 kHz  (<6GHz), 60 or 120 kHz (>6GHz) are applicable.

-- Corresponds to L1 parameter 'Numerology' (see 38.211, section FFS_Section)

subcarrierSpacing						SubcarrierSpacing,

-- List of cells

csi-RS-CellList-Mobility 	SEQUENCE (SIZE (1..maxNrofCSI-RS-CellsRRM))	OF CSI-RS-CellMobility

}

CSI-RS-CellMobility ::=				SEQUENCE {

cellId									PhysCellId,

csi-rs-MeasurementBW					SEQUENCE {

-- Allowed size of the measurement BW in PRBs

-- Corresponds to L1 parameter 'CSI-RS-measurementBW-size' (see FFS_Spec, section FFS_Section)

nrofPRBs			ENUMERATED { size24, size48, size96, size192, size264},

-- Starting PRB index of the measurement bandwidth

-- Corresponds to L1 parameter 'CSI-RS-measurement-BW-start' (see FFS_Spec, section FFS_Section)

-- FFS_Value: Upper edge of value range unclear in RAN1

startPRB			INTEGER(0..2169)

},

-- Frequency domain density for the 1-port CSI-RS for L3 mobility

-- Corresponds to L1 parameter 'Density' (see FFS_Spec, section FFS_Section)

density									ENUMERATED {d1,d3}												OPTIONAL,

-- List of resources

csi-rs-ResourceList-Mobility 	SEQUENCE (SIZE (1..maxNrofCSI-RS-ResourcesRRM))	OF CSI-RS-Resource-Mobility

}

[…]

We can see that single numerology and cell-specific CSI-RS resource configuration is configured in the CSI-RS resource configuration. The cell-specific CSI-RS resource configuration may contain one or more cells with the association to the single measurement BW per each cell.

Observation 3: The current RRC specification can support that network configure UE to measure a measurement object comprising single numerology and at least one cell information (including cell ID and a measurement BW).

  - Intra-cell RRM measurement for BWP (re-)configuration

It may be better to think about use-cases of BWP reconfiguration before discussing about intra-cell RRM measurement. From RAN1 agreement on DCI for BWP activation/deactivation, we assume that BWP reconfiguration will occur in longer time scale than that of BWP switching. BWP switching is basically scheduling operation. By the way, there has been a debate on whether L1-based channel measurement/feedback is sufficient for BWP reconfiguration. From the RAN2 perspective, depending only instant L1 feedback is vulnerable to dynamic channel fluctuation and may incur additional implementation complexity at network side. In addition, L1-based measurement configuration may need two different types i.e. BWP-specific and cell common. BWP-specific configuration means that CSI-RS configuration is associated to the configured BWPs. However BWP-specific configuration is not possible to frequency part where BWP is not yet configured. So cell common CSI-RS configuration is also needed to indicate measurement to UE outside of configured BWPs. It is good to have a common measurement framework for both cases of within and out of configured BWPs. Accordingly RAN2 agreed on a RRM measurement BW configuration to be utilized for either BWP-specific or cell common RRM configuration.

Within configured RRM measurement BW, UE needs to monitor RRM RS in a frequency part of the serving carrier. RRM RS may be either NR-SS or CSI-RS which locates in the current monitoring part of UE. During Connected state, the monitoring part of UE depends on current active BWP of the UE. Note that UE does not need to monitor any DL signal outside of active BWP, according to RAN1 agreement. And, exceptionally, UE can monitor DL signal outside of active BWP through measurement gap.

By the way, the type of RRM RS – NR-SS or CSI-RS may affect to the way of measurement configuration. If only NR-SS needs to be configured for a UE as RRM RS, UE can be configured with measurement object including an indication of the NR-SS with ARFCN. If only CSI-RS needs to be configured for a UE as RRM RS, UE may be configured with measurement object including an indication of a set of CSI-RS and RRM measurement BW to confine the list of CSI-RS. At least a RRM BW is useful when UE decides monitoring BW and numerology. Parameters (i.e. subcarrier spacing, and BW) to define a RRM BW is configured in a measurement object since the RRM BW is a frequency part of the carrier which is mapped to the measurement object. The location and bandwidth of RRM BW is configured in PRB unit explicitly in CSI-RS resource configuration in MO.

Figure 2: Multiple measurement object configuration for single carrier

When a measurement object contains information of just a RRM BW, triggering events based on absolute threshold such as A1 event can be supported. In addition, another triggering events based on comparison can be also supported. If network intends that UE measures different frequency parts of the same carrier, it can configure multiple MOs associated to the carrier with different parameters per MO. By multiple MO configurations, UE can measure different RRM BW of each MO even in the same carrier. ‘isServingCellMO’ field may indicate whether this MO is associated to the same serving carrier. However, according to current agreements, it is only limited to the case that more than one MO with CSI-RS resources for measurement is associated to “the same” SSB location in frequency. This may restrict the freedom of network configuration.

Observation 4: To support intra-cell RRM measurement, it is beneficial that multiple MOs with different RRM BW are configured to the same carrier.

Observation 5: ‘isServingCellMO’ indicates that multiple MOs are associated to the same SSB, so far.

Proposal 1: ‘isServingCellMO’ field indicates whether the MO is associated to the same serving carrier (i.e. not associated to the same SSB).

  - Inter-cell RRM measurement considering BWP

To achieve optimal performance for handover, monitoring full bandwidth at UE is beneficial to compensate the radio aspects of frequency selectivity or shadowing. However monitoring full bandwidth is critical for UE’s battery lifetime. Even though UE monitors in a narrow scheduling BW in serving cell, whole power consumption may be high due to monitoring wider bandwidth and higher power drain for RRM measurement to neighboring cells. Therefore it will be beneficial for UE to monitor only a RRM BW which is narrower than full bandwidth of the neighbor cell. If NR-SS is used for measurement, RRM BW is not needed explicitly. Otherwise, if CSI-RS is used, UE may need the constrained RRM BW configuration in the measurement object.

Observation 6: Narrow bandwidth for inter-cell RRM measurement is essential for UE power saving.

Proposal 2: Among multiple MOs within the serving carrier, UE monitors only the BWs which are configured in the serving cell MOs (i.e. indicated by ‘isServingCellMO’ field)

# 3. Summary

Based on the discussion and identified observations in this contribution, we propose followings:

Observation 1: A BWP configuration is per UE and contains BW information (location, bandwidth), numerology, channels, and so on.

Observation 2: For UE not supporting mixed numerology, we can assume that all CSI-RSs associated to a cell in a measurement object are configured in single numerology.

Observation 3: The current RRC specification can support that network configure UE to measure a measurement object comprising single numerology and at least one cell information (including cell ID and a measurement BW).

Observation 4: To support intra-cell RRM measurement, it is beneficial that multiple MOs with different RRM BW are configured to the same carrier.

Observation 5: ‘isServingCellMO’ indicates that multiple MOs are associated to the same SSB, so far.

Observation 6: Narrow bandwidth for inter-cell RRM measurement is essential for UE power saving.

Proposal 1: ‘isServingCellMO’ field indicates whether the MO is associated to the same serving carrier (i.e. not associated to the same SSB).

Proposal 2: Among multiple MOs within the serving carrier, UE monitors only the BWs which are configured in the serving cell MOs (i.e. indicated by ‘isServingCellMO’ field)

References

[1] RP-170823, New WID Proposal: Work Item on New Radio (NR) Access Technology