3GPP TSG-RAN WG2 Meeting #101bis	R2-1806010

# Sanya, China, 16th – 20th April 2018

| CR-Form-v11.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.321 | CR | 0096 | rev | - | Current version: | 15.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction on BFR RA procedure |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | LG Electronics Inc |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R2 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT-Core |  |  |  |  |  |  | Date: |  | 2018-04-06 |
| Category: | F |  |  |  |  |  |  | Release: |  | Rel-F |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | - For BFR procedure, the selection of SSB and CSI-RS is based on candidateBeamThreshold and Qin,LR threshold for CSI-RS according to subclause 6 in TS 38.213 [13], not rsrp-ThresholdSSB and csirs-Threshold. - In R2-1800003, RAN1 informs RAN2 of Beam related agreements. Among them, the following has not be captured: If the Candidate-Beam-RS-List includes both CSI-RS resource indexes and SSB indexes, AND only SSB indexes are associated with PRACH resources, - UE identifies PRACH resources for CSI-RS resource(s) in the Candidate-Beam-RS-List via spatial QCL indication between SSBs and CSI-RS resources, if UE-identified new beam(s) is associated with CSI-RS resource(s) - UE sends BFRQ through a PRACH resource associated with the SSB, which is spatially QCLed with the CSI-RS resource. - Note: in case the Candidate-Beam-RS-List includes both CSI-RS resource indexes and SSB indexes, AND only SSB indexes are associated with PRACH resources, a UE is not expected to be configured by Candidate-Beam-RS-List a CSI-RS resource which does not have a spatial QCL association with any of the SSB in the same Candidate-Beam-RS-List. It means that if PRACH resource associated with CSI-RS has not been provided, the UE shall select the PRACH resource associated with SSB which is linked to the selected CSI-RS, i.e., spatially quasi-colocated SSB. However, the current MAC specification specifies by assuming that there is always a ra-PreambleIndex associated with CSI-RS. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - the threshold parameters are corrected for SSB and CSI-RS selection in BFR RA procedure. - When MAC entity selects a preamble for BFR procedure, the MAC shall select a preamble from SSB which is spatially quasi-colocated with CSI-RS. Impact analysis Impacted functionality: The changes only impact RA procedure triggered for BFR procedure. Inter-operability: 1. if the network is implemented according to the CR and the UE is not, the UE may not be able to transmit a RAP for BFR procedure if there is no preamble allocated for the selected CSI-RS. 2. If the UE is implemented according to the CR and the network is not, the UE may not properly select the SSB/CSI-RS by refering to the wrong value, resulting in BFR performance degradation. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | RA for BFR procedure does not operate properly. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.1.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | x | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

5.1.2	Random Access Resource selection

The MAC entity shall:

1>	if the Random Access procedure was initiated for beam failure recovery (as specified in subclause 5.17); and

1>	if the contention-free Random Access Resources for beam failure recovery request associated with any of the SSBs and/or CSI-RSs have been explicitly provided by RRC; and

1>	if at least one of the SSBs with SS-RSRP above  candidateBeamThreshold amongst the associated SSBs or the CSI-RSs with CSI-RSRP above Qin,LR threshold for CSI-RS according to subclause 6 in TS 38.213 [7] amongst the associated CSI-RSs is available:

2>	select an SSB with SS-RSRP above candidateBeamThreshold amongst the associated SSBs or a CSI-RS with CSI-RSRP above Qin,LR threshold for CSI-RS according to subclause 6 in TS 38.213 [7] amongst the associated CSI-RSs;

2>	if CSI-RS is selected and there is no ra-PreambleIndex associated with the selected CSI-RS:

3>	set the PREAMBLE_INDEX to a ra-PreambleIndex corresponding to the SSB which is quasi collocated with repsect to ‘QCL-TypeD’ according to sublause 5.1.5 of TS 38.214 [7]

2>	else:

3>	set the PREAMBLE_INDEX to a ra-PreambleIndex corresponding to the selected SSB or CSI-RS from the set of Random Access Preambles for beam failure recovery request.

1>	else if the ra-PreambleIndex has been explicitly provided by either PDCCH or RRC; and

1>	if the ra-PreambleIndex is not 0b000000; and

1>	if contention-free Random Access Resource associated with SSBs or CSI-RS have not been explicitly provided by RRC:

2>	set the PREAMBLE_INDEX to the signalled ra-PreambleIndex.

1>	else if the contention-free Random Access Resources associated with SSBs have been explicitly provided by RRC and at least one SSB with SS-RSRP above rsrp-ThresholdSSB amongst the associated SSBs is available:

2>	select an SSB with SS-RSRP above rsrp-ThresholdSSB amongst the associated SSBs;

2>	set the PREAMBLE_INDEX to a ra-PreambleIndex corresponding to the selected SSB.

1>	else if the contention-free Random Access Resources associated with CSI-RSs have been explicitly provided by RRC and at least one CSI-RS with CSI-RSRP above csirs-Threshold amongst the associated CSI-RSs is available:

2>	select a CSI-RS with CSI-RSRP above csirs-Threshold amongst the associated CSI-RSs;

2>	set the PREAMBLE_INDEX to a ra-PreambleIndex corresponding to the selected CSI-RS.

1>	else:

2>	if at least one of the SSBs with SS-RSRP above rsrp-ThresholdSSB is available:

3>	select an SSB with SS-RSRP above rsrp-ThresholdSSB.

2>	else:

3>	select any SSB.

2>	if Msg3 has not yet been transmitted:

3>	if Random Access Preambles group B exists; and

3>	if the potential Msg3 size (UL data available for transmission plus MAC header and, where required, MAC CEs) is greater than ra-Msg3SizeGroupA and the pathloss is less than PCMAX (of the Serving Cell performing the Random Access Procedure) – preambleReceivedTargetPower – deltaPreambleMsg3 – messagePowerOffsetGroupB:

4>	select the Random Access Preambles group B.

3>	else:

4>	select the Random Access Preambles group A.

2>	else (i.e. Msg3 is being retransmitted):

3>	select the same group of Random Access Preambles as was used for the Random Access Preamble transmission attempt corresponding to the first transmission of Msg3.

2>	if the association between Random Access Preambles and SSBs is configured:

3>	select a ra-PreambleIndex randomly with equal probability from the Random Access Preambles associated with the selected SSB and the selected Random Access Preambles group.

2>	else:

3>	select a ra-PreambleIndex randomly with equal probability from the Random Access Preambles within the selected Random Access Preambles group.

2>	set the PREAMBLE_INDEX to the selected ra-PreambleIndex.

1>	if an SSB is selected above and an association between PRACH occasions and SSBs is configured:

2>	determine the next available PRACH occasion from the PRACH occasions corresponding to the selected SSB permitted by the restrictions given by the ra-ssb-OccasionMaskIndex if configured (the MAC entity may take into account the possible occurrence of measurement gaps when determining the next available PRACH occasion corresponding to the selected SSB).

1>	else if a CSI-RS is selected above and an association between PRACH occasions and CSI-RSs is configured:

2>	determine the next available PRACH occasion from the PRACH occasions in ra-OccasionList corresponding to the selected CSI-RS (the MAC entity may take into account the possible occurrence of measurement gaps when determining the next available PRACH occasion corresponding to the selected CSI-RS).

1>	else:

2>	determine the next available PRACH occasion (the MAC entity may take into account the possible occurrence of measurement gaps when determining the next available PRACH occasion).

1>	perform the Random Access Preamble transmission procedure (see subclause 5.1.3).