3GPP TSG-RAN WG2 Meeting #101bis 	R2-1806077

# Sanya, China, 16th - 20th April 2018

| CR-Form-v11.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.322 | CR | 0007 | rev | - | Current version: | 15.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Clarification on data volume calculation |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | LG Electronics Inc. |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R2 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT-Core |  |  |  |  |  |  | Date: |  | 2018-04-05 |
| Category: | F |  |  |  |  |  |  | Release: |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | According to TS 38.323, when a PDCP entity is associated with two RLC entity, the term “RLC data volume pending for initial transmission” is used for uplink routing and MAC buffer status reporting functions for UL split bearers. The original intention of “RLC data volume pending for initial transmission” is to include all RLC SDUs and RLC data PDUs pending for initial transmission. However, according to the current wording of TS 38.322, if only pending for initial tansmission is applied to calculate RLC data volume, only RLC data PDUs may be considered and RLC SDUs may not be calculated. Furthermore, even if RLC SDUs have not yet been included in an RLC data PDU, those RLC SDUs are also pending for initial transmission at the RLC entity. This confusion should be removed in the current TS 38.322. |  |  |  |  |  |  |  |  |
| Summary of change: |  | In Data volume calculation (5.5), replace the wording “have not yet been included in an RLC data PDU” with “are pending for initial transmission” to avoid confusion. Impact analysis Impacted functionality: Data volume calculation operation Inter-operability: If the network is implemented according to this CR while the UE is not, the UE may route UL data based on the wrong data volume information and report BSR with wrong BS information to the network. If the UE is implemented according to this CR while the network is not, the network may route DL data based on the wrong data volume information. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The UE may route UL data based on the wrong data volume information and report BSR with wrong BS information to the network. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.5 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  | TS/TR ... CR ... |  |  |  |
| affected: |  |  |  | Test specifications |  |  | TS/TR ... CR ... |  |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  | TS/TR ... CR ... |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

Start of the change

## 5.5	Data volume calculation

For the purpose of MAC buffer status reporting, the UE shall consider the following as RLC data volume:

-	RLC SDUs and RLC SDU segments that are pending for initial transmission;

-	RLC data PDUs that are pending for initial transmission;

-	RLC data PDUs that are pending for retransmission (RLC AM).

In addition, if a STATUS PDU has been triggered and t-StatusProhibit is not running or has expired, the UE shall estimate the size of the STATUS PDU that will be transmitted in the next transmission opportunity, and consider this as part of RLC data volume.

Start of the change