3GPP TSG-RAN WG2 Meeting #101bis 	R2- 1806151

# Sanya, China, 16th - 20th April 2018

| CR-Form-v11.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.321 | CR | 0098 | rev | - | Current version: | 15.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | RLC failure handling |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | LG Electronics Inc. |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R2 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT-Core |  |  |  |  |  |  | Date: |  | 2018-04-05 |
| Category: | F |  |  |  |  |  |  | Release: |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | According to above agreements “No further optimization are considered, the UE continues normal operation”, if the AM RLC entity indicates the max. number of retransmission to the RRC layer, the RRC layer reports the max. number of retransmission to the gNB by e.g. SCell-RLF procedure, without initiating the RRC connection re-establishment procedure. After that, the UE continues normal operation. The meaning of ‘the UE continues normal operation’ is that the AM RLC entity which has indicated the max. number of retransmission keeps the transmission of the RLC SDUs. Note that the gNB cannot differentiate between logical channels belonging to an LCG. Thus, it is difficult for the gNB not to provide UL grant for a problematic logical channel if there are multiple logical channels within an LCG. Moreover, if the problematic logical channel has higher logical channel priority, all the UL grants are allocated to the problematic logical channel. Thus, the RLC entity reaching the maximum number of retransmssion should be ignored at LCP procedure. |  |  |  |  |  |  |  |  |
| Summary of change: |  | In section 5.4.3.1.2, the MAC entity ignore the logical channel which is associated RLC entity is reaching the maximum number of retransmission. Impact analysis Impacted functionality: Selection of logical channels Inter-operability: If the network is implemented according to this CR while the UE is not, the UE may allocate UL grant to the logical channel which is associated RLC entity is reaching the maximum number of retransmission. If the UE is implemented according to this CR while the network is not, no interoperability issue is foreseen. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The UE may allocate UL grant which is associated RLC entity is reaching the maximum number of retransmission. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.4.3.1.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  | TS/TR ... CR ... |  |  |  |
| affected: |  |  |  | Test specifications |  |  | TS/TR ... CR ... |  |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  | TS/TR ... CR ... |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

Start of the change

##### 5.4.3.1.2	Selection of logical channels

The MAC entity shall, when a new transmission is performed:

1>	select the logical channels for each UL grant that satisfy all the following conditions:

2>	the set of allowed Subcarrier Spacing index values in allowedSCS-List, if configured, includes the Subcarrier Spacing index associated to the UL grant; and

2>	maxPUSCH-Duration, if configured, is larger than or equal to the PUSCH transmission duration associated to the UL grant; and

2>	configuredGrantType1Allowed, if configured, is set to TRUE in case the UL grant is a Configured Grant Type 1; and

2>	allowedServingCells, if configured, includes the Cell information associated to the UL grant; and

2> associated RLC entity is not reaching maximum number of retransmission.

NOTE:	The Subcarrier Spacing index, PUSCH transmission duration and Cell information are included in Uplink transmission information received from lower layers for the corresponding scheduled uplink transmission.

Start of the change