3GPP TSG-RAN WG2 #101Bis	R2-1806153

Sanya, P. R. of China, 16th – 20th April 2018                                (Update of R2-1803330)

Agenda Item:	10.4.1.4.6

Source:	Ericsson

Title:	ANR framework for EN-DC and NR standalone

Document for:	Discussion, Decision

# Introduction

Since the first release of LTE, Automatic Neighbour Relation (ANR) functionality has been introduced to automatically generate the relations between eNodeBs. Such relations are used to establish connections between eNodeBs, support mobility, load balancing, dual connectivity, etc., and ANR is seen as significantly reducing the planning and operation costs for operators [1]. Therefore, it is our understanding that ANR will continue to be an important feature in NR. One can expect that the challenging NR requirements will result in an even higher emphasis on the need for seamless mobility than in legacy RATs, which further increases the importance of automatic relation establishment in NR. In addition, an extensive use of beamforming, together with a lean design of NR will bring new challenges and additional scope to the relation management.

- Detection of neighbor cell for measurement is based on NR-SS

In RAN1#88, it was agreed that;

In this paper, further details of how the UE could aid the neighbour relation establishment process is provided and scenarios wherein the UE fails to identify a neighbouring cell is provided.

# Discussion

## ANR framework in LTE

In LTE, a UE detects cells based on their PSS/SSS, which encode a Physical Cell Identifier (PCI). An important property of these signals is that the UE autonomously detects the neighbour cell IDs from an acquired PSS/SSS, i.e., the network does not need to provide a neighbour cell list. UEs typically detect and measure neighbour cells by sampling a short time window (e.g. 5 ms) on the target frequency (which may be the same or different from serving) and search (possibly offline) for PSS/SSS occurrences within that sample. For each detected PSS/SSS, the UE can also perform a measurement using the CRS corresponding to the PCI. The result of that action is a list of neighbour cell IDs and corresponding measurement sample.

Once the UE performs measurements, mobility events can trigger the transmission of measurement reports. Based on these reports the serving eNB is able to identify that a given PCI belongs to a neighbour eNB in order to trigger a handover preparation procedure. In order to do that, the serving eNB needs to maintain a Neighbour Relation Table that maps locally unique identifiers, i.e. the PCIs in LTE, with globally unique identifiers, i.e. the E-UTRAN Cell Global identifier (E-CGI). E-CGI is defined in LTE as a combination of PLMN ID (Combination of country code and network code) and ECI (combination of eNB ID and cell ID).

The procedure is illustrated in Figure 1. In step 1, the UE sends a measurement report to the serving eNB, containing the PCI of cell B. The eNB then instructs the UE to read the ECGI, the TAC and all available PLMN ID(s) of cell B. The neighbour cell broadcasts its PLMN IDs, TAC etc. in the SIB1 message. After measuring these values, the UE reports the detected ECGI to the serving eNB, and the eNB decides to add this neighbour relation. The eNB can use the PCI and ECGI to lookup a transport layer address to the new eNB, update the Neighbour Relation list, and, if needed, setup a new X2 interface towards this eNB [2].

Figure 1. Automatic Neighbour Relation Function in LTE [2]

- In LTE, the UE reports to the serving eNB measurement reports associated to locally unique identifiers (e.g. PCIs).

- For handover decisions based on UE reporting of local IDs, a Neighbour Relation Table (NRT) mapping local IDs to global IDs (e.g. E-CGI) is necessary to identify the target eNB in order to send a handover preparation and/or a handover request.

- In LTE, a serving eNB can request the UE to acquire a global identifier (e.g. ECGI), TAC, and all available PLMN ID(s) associated to a previously reported local identifier (PCI).

- In LTE, a globally unique identifier (ECGI) is broadcasted in SIB1 and can be associated to a locally unique identifier (PCI).

In LTE, this Global ID, the ECGI, is not necessary as often as the PCI. Therefore, it is transmitted in SIB1 every 80 ms while the PCI is transmitted every 5ms.

- In LTE, a globally unique identifier (ECGI) is broadcasted with a larger periodicity than the locally unique identifier (PCI).

## ANR framework in NR

In RAN2#97 NR meeting in Athens, it was noted that an RRC_CONNECTED UE should be able to perform RRM measurements on the so-called IDLE RS, later renamed to an NR-SS Block.

An RRC_CONNECTED UE should be able to perform RRM measurements on always on IDLE RS (e.g. synchronization signal).

The measurement reports based on SS Blocks including cell measurement results will be associated to local identifiers (like the LTE PCIs) that the UE has measured.

- The RRM measurements on SS Blocks are reported as per the measured PCI values by the UE.

Based on such a measurement report, if the serving gNodeB does not recognize the reported PCI in its neighbour relation list, then the serving gNodeB could reuse the globally unique identifier, TAC and PLMN ID(s) measurement and reporting procedure for the UE, as listed in LTE, for NR neighbour relation establishment also.

It is worth noting that the information required to derive the globally unique identifier (GID) are provided in NR as part of the SIB1 message as well. It can be discussed further as to how is the GID is structured (number of bits, what sub-identifiers does it encompass in what position like PLMN ID, CID etc.) in NR and if it is of similar nature as ECGI of LTE or not.

- The information elements required to derive the Global Identifier, GID are provided in NR as expected to be part of the SIB1 message.

Based on this, we propose the following:

To support ANR across NR cells, as in LTE, NG-RAN can configure the UE to read and report NR neighboring cell’s Global identifier(s) (GID), TAC(s) and all available PLMN ID(s) to aid neighbor relation establishment procedure. FFS structure of GID is same as the structure of ECGI in LTE.

Moreover, in RAN WG2 #98-Ad Hoc meeting, it was agreed that the LTE inter-RAT ANR framework will be reused to cover NR.

Agreements

1	LTE inter-RAT ANR framework is extended to cover NR (target to complete this at end of Rel-15, not December 2017)

In our view, the previous discussion is applicable to both EN-DC case, where E-UTRAN configures the UE to report information broadcasted by the NR cell whose PCI is unknown to the serving gNodeB (either for SCG addition or SCG change) and to NR Standalone case, where the NG-RAN configures the UE. Therefore, the following is proposed:

To support LTE IRAT ANR procedure for NR cells, including EN-DC, E-UTRAN can configure the UE to read and report neighboring NR cell’s Global identifier(s) (GID), TAC(s) and all available PLMN ID(s) to aid neighbor relation establishment.

## CGI reporting

In R2-1710839 (Summary of email discussion [99#32][NR] TP on RRM), companies expressed that measurement configuration in NR should be used for CGI reporting, although changes in the LTE ASN.1 structure are foreseen. Hence, the following is proposed:

E-UTRAN measurement configuration (defined in 36.331) should be used to support CGI reporting.

NG-RAN measurement configuration (defined in 38.331) should be used to support CGI reporting.

In RAN2#101, it was further agreed that for LTE reportCGI the UE acquires the cell ID from SIB1 of the NR cell. However, it is still FFS what the UE should perform in case the NR cell does not broadcast SIB1.

Agreements

1:	For LTE reportCGI of an NR cell the UE acquires the cell ID from SIB1 of the NR cell (for post EN-DC early freeze)

FFS Whether the UE reports anything to indicate that the NSA cell was not broadcasting SIB1.

2:	Introduce a capability bit in LTE for the capability of reportCGI towards NR neighbours (no differentiation between SA/NSA case).

FFS on differentiation between FR1 and FR2.

In our view, no further action from the UE is required in case it does not find the CGI of the NR cell as this can be inferred by the lack of UE report. That is, if the NW does not receive the UE report after it configures the UE to report CGI of an unknown NR cell, it knows that the UE is not able to acquire the cell ID. This inference can be rather fast as in the case that the NR cell does not broadcast SIB1, then more UEs will end up in the same situation, and hence do not send any report back to the serving node.

Moreover, in RAN2#101, it was also agreed to introduce a capability bit in LTE to indicate whether the UE is able to perform CGI reporting towards the NR cell. While we believe that such a CGI reporting should be mandated (regardless of SA or NSA) as it is needed for inter-RAT ANR, we understand that it is not possible for the time being due to lack of time for the completion of the specs. However, we do not foresee any use case for making this frequency specific. Hence, we propose to not differentiate between FR1 and FR2 on the LTE reportCGI capability bit:

There is no need to differentiate between FR1 and FR2 on the LTE reportCGI capability bit.

# Conclusion

In the previous section, the following has been observed:

- In LTE, the UE reports to the serving eNB measurement reports associated to locally unique identifiers (e.g. PCIs).

- For handover decisions based on UE reporting of local IDs, a Neighbour Relation Table (NRT) mapping local IDs to global IDs (e.g. E-CGI) is necessary to identify the target eNB in order to send a handover preparation and/or a handover request.

- In LTE, a serving eNB can request the UE to acquire a global identifier (e.g. ECGI), TAC, and all available PLMN ID(s) associated to a previously reported local identifier (PCI).

- In LTE, a globally unique identifier (ECGI) is broadcasted in SIB1 and can be associated to a locally unique identifier (PCI).

- In LTE, a globally unique identifier (ECGI) is broadcasted with a larger periodicity than the locally unique identifier (PCI).

- The RRM measurements on IDLE RSs are expected to be reported as per the measured PCI values by the UE.

- The information elements required to derive the ECGI (PLMN ID and the Cell Identity) are provided in NR as expected to be part of the SIB1 message.

Based on these observations, we have proposed the following:

- To support ANR across NR cells, as in LTE, NG-RAN can configure the UE to read and report NR neighboring cell’s Global identifier(s) (GID), TAC(s) and all available PLMN ID(s) to aid neighbor relation establishment procedure. FFS structure of GID is same as the structure of ECGI in LTE.

- To support LTE IRAT ANR procedure for NR cells, including EN-DC, E-UTRAN can configure the UE to read and report neighboring NR cell’s Global identifier(s) (GID), TAC(s) and all available PLMN ID(s) to aid neighbor relation establishment.

E-UTRAN measurement configuration (defined in 36.331) should be used to support CGI reporting.

NG-RAN measurement configuration (defined in 38.331) should be used to support CGI reporting.

There is no need to differentiate between FR1 and FR2 on the LTE reportCGI capability bit.

# References

A. Dahlén, A. Johansson, F. Gunnarsson, J. Moe, T. Rimhagen, H. Kallin, ”Evaluations of LTE Automatic Neighbor Relations”, in Proc. IWSON at VTC Spring, Budapest, Hungary, 2011.

3GPP TS 36.300 V13.4.0 (2016-06), “Evolved Universal Terrestrial Radio Access (E-UTRA) and Evolved Universal Terrestrial Radio Access Network (E-UTRAN); Overall description”