3GPP TSG-RAN WG2#101bis 	R2-1806226

# Sanya, China, 16th April-20th April 2018

| CR-Form-v11.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.321 | CR | 53 | rev | 2 | Current version: | 15.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Overriding rule in a repetition bundle |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | CATT |  |  |  |  |  |  |  |  |  |
| Source to TSG: | RAN2 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT-Core |  |  |  |  |  |  | Date: |  | 2018-02-08 |
| Category: | F |  |  |  |  |  |  | Release: |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | RAN2 agreed the principle that a dynamic grant always overrides a configured grant. However the following usecase is currently not addressed in the specification: a configured grant repetition (in a bundle) overlaps with a scheduled grant: this is left undefined in the specification so far since in that case repetitions of configured grants in a bundle do not go through Section 5.4.1 (since they occur while the timer is still running) but are rather handled autonomously by the HARQ entity (Section 5.4.2.1), where no mention is done of such possible collision. In this situation, we should keep the rule that any dynamic grant overrides any overlapping configured grant, even within a repetition cluster. |  |  |  |  |  |  |  |  |
| Summary of change: |  | In Section 5.4.2.1, the configured uplink grant is ignored in the retransmission branch if it overlaps with the PUSCH duration of a dynamic grant. Impact analysis Impacted functionality: HARQ entity. Inter-operability: - If the network is implemented according to the CR and the UE is not, the UE may choose to transmit either the dynamic UL grant or the configured UL grant by implementation. - If the UE is implemented according to the CR and the network is not, there is no impact since the network is not expected to schedule a dynamic UL grant during a configured grant repetition bundle since the feature is not supported. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The UL transmissio can’t be decoded correctly and may incur UL interference with other UE(s). |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.4.2.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  | TS/TR ... CR ... |  |  |  |
| affected: |  |  | X | Test specifications |  |  | TS/TR ... CR ... |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  | TS/TR ... CR ... |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

Start of change

### 5.4.2	HARQ operation

#### 5.4.2.1	HARQ Entity

The MAC entity includes a HARQ entity for each Serving Cell with configured uplink (including the case when it is configured with supplementaryUplink), which maintains a number of parallel HARQ processes.

The number of parallel UL HARQ processes per HARQ entity is specified in TS 38.214 [7].

Each HARQ process supports one TB.

Each HARQ process is associated with a HARQ process identifier. For UL transmission with UL grant in RA Response, HARQ process identifier 0 is used.

When the MAC entity is configured with pusch-AggregationFactor > 1, the parameter pusch-AggregationFactor provides the number of transmissions of a TB within a bundle of the dynamic grant. After the initial transmission, pusch-AggregationFactor – 1 HARQ retransmissions follow within a bundle. When the MAC entity is configured with repK > 1, the parameter repK provides the number of transmissions of a TB within a bundle of the configured grant. After the initial transmission, HARQ retransmissions follow within a bundle. For both dynamic grant and configured grant, bundling operation relies on the HARQ entity for invoking the same HARQ process for each transmission that is part of the same bunde. Within a bundle, HARQ retransmissions are triggered without waiting for feedback from previous transmission according to pusch-AggregationFactor for a dynamic grant and repK for a configured grant, respectively. Each transmission within a bundle is a separate uplink grant after the initial uplink grant within a bundle is delivered to the HARQ entity.

For each transmission within a bundle of the dynamic grant, the sequence of redundancy versions is determined according to subclause 6.1.4 of TS 38.214 [7]. For each transmission within a bundle of the configured grant, the sequence of redundancy versions is determined according to subclause 6.1.2.3 of TS 38.214 [7].

For each uplink grant, the HARQ entity shall:

1>	identify the HARQ process associated with this grant, and for each identified HARQ process:

2>	if the received grant was not addressed to a Temporary C-RNTI on PDCCH, and the NDI provided in the associated HARQ information has been toggled compared to the value in the previous transmission of this TB of this HARQ process; or

2>	if the uplink grant was received on PDCCH for the C-RNTI and the HARQ buffer of the identified process is empty; or

2>	if the uplink grant was received in a Random Access Response; or

2>	if the uplink grant is part of a bundle of the configured grant, and may be used for initial transmission according to subclause 6.1.2.3 of TS 38.214 [7], and if no MAC PDU has been obtained for this bundle:

3>	if there is a MAC PDU in the Msg3 buffer and the uplink grant was received in a Random Access Response:

4>	obtain the MAC PDU to transmit from the Msg3 buffer.

3>	else:

4>	obtain the MAC PDU to transmit from the Multiplexing and assembly entity, if any;

3>	if a MAC PDU to transmit has been obtained:

4>	deliver the MAC PDU and the uplink grant and the HARQ information of the TB to the identified HARQ process;

4>	instruct the identified HARQ process to trigger a new transmission;

4>	if the uplink grant is addressed to CS-RNTI; or

4>	if the uplink grant is a configured uplink grant; or

4>	if the uplink grant is addressed to C-RNTI, and the identified HARQ process is configured for a configured grant:

5>	start or restart the configuredGrantTimer, if configured, for the corresponding HARQ process when the transmission is performed.

2>	else (i.e. retransmission):

3>	if the uplink grant received on PDCCH was addressed to CS-RNTI and if the HARQ buffer of the identified process is empty; or

3>	if the uplink grant is part of a bundle of the configured grant and if no MAC PDU has been obtained for this bundle; or

3> if the uplink grant is part of a bundle of the configured grant, and the PUSCH of the uplink grant overlaps with a PUSCH of another uplink grant received on the PDCCH for this Serving Cell:

4>	ignore the uplink grant.

3>	else:

4>	deliver the uplink grant and the HARQ information (redundancy version) of the TB to the identified HARQ process;

4>	instruct the identified HARQ process to trigger a retransmission;

4>	if the uplink grant is addressed to CS-RNTI; or

4>	if the uplink grant is addressed to C-RNTI, and the identified HARQ process is configured for a configured grant:

5>	start or restart the configuredGrantTimer, if configured, for the corresponding HARQ process when the transmission is performed.

When determining if NDI has been toggled compared to the value in the previous transmission the MAC entity shall ignore NDI received in all uplink grants on PDCCH for its Temporary C-RNTI.

End of change