# 3GPP TSG-RAN2 Meeting #101bis																                                     R2-1806384

# Sanya, China, 16th - 20th April 2018

| CR-Form-v11.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.300 | CR | 0023 | rev | - | Current version: | 15.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CR to TS38.300 for CD-SSB |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Mediatek Inc. |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R2 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT-Core |  |  |  |  |  |  | Date: |  | 2018-04-17 |
| Category: | F |  |  |  |  |  |  | Release: |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | Introduction of CD-SSB |  |  |  |  |  |  |  |  |
| Summary of change: |  | Capture the agreements related to CD-SSB: - From UE perspective, a cell is associated with a single SS block - Note: The cell defining SS block has an associated RMSI - For a UE, the PCell, PSCell and each SCell has a single associated SSB in frequency (RAN1 terminology is the is the 'cell defining SSB') - Cell defining SS block can be changed by synchronous reconfiguration for PCell/PSCell and SCell release/add for the SCell. - Each SS block frequency which needs to be measured by the UE should be configured as individual measurement object (i.e. one measurement object corresponds to a single SS block frequency). - The CD-SSB is used for RRM serving cell measurements (irrespective of which BWP is activated) and differnet SSBs are considered as different cells. - RAN2 understand that the SSB of the cell where Idle/inactive UE camps is the cell defining SSB. - BWPs have no specification impact to cell selection and reselection. Cell selection and reselection is based on SSB. Introduce: 1 Abbreviations on CD-SSB; section 3.1 2 Definition of CD-SSB; section 3.2 3 Add one annex for the deployment scenario of multiple SSBs in a carrier and capture the agreements; Annex B |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The concept of CD-SSB is not supported. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 3.1, 3.2, Annex B |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

3.1	Abbreviations

For the purposes of the present document, the abbreviations given in 3GPP TR 21.905 [1], in 3GPP TS 36.300 [2] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in 3GPP TR 21.905 [1] and 3GPP TS 36.300 [2].

5GC	5G Core Network

5QI	5G QoS Identifier

AMBR	Aggregate Maximum Bit Rate

AMC	Adaptive Modulation and Coding

AMF	Access and Mobility Management Function

ARP	Allocation and Retention Priority

BA	Bandwidth Adaptation

BCH	Broadcast Channel

CD-SSB	Cell-defining SSB

CMAS	Commercial Mobile Alert Service

DCI	Downlink Control Information

DL-SCH	Downlink Shared Channel

ETWS	Earthquake and Tsunami Warning System

GFBR	Guaranteed Flow Bit Rate

MICO	Mobile Initiated Connection Only

MFBR	Maximum Flow Bit Rate

NCGI	NR Cell Global Identifier

NCR	Neighbour Cell Relation

NG-RAN	NG Radio Access Network

NGAP	NG Application Protocol

NR	NR Radio Access

PCH	Paging Channel

PDCCH	Physical Downlink Control Channel

PSS	Primary Synchronisation Signal

PUCCH	Physical Uplink Control Channel

PWS	Public Warning System

QFI	QoS Flow ID

RACH	Random Access Channel

RMSI	Remaining Minimum SI

RNA	RAN-based Notification Area

RNAU	RAN-based Notification Area Update

RQA	Reflective QoS Attribute

RQoS	Reflective Quality of Service

SDAP	Service Data Adaptation Protocol

SMF	Session Management Function

SPS	Semi-Persistent Scheduling

SSB	SS/PBCH block

SSS	Secondary Synchronisation Signal

SUL	Supplementary Uplink

TA	Timing Advance

UCI	Uplink Control Information

UL-SCH	Uplink Shared Channel

UPF	User Plane Function

URLLC	Ultra-Reliable and Low Latency Communications

XnAP	Xn Application Protocol

Xn-C	Xn-Control plane

Xn-U	Xn-User plane

3.2	Definitions

For the purposes of the present document, the terms and definitions given in 3GPP TR 21.905 [1], in 3GPP TS 36.300 [2] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in 3GPP TR 21.905 [1] and 3GPP TS 36.300 [2].

Cell-defining SSB: a SSB located on the synchronization raster and associated with RMSI;

gNB: node providing NR user plane and control plane protocol terminations towards the UE, and connected via the NG interface to the 5GC.

MSG1: preamble transmission of the random access procedure.

MSG3: first scheduled transmission of the random access procedure.

ng-eNB: node providing E-UTRA user plane and control plane protocol terminations towards the UE, and connected via the NG interface to the 5GC.

NG-C: control plane interface between NG-RAN and 5GC.

NG-U: user plane interface between NG-RAN and 5GC.

NG-RAN node: either a gNB or an ng-eNB.

Numerology: corresponds to one subcarrier spacing in the frequency domain. By scaling a reference subcarrier spacing by an integer N, different numerologies can be defined.

TTI duration: corresponds to a number of consecutive symbols in the time domain in one transmission direction. Different TTI durations can be defined when using different number of symbols.

Xn: network interface between NG-RAN nodes.

Annex B (informative):

Deployment Scenarios

B.2 Multiple SSBs in a carrier

Multiple SSBs may be transmitted within the frequency span of a carrier configured as serving cell to a UE in RRC_CONNECTED. SSBs transmitted within the carrier may have the same or different PCIs. From the UE perspective, each cell has a single associated CD-SSB regardless whether one or multiple SSBs are transmitted in the frequency span of the carrier.

In RRC_IDLE and in RRC_INACTIVE, the UE performs cell selection/reselection based on CD-SSBs.

In RRC_CONNECTED, the CD-SSB associated to a serving cell can be changed by reconfiguration with sync and SCell release/addition for PCell and SCells respectively. For SSB-based RRM measurement, one measurement object corresponds to a single SSB. Different SSBs are considered as different cells from UE perspective.