3GPP TSG-RAN WG2 #101bis			R2-1806390

Sanya, China, 16th-20th April 2018

Title:	Draft LS on different CGIs for different SSBs

Release:	Rel-15

Work Item:	NR_newRAT-Core

Source:	ZTE (to be RAN2)

To:	RAN3

Cc:

Contact Person:

#### Name:	Sergio Parolari

E-mail Address:	  Sergio.Parolari@zte.com.cn

1. Overall Description:

RAN2 discussed the implications of the agreement that there can be multiple SS/PBCH blocks (SSBs) within a wideband carrier, which can have the same or different PCI.

RAN2 agreed that different SSBs, if associated with a corresponding RMSI, define different individual cells (regardless of whether the same or different PCI is used) characterized by different Cell Global Identifiers (CGIs), which are globally unique in a PLMN.

RAN2 would also like to highlight that, for a UE in Connected state, the overall bandwidth of the cell defined by a given SSB can be different (larger) than the "Initial Bandwidth Part" associated to the SSB. For a UE in Connected state, the cell defined by a given SSB can also use radio resources that overlap in the frequency domain with the radio resources of other cells defined by other SSBs.

RAN2 invites RAN3 to take this into account where needed, e.g. for Xn and F1 procedures.

2. Actions:

RAN2 invites RAN3 to take the above RAN2 agreements into account.

3. Date of Next RAN2 Meetings:

RAN2#102	21 - 25 May 2018 	Busan, South Korea

RAN2#AH1807	2 - 6 July 2018		Montreal, Canada