3GPP TSG RAN WG2 Meeting #101Bis                   R2-1806500

Sanya, China, April 16th – 20th, 2018

Title:	Reply LS on Simultaneous PDSCH Reception with Paging

Response to:	R1-1803537 LS on Simultaneous PDSCH Reception with Paging

# Release:	Rel-15

Work Item:	NR_newRAT

Attachment: 	None

Source:	RAN2

To:	RAN1

# Contact Person:

# Name:	Ozcan Ozturk

E-mail Address:	oozturk@qti.qualcomm.com

# 1. Overall Description:

RAN2 thanks RAN1 for the LS to inform RAN1 working assumption regarding reception of P-RNTI PDSCH with paging by UEs in RRC_CONNECTED mode.

RAN2 has discussed the RAN1 LS and would like to provide the following feedback:

- RAN2#101 has agreed that a “RRC_CONNECTED UE monitors for SI update notification in any paging occasion”. Therefore, one consequence of RAN1 working assumption is that a connected UE only monitors PDCCH and thus does not need to decode PDSCH scheduled by P-RNTI in one or multiple paging occasion during modification period (which is 10 seconds in LTE). Since system notifications (SI update, CMAS, ETWS notifications) are usually rare, this may provide UE power savings.

- Another consequence of the RAN1 working assumption is that it can limit the ability of network to use the same message for both paging Idle UEs and also use PDCCH paging for system notifications. If many Idle mode UEs have to be paged, this may delay either their paging or the system notifications for some connected UEs if their C-DRX active durations overlap with also paging occasions.

RAN2 respectfully asks RAN1 to take the above observations into account in their agreements on this issue.

# 2. Actions to RAN1:

RAN2 respectfully asks RAN1 to take the above observations into account in their agreements on this issue.

# 3. Date of Next TSG-RAN WG2 Meetings:

TSG RAN WG2 Meeting #102	21 - 25 May 2018	Busan, Korea

TSG RAN WG2 Meeting 	AH-1807 	2 - 6 July 2018	Montreal, Canada