3GPP TSG RAN WG2 #102                                               	R2-1806732

Busan, Republic of Korea, May 21 – 25, 2018

Source: 	KDDI Corporation

Title:	Consideration on handling Pmax in HPUE

Tdoc Type:	Discussion

Agenda Item:	7.3 Other LTE Rel-13 WIs

Document for:	Discussion and decision

Release:	Rel-13

Introduction

In RAN2#101bis, there were two discussions related to Pmax HPUE handling. The one is LS from RAN4[1] and it is for single UL case, and the other is for UL CA case[2]. In this contribution, we summarized the current understanding of RAN4 specifications, and identify what RAN2 should discuss on this topic.

Discussion

  - Single UL case

RAN4 specified the default power class in TS36.101 as follows.

The default power class PPowerClass_Default for an operating band is Power Class 3 unless otherwise stated.

For a power class 2 capable UE operating on Band 41, when an IE P-max as defined in TS 36.331 [7] of 23 dBm or lower is indicated in the cell or if the uplink/downlink configuration is 0 or 6, the requirements for power class 2 are not applicable, and the corresponding requirements for a power class 3 UE shall apply.

Above specification, the following observations are made.

Observation1;	RAN4 specification defines the default power class as follows.

- For B41, the default power class is PC2(26dBm)

- For other than B41, the default power class is PC3(23dBm)

Observation2; From RAN4 specification, if Pmax is absent in RRC signalling, then the following UE behaviour are required.

- B41 PC2 capable UEs(B41 HPUEs) can use PC2(26dBm)

- B38/B47 PC2 capable UEs can use PC3(23dBm). e.g. B38/B47 PC2 capable UEs can use 26dBm only when eNB explicitly indicate “26dBm”.

- B3/B14/B21/B28 PC1 capable UEs can PC3(23dBm). e.g. B3/B14/B21/B28 PC1 capable UEs can use 31dBm only when eNB explicitly indicate “31dBm”.

- All PC3 capable UE can use PC3(23dBm)

The following table1 summarizes the UE behaviors which are affected the RAN2 CR [3].

|  | Pmax in PCell |  | Original UE behavior | Modified UE behavior [3] |
| --- | --- | --- | --- | --- |
| 1 | Absent | B41 | 26 dBm | 23 dBm |
|  |  | Other than B41 | 23 dBm | 23 dBm |
| 2 | 23 dBm |  | 23 dBm |  |
| 3 | 26 dBm |  | 26 dBm |  |

Table 1: UE behaviour in single UL case

And based on the above observations, we propose:

Proposal 1; RAN2 should take into account the above two observations and discuss whether the current RAN2 specification can capture correctly what RAN4 specification specifies.

  - UL CA case

In RAN2#101bis, the following CR [2] was agreed.

p-Max

Pmax to be used in the target cell. If absent the UE applies the maximum power of the default power class for the band used in the target cell, defined according to 36.101 [42]. In case the UE is configured with uplink intra-band contiguous CA and the UE indicates ue-CA-PowerClass-N in that band combination, then the p-Max in RadioResourceConfigCommonSCell for that SCell, if present, also applies for that band combination whenever that SCell is activated.

Above agreed CR, the following observations are made.

Observation3; The agreed CR in RAN2#101bis meeting doesn’t itself mention about the UE behavior when Pmax in Scell is absent.

Observation4; In case where Pmax in Scell is absent, in the current specification it is not clear whether B41 HPUE CA capable UE should use 23 dBm or 26 dBm.

The following table2 summarizes the UE behaviors which are affected the RAN2 CR [2].

| Case | Pmax in PCell | Pmax in SCell |  | UE behavior |
| --- | --- | --- | --- | --- |
| 1 | Absent | Absent | B41 | 23 dBm? 26 dBm? |
| 2 |  |  | Other than B41 | 23 dBm |
| 3 |  | 23 dBm |  | 23 dBm |
| 4 |  | 26 dBm |  | 26 dBm |
| 5 | 23 dBm | Absent | B41 | Same as case1 |
| 6 |  |  | Other than B41 | Same as case2 |
| 7 |  | 23 dBm |  | Same as case3 |
| 8 |  | 26 dBm |  | Same as case4 |
| 9 | 26 dBm | Absent | B41 | Same as case1 |
| 10 |  |  | Other than B41 | Same as case2 |
| 11 |  | 23 dBm |  | Same as case3 |
| 12 |  | 26 dBm |  | Same as case4 |

Table 2: UE behaviour in single UL case

And based on the above observations, we propose:

Proposal 2; RAN2 agree the related CRs [4][5][6] which were agreed in principle in RAN2#101bis, since it doesn’t mention about the UE behavior when Pmax in Scell is absent.

Proposal 3; RAN2 should discuss the UE behavior when Pmax in Scell is absent, and have a clear description for it on TS36.331.

Proposal

Based on the above discussion, we made the following observations and propose the following:

For single UL case,

Observation1;	RAN4 specification defines the default power class as follows.

- For B41, the default power class is PC2(26dBm)

- For other than B41, the default power class is PC3(23dBm)

Observation2; From RAN4 specification, if Pmax is absent in RRC signalling, then the following UE behaviour are required.

- B41 PC2 capable UEs(B41 HPUEs) can use PC2(26dBm)

- B38/B47 PC2 capable UEs can use PC3(23dBm). e.g. B38/B47 PC2 capable UEs can use 26dBm only when eNB explicitly indicate “26dBm”.

- B3/B14/B21/B28 PC1 capable UEs can PC3(23dBm). e.g. B3/B14/B21/B28 PC1 capable UEs can use 31dBm only when eNB explicitly indicate “31dBm”.

- All PC3 capable UE can use PC3(23dBm)

Proposal 1; RAN2 should take into account the above two observations and discuss whether the current RAN2 specification can capture correctly what RAN4 specification specifies.

For UL CA case,

Observation3; The agreed CR in RAN2#101bis meeting doesn’t itself mention about the UE behavior when Pmax in Scell is absent.

Observation4; In case where Pmax in Scell is absent, in the current specification it is not clear whether B41 HPUE CA capable UE should use 23 dBm or 26 dBm.

Proposal 2; RAN2 agree the related CRs [4][5][6] which were agreed in principle in RAN2#101bis, since it doesn’t mention about the UE behavior when Pmax in Scell is absenct.

Proposal 3; RAN2 should discuss the UE behavior when Pmax in Scell is absent, and have a clear description for it on TS36.331.

References

[1] R2-1806423 LS to RAN2 on P-max procedure for high-power UEs (R4-1805695)	RAN4

[2] R2-1806293 Handling of Pmax for PC2 and uplink intra-band contiguous CA capable UEs
KDDI, Ericsson and Qualcomm

[3] R2-1803837 Correction to handling of p-Max procedure for high-power UEs
Intel Corporation

[4] R2-1806733 Handling of Pmax for PC2 and uplink intra-band contiguous CA capable UEs
KDDI, Ericsson and Qualcomm	CR	Rel-13

[5] R2-1806734 Handling of Pmax for PC2 and uplink intra-band contiguous CA capable UEs
KDDI, Ericsson and Qualcomm	CR	Rel-14

[6] R2-1806735 Handling of Pmax for PC2 and uplink intra-band contiguous CA capable UEs
KDDI, Ericsson and Qualcomm	CR	Rel-15